Korea Lunar Exploration Program

KPLO DTNPL Specifications

| Date: 20 Jan. 2020 | Doc. No: KPLO-SP-550-001 Issue: F.00 Total page: 20 |
| --- | --- |

Superseding     Revised

| Prepared By: Jin-Ho Jo Date DTN System Engineer, ETRI |  |  |
| --- | --- | --- |
| Reviewed By: Kyung-Rak Lee Date DTN Payload Engineer, ETRI |  | Sin-Ae Ji Date Proximity-1 System Engineer, ETRI |
| Moon-Jin Jeon Date Engineer, KPLO Electrical System, KARI |  | Hyun-Jin Shin Date KPLO Payload Development Management, KARI |
| Approval Signature: Cheol-Oh Jeong Date DTN Product Assurance, ETRI |  | Byoung-Sun Lee Date DTN Project Leader, ETRI |

Original CDMO Release__________________

| Revision History |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version | Author(s) |  | Date | Description |
| 0.1 | Jin-Ho Jo |  | 2016. 11. 30 | Draft |
| 1.0 | Jin-Ho Jo |  | 2017. 08. 21 | 1st Issue |
| 1.1 | Kyung-Rak Lee |  | 2018. 01. 10 | Update KARI comments |
| 1.2 | Jin-Ho Jo |  | 2018. 07. 03 | Update EM PTR comments |
| 1.3 | Jin-Ho Jo |  | 2019. 02. 13 | Update ICD changes |
| F.00 | Jin-Ho Jo |  | 2020. 01. 20 | Issue Final version 00 |

| Revision/Change Records |  |  |  |
| --- | --- | --- | --- |
| Version | Date | Revision/Change Description | Pages |
| 1.0 | 2016. 11. 30 | Draft | All |
| 1.0 | 2017. 08. 21 | Update words as following - Mode → Model - MCC → MOC - DSN → KDSA | 11/12 |
| 1.1 | 2018. 01. 10 | Specify documents number of KPLO EV spec. and KPLO EMC requirements Change section title from [other document] to [KARI document] | All 8 |
| 1.2 | 2018. 07. 02 | Change reference document in 30150 Add missing SSID in the trace matrix (20040, 10080, 30130, 30140, 30150) Update verification matrix | 16 17-18 19-20 |
| 1.3 | 2019. 02. 13 | Change AMS to BPM Change computing power value from 50MIPS to 45MIPS Change power connector gender to male Change date connector gender to female | 13 14 14 15 |
| F.00 | 2020. 01. 20 | Delete [TBD] in SSD-DPL-10070 Delete [TBC] in SSD-DPL-30100 Delete [TBC] in SSD-DPL-30110 Delete [TBC] in SSD-DPL-40020 | 13 115 15 16 |

Documents Tree

Contents

1.	Introduction	8

1.1.	Purpose and Scope	8

1.2.	Applicable Documents	8

1.2.1.	ETRI Documents	8

1.2.2.	KARI Documents	8

1.3.	Abbreviations	8

2.	Operation Description	10

2.1.	DTN and Space Communications	10

2.2.	Mission of DTN Operation	10

2.3.	DTN Configurations	10

2.4.	Communication Nodes	11

2.4.1.	DTN nodes	11

2.4.2.	Non-DTN nodes	11

3.	System Configuration of DTN Payload	12

4.	System Specifications	13

4.1.	Functional Specifications	13

4.2.	Performance Specifications	14

4.3.	Interface Specifications	14

4.4.	Other Specifications	16

5.	Trace Matrix	17

6.	Verification Matrix	19

Figures

Figure 2.1 DTN test configuration in the first phase	11

Figure 3.1 Interface diagram of DTN payload	12

Tables

Introduction

Purpose and Scope

This document describes the system specifications of DTN payload for Korea lunar exploration program.

Applicable Documents

ETRI Documents

| Document Number | Document Title |
| --- | --- |
| KPLO-D0-551-002 | Operation Scenario of DTN |
| KPLO-D0-550-000 | System Requirements of DTN Payload |

KARI Documents

| Document Number | Document Title |
| --- | --- |
| KPLO-SP-310-001 | KPLO System Specification Document |

Abbreviations

| AMS | Asynchronous Message Service |
| --- | --- |
| AOS | Advanced Orbiting System |
| BP | Bundle Protocol |
| BSS | Bundle Streaming Service |
| CCSDS | Consultative Committee for Space Data Systems |
| CFDP | CCSDS File Delivery Protocol |
| DCC | DTN Control Center |
| DTN | Delay(Disruption) Tolerant Network |
| DTNPL | DTN Payload |
| IP | Internet Protocol |
| KDSA | Korea Deep Space Antenna |
| KPLO | Korea Pathfinder Lunar Orbiter |
| LCM | Lander Communication Model |
| LTP | Licklider Transmission Protocol |
| MOC | Mission Operation Center |
| M&C | Monitor & Control |
| RCM | Rover Communication Model |
| SBMU | Spacecraft Bus Management Unit |
| SLE | Space Link Extension |
| TBC | To Be Confirmed |
| TBD | To Be Defined |
| TCP | Transmission Control Protocol |
| UART | Universal Asynchronous Receiver Transmitter |
| USRT | Universal Synchronous Receiver Transmitter |
| UDP | User Datagram Protocol |

Operation Description

DTN and Space Communications

Delay/disruption tolerant networking (DTN) technology offers a new solution to highly stressed communications in space environments, especially those with long link delay and frequent link disruptions in deep space missions. Delay/disruption tolerant networking (DTN) technology offers a new solution that use a store-and-forward mechanism to combat long link delay and frequent link disruptions generally characterizing space communications. DTN communications use a bundle protocol (BP), to construct a store-and-forward overlay network. The BP, situated at the application layer of the Internet model, utilizes conventional Internet protocols to send and receive data bundles. The benefits of DTN communications are as follows.

- Interoperability and Reuse: A standardized DTN protocol suite enables interoperability of multi-agency communication assets and also allows NASA to use the same communication stack for future missions (LEO, NEO or Deep Space).

- Space Link Efficiency, Utilization and Robustness: DTN enables more reliable and efficient data transmissions resulting in more usable bandwidth. DTN also improves link reliability by having multiple network paths and assets for potential communication hops.

- Security: The DTN Bundle Security Protocol (BSP) allows for integrity checks, authentication and encryption, even on links where not previously used.

- Quality of Service: The DTN protocol suite allows for many priority levels to be set for different data types, ensuring that the most important data is received ahead of less important data.

- Improved Operations and Situational Awareness: The DTN store-and-forward mechanism along with automatic retransmission provides more insight into events during communication outages and significantly reduces the need for ground-based scheduling.

Mission of DTN Operation

The mission of DTN is to validate DTN communication protocols by space link testing through KPLO. So DTN operations are focused on testing the DTN protocols using KPLO space links.

DTN Configurations

In the first phase of lunar exploration, test lunar orbiter called Korea Pathfinder Lunar Orbiter (KPLO) will be launched without lunar lander. Figure 2.1 shows DTN configuration in first phase of lunar exploration. There are no Lander and Rover on the moon in first phase. So Lander Communication Model (LCM) and Rover Communication Model (RCM) will be located on earth for the DTN communication testing.

DTN Payload (DTNPL) is a DTN node in the space. DTNPL will be mounted on the KPLO for DTN communication. DTN Control Center (DCC) is a control center of DTN communication nodes. LCM and RCM are DTN nodes on ground. All DTN nodes on ground are interconnected by the terrestrial network such as Internet. Mission Operation Center (MOC) and Korea Deep Space Antenna (KDSA) are not DTN nodes. They just transfer DTN packets to next communication nodes.

Figure 2.1 DTN test configuration in the first phase

Communication Nodes

Many communication nodes are needed for the DTN operation. Some of communication nodes in the network are DTN nodes whereas otherwise nodes are non-DTN nodes. The function of communication nodes used in the DTN operation is as follows.

DTN nodes

- DCC is a ground DTN node which monitor and controls DTN nodes. DCC send commands to DTN nodes such as DTNPL, LCM and RCM. It also receives telemetries or acquired data from some DTN nodes such as RCM or LCM.

- DTNPL is a DTN node mounted on the KPLO. DTNPL can be operated as a DTN end point. And it also can be operated as relay nodes which relay DTN data between DCC and RCM or LCM.

- LCM is a ground DTN node. LCM relays DTN data between MOC and RCM.

- RCM is a ground DTN node and a DTN end point. The commands sent by the DCC is terminated at the RCM and the RCM returns telemetry or data to DCC through KPLO.

Non-DTN nodes

- MOC is a non-DTN node. MOC monitors and controls KPLO and payloads. MOC also receives and processes data generated from the payloads and transfers to each payload control center.

- KDSA is a ground station for deep space communication. KDSA will have a big size of antenna and RF equipment for space link interface. KDSA also have network equipment for terrestrial communication interface such as SLE link. KDSA is a non-DTN node.

System Configuration of DTN Payload

The DTN payload shall have interface with KPLO for the operational power supply and for the communications. The following figure shows DTN payload interface diagram with KPLO.

Figure 3.1 Interface diagram of DTN payload

There are four kinds of interface types between DTNPL and KPLO. Each interface types are described as following.

- TM/TC (bi-level): One bi-level TC and one bi-level TM for DTNPL emergency operation.

- Bi-level TC: System reset command in the emergence condition

- Bi-level TM: Telemetry showing that DTNPL is in alive

- TM/TC (UART): Serial communication channel for TM/TC communication with KPLO SBMU

- Data downlink (USRT): Serial data downlink channel to KPLO PDHU

- Power: Operation DC power +28V from the KPLO PCDU

System Specifications

Functional Specifications

| ID | Requirements |
| --- | --- |
| SSD-DPL-10010 | DTN Protocols The DTNPL shall support DTN protocols which include at least Bundle Protocol (BP) and Licklider Transmission Protocol (LTP) |
| SSD-DPL-10020 | DTN User Utilities The DTNPL shall support DTN user utilities which include CCSDS File Delivery Protocol (CFDP), Bundle Protocol Message (BPM) and Bundle Streaming Service (BSS) |
| SSD-DPL-10030 | DTNPL Modes The DTNPL shall support two kinds of modes. One is DTN end point mode and the other is DTN way point mode. |
| SSD-DPL-10040 | DTN End Point Mode In the DTN end point mode, DTNPL shall be operated as a source or destination node for the DTN communications. This mode necessary for the one-way trip test defined in the document ‘Operation Scenario of DTN’ (KPLO-D0-551-002)’. |
| SSD-DPL-10050 | DTN Way Point Mode In the DTN way point mode, DTNPL shall be operated as relay node for the DTN communications. This mode necessary for the round trip test defined in the document Operation Scenario of DTN’ (KPLO-D0-551-002)’. |
| SSD-DPL-10060 | Health Report The DTNPL shall report its health status to KPLO SBMU for the reliable operation. The health status shall be included following information. - Payload time, FW version - Payload 1st voltage, Payload 1st current, Payload 2nd voltage - Board temperature, FPGA temperature |
| SSD-DPL-10070 | DTN Status Report The DTNPL shall report DTN status to KPLO SBMU for the DTN operation. The DTN status shall be included following information. - DTN status (enabled/disabled) - ION version |
| SSD-DPL-10080 | Time Synchronization The DTNPL shall receive KPLO system time through RS-422 UART interface and set payload time for synchronize to KPLO system time. |
| SSD-DPL-10090 | Fault Detection The DTNPL shall provide its abnormal state to KPLO SBMU using bi-level telemetry. |
| SSD-DPL-10100 | Fault Recovery When abnormal state is reported, DTNPL shall be re-initialized by the bi-level command issued by the KPLO SBMU. |
| SSD-DPL-10110 | Radiation Mitigation The DTNPL shall be designed to avoid critical failures causing permanent or temporal mission loss from space radiation environment defined in the document ‘KPLO Environmental Design and Test Specification (KLPO-SP-320-002)’ |
| SSD-DPL-10120 | Abrupt Power Off DTNPL shall not be damaged by abrupt power off condition. |

Performance Specifications

| ID | Requirements |
| --- | --- |
| SSD-DPL-20010 | Computing Power The DTNPL computing power shall be more than 45MIPS. |
| SSD-DPL-20020 | TM/TC Data Interface Speed The speed of TM/TC bus interface shall be 115.2kbps. |
| SSD-DPL-20030 | Payload Data Interface Speed The speed of data bus interface shall be more than 1Mbps. |
| SSD-DPL-20040 | Number of Analog I/O Channels The DTNPL shall provide up to 8 analog I/O channels for analog telemetry gathering. |
| SSD-DPL-20050 | Memory Size The DTNPL shall have more than 2G byte memory space for the DTN data store. |
| SSD-DPL-20060 | Life Time The operational life time of the DTNPL shall be no less than 1 year from the entering lunar orbit. |
| SSD-DPL-20070 | Reliability DTNPL reliability shall be more than 0.7 over the operational life time |

Interface Specifications

| ID | Requirements |
| --- | --- |
| SSD-DPL-30010 | DC Power Interface The DTNPL shall have DC power interface to get operation power from the KPLO. |
| SSD-DPL-30020 | Operating Voltage The DTNPL shall be operate over the voltage range 24VDC ~ 32.8VDC |
| SSD-DPL-30030 | DC Power Connector The DC power interface connector in the DTNPL shall be a 15 pins D-sub male type. The pin assignments are defined in the document ‘Interface Control Document between DTNPL and KPLO (KPLO-D0-550-002).’ |
| SSD-DPL-30040 | TM/TC Serial Bus Type The serial data bus type for TM/TC communication between DTNPL and KPLO SBMU shall be a RS-422 UART. |
| SSD-DPL-30050 | TM/TC Serial Data Format The TM/TC communication data format between DTNPL and KPLO SBMU shall be compatible with defined in the document ‘Interface Control Document between DTNPL and KPLO (KPLO-D0-550-002).’ |
| SSD-DPL-30060 | TM/TC Serial Bus Connector The TM/TC interface connector in the DTNPL shall be a 26 pins D-sub female type. The pin assignments are defined in the document ‘Interface Control Document between DTNPL and KPLO (KPLO-D0-550-002).’ |
| SSD-DPL-30070 | Payload Data Bus Type The serial data bus type for payload data communication between DTNPL and KPLO PDHU shall be RS-422 USRT |
| SSD-DPL-30080 | Payload Data Format The payload communication data format between DTNPL and KPLO PDHU shall be compatible with defined in the document ‘Interface Control Document between DTNPL and KPLO (KPLO-D0-550-002).’ |
| SSD-DPL-30090 | Payload Data Bus Connector The payload data interface connector in the DTNPL shall be a 26 pins D-sub male type. The pin assignments are defined in the document ‘Interface Control Document between DTNPL and KPLO (KPLO-D0-550-002).’ |
| SSD-DPL-30100 | Bi-Level TM/TC Level Bi-level TM/TC voltage levels are 0V for low state and +5V for high state. |
| SSD-DPL-30110 | Bi-Level TM/TC Connector The bi-level TM/TC interface connector in the DTNPL shall be a 26 pins D-sub male type. The pin assignments are defined in the document ‘Interface Control Document between DTNPL and KPLO (KPLO-D0-550-002).’ |
| SSD-DPL-30120 | Mounting Holes The DTNPL shall have mounting holes for the mounting on the KPLO panel. Detailed information is defined in the document ‘Interface Control Document between DTNPL and KPLO (KPLO-D0-550-002).’ |
| SSD-DPL-30130 | Thermal Interface The DTNPL shall comply temperature limit requirements defined in the ‘Environmental Design and Test Specification (KLPO-SP-320-002)’ |
| SSD-DPL-30140 | EMI/EMC The DTNPL shall be comply EMC requirements specification defined in the ‘KPLO EMC Requirements Specification (KPLO-SP-330-001)’ |
| SSD-DPL-30150 | Grounding The grounding of DTNPL shall comply grounding requirements defined in the ‘KPLO EMC Requirements Specifications (KPLO-SP-330-001). |

Other Specifications

| ID | Requirements |
| --- | --- |
| SSD-DPL-40010 | Mass DTNPL mass shall be less than 2kg. |
| SSD-DPL-40020 | Volume DTNPL volume shall be less than 170 x 120 x 50(mm). |
| SSD-DPL-40030 | Power Consumption DTNPL power consumption shall be less than 12W. |
| SSD-DPL-40040 | Withstand Conditions The DTNPL shall withstand at the conditions defined in the document ‘Environmental Design and Test Specification (KLPO-SP-320-002)’. |
| SSD-DPL-40050 | Operation Conditions The DTNPL performance shall not degrade at the conditions defined in the document ‘Environmental Design and Test Specification (KLPO-SP-320-002)’. |
| SSD-DPL-40060 | Storage and Handling The DTNPL shall be able to store over 1 year in the container specially manufactured for the long term storage |

Trace Matrix

| SRS ID | SRS Title | SSD ID | SSD Title |
| --- | --- | --- | --- |
| SRS-DPL-10010 | DTN Protocols | SSD-DPL-10010 | DTN Protocols |
| SRS-DPL-10020 | DTN User Utilities | SSD-DPL-10020 | DTN User Utilities |
| SRS-DPL-10030 | DTNPL Modes | SSD-DPL-10030 | DTNPL Modes |
| SRS-DPL-10040 | DTN End Point Mode | SSD-DPL-10040 | DTN End Point Mode |
| SRS-DPL-10050 | DTN Way Point Mode | SSD-DPL-10050 | DTN Way Point Mode |
| SRS-DPL-10060 | Health Report | SSD-DPL-10060 SSD-DPL-10070 SSD-DPL-20040 | Health Report DTN Status Report Number of Analog I/O Channels |
| SRS-DPL-10070 | Fault, Detection and Recovery | SSD-DPL-10090 SSD-DPL-10100 | Fault Detection Fault Recovery |
| SRS-DPL-10080 | Radiation Mitigation | SSD-DPL-10110 | Radiation Mitigation |
| SRS-DPL-10090 | Abrupt Power Off | SSD-DPL-10120 | Abrupt Power Off |
| SRS-DPL-20010 | Computing Power | SSD-DPL-20010 | Computing Power |
| SRS-DPL-20020 | Data Interface Speed | SSD-DPL-20020 SSD-DPL-20030 | TM/TC Data Interface Speed Payload Data Interface Speed |
| SRS-DPL-20030 | Memory Size | SSD-DPL-20050 | Memory Size |
| SRS-DPL-20040 | Life Time | SSD-DPL-20060 | Life Time |
| SRS-DPL-20050 | Reliability | SSD-DPL-20070 | Reliability |
| SRS-DPL-30010 | DC Power Interface | SSD-DPL-30010 SSD-DPL-30030 | DC Power Interface DC Power Connector |
| SRS-DPL-30020 | Operating Voltage | SSD-DPL-30020 | Operating Voltage |
| SRS-DPL-30030 | Data Interface | SSD-DPL-30040 SSD-DPL-30060 SSD-DPL-30070 SSD-DPL-30090 SSD-DPL-30110 SSD-DPL-10080 | TM/TC Serial Bus Type TM/TC Serial Bus Connector Payload Data Bus Type Payload Data Bus Connector Bi-Level TM/TC Connector Time Synchronization |
| SRS-DPL-30040 | Data Interface Format | SSD-DPL-30050 SSD-DPL-30080 SSD-DPL-30100 | TM/TC Serial Data Format Payload Data Format Bi-Level TM/TC Level |
| SRS-DPL-30060 | Mounting Holes | SSD-DPL-30120 | Mounting Holes |
| SRS-DPL-40010 | Mass | SSD-DPL-40010 | Mass |
| SRS-DPL-40020 | Volume | SSD-DPL-40020 | Volume |
| SRS-DPL-40030 | Power Consumption | SSD-DPL-40030 | Power Consumption |
| SRS-DPL-40040 | Withstand Conditions | SSD-DPL-40040 | Withstand Conditions |
| SRS-DPL-40050 | Operation Conditions | SSD-DPL-40050 SSD-DPL-30130 SSD-DPL-30140 SSD-DPL-30150 | Operation Conditions Thermal Interface EMI/EMC Grounding |
| SRS-DPL-40060 | Storage and Handling | SSD-DPL-40060 | Storage and Handling |

Verification Matrix

| SSD ID | Title | Methods |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T | A | I | D | S |
| SSD-DPL-10010 | DTN Protocols | O |  |  |  |  |
| SSD-DPL-10020 | DTN User Utilities | O |  |  |  |  |
| SSD-DPL-10030 | DTNPL Modes | O |  |  |  |  |
| SSD-DPL-10040 | DTN End Point Mode | O |  |  |  |  |
| SSD-DPL-10050 | DTN Wat Point Mode | O |  |  |  |  |
| SSD-DPL-10060 | Health Reports | O |  |  |  |  |
| SSD-DPL-10070 | DTN Status Report | O |  |  |  |  |
| SSD-DPL-10080 | Time Synchronization | O |  |  |  |  |
| SSD-DPL-10090 | Fault Detection |  |  |  | O |  |
| SSD-DPL-10100 | Fault Recovery |  |  |  | O |  |
| SSD-DPL-10110 | Radiation Mitigation |  | O |  |  |  |
| SSD-DPL-10120 | Abrupt Power Off | O |  |  |  |  |
| SSD-DPL-20010 | Computing Power |  |  |  | O |  |
| SSD-DPL-20020 | TM/TC Data Interface Speed | O |  |  |  |  |
| SSD-DPL-20030 | Payload Data Interface Speed | O |  |  |  |  |
| SSD-DPL-20040 | Number of Analog I/O Channels |  |  |  | O |  |
| SSD-DPL-20050 | Memory Size |  |  |  | O |  |
| SSD-DPL-20060 | Life Time |  | O |  |  |  |
| SSD-DPL-20070 | Reliability |  | O |  |  |  |
| SSD-DPL-30010 | DC Power Interface |  |  | O |  |  |
| SSD-DPL-30020 | Operating Voltage | O |  |  |  |  |
| SSD-DPL-30030 | DC Power Connector |  |  | O |  |  |
| SSD-DPL-30040 | TM/TC Serial Bus Type |  |  |  | O |  |
| SSD-DPL-30050 | TM/TC Serial Data Format | O |  |  |  |  |
| SSD-DPL-30060 | TM/TC Serial Bus Connector |  |  | O |  |  |
| SSD-DPL-30070 | Payload Data Bus Type |  |  |  | O |  |
| SSD-DPL-30080 | Payload Data Format | O |  |  |  |  |
| SSD-DPL-30090 | Payload Data Bus Connector |  |  | O |  |  |
| SSD-DPL-30100 | Bi-Level TM/TC Level |  |  |  | O |  |
| SSD-DPL-30110 | Bi-Level TM/TC Connector |  |  | O |  |  |
| SSD-DPL-30120 | Mounting Holes |  |  | O |  |  |
| SSD-DPL-30130 | Thermal Interface |  | O |  |  |  |
| SSD-DPL-30140 | EMI/EMC | O |  |  |  |  |
| SSD-DPL-30150 | Grounding |  |  | O |  |  |
| SSD-DPL-40010 | Mass |  |  |  | O |  |
| SSD-DPL-40020 | Volume |  |  |  | O |  |
| SSD-DPL-40030 | Power Consumption |  |  |  | O |  |
| SSD-DPL-40040 | Withstand Conditions | O |  |  |  |  |
| SSD-DPL-40050 | Operation Conditions | O |  |  |  |  |
| SSD-DPL-40060 | Storage and Handling |  | O |  |  |  |

Where;

- T(Test), A(Analysis), I(Inspection), D(Demo), S(Similarity)