Korea Lunar Exploration Program

DTNPL

Interface Control Document

| Date: 04 May. 2020 | Doc. No: KPLO-SP-550-002 Issue: F.01 Total page: 30 |
| --- | --- |

Superseding     Revised

| Prepared By: Kyung-Rak Lee Date DTN Payload Engineer, ETRI |  | Jin-Ho Jo Date DTN System Engineer, ETRI |
| --- | --- | --- |
| Reviewed By: Sin-Ae Ji Date Proximity-1 System Engineer, ETRI |  | Eunhyeuk Kim Date RLE, KPLO Payload I/F Management, KARI |
| Seung-Young Min Date Engineer, KPLO Mechanical System, KARI |  | Changkyoon Kim Date RDE, KPLO Data Handling Unit, KARI |
| Approval Signature: Cheol-Oh Jeong Date DTN Product Assurance, ETRI |  | Byoung-Sun Lee Date DTN Project Leader, ETRI |
| Guen-Young Park Date RLE KPLO Product Assurance, KARI |  | Dae-Kwan Kim Date Head, KPLO SE&I, KARI |
| Yee-Jin Cheon Date Director, Satellite Tech. Research Development Division, KARI |  | Sang-Ryool Lee Date Director, Lunar Exploration Program Office, KARI |  |

Original CDMO Release__________________

| Revision History |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version | Author(s) |  | Date | Description |
| 0.1 | Jin-Ho Jo |  | 2016. 12. 30 | Draft |
| 0.2 | Jin-Ho Jo |  | 2017. 04. 12 | Update KARI review comments Update TM/TC packet structure |
| 1.0 | Kyung-Rak Lee |  | 2017. 10. 25 | Update PDHU interface |
| 1.1 | Kyung-Rak Lee |  | 2017. 11. 02 | Add power connector information Update Table 3-5 Add comm. connector information PDHU data packet structure was updated Add mechanical interface section Add thermal interface section |
| 1.2 | Kyung-Rak Lee |  | 2018. 01. 10 | Update KARI review comments |
| 1.3 | Jin-Ho Jo |  | 2018. 09. 13 | Add PDHU interface timing diagram |
| 1.4 | Kyung-Rak Lee |  | 2018. 10. 26 | Update TM/TC header format Update SOH structure Update the role of MOC |
| 1.5 | Jin-Ho Jo |  | 2019. 03. 13 | Change UART ACK mode Update TM list |
| 1.6 | Dae Soo Oh |  | 2019. 04. 18 | Mechanical interface was updated Thermal interface was updated |
| 1.7 | Jin-Ho Jo |  | 2019. 04. 25 | Update SOH TM |
| 1.8 | Kyung-Rak Lee |  | 2019. 06. 05 | Update TM/TC list (cyan colored) |
| F.00 | Jin-Ho Jo |  | 2020. 01. 20 | Issue version F.00 |
| F.01 | Jin-Ho Jo |  | 2020. 05. 04 | Update ACK definition Add appendix |

| Revision/Change Records |  |  |  |
| --- | --- | --- | --- |
| Version | Date | Revision/Change Description | Pages |
| 0.1 | 2016. 12. 30 | Initial | All |
| 0.2 | 2017. 04. 12 | Update Applicable Documents Update Figure 2-1 Update Table 3-1 (red color) Add TBD to the Table 3-5 title Update Table 3-1 (red color) Update Figure 4-1 Update Table 4-2 (red color) Update Table 4-4 (red color) | 8 11 13 15 15 17 18 18 |
| 1.0 | 2017. 10. 25 | Add abbreviations to Section 1.3 Update Figure 3-1 Update Figure 3-2 Update Table 3-1 Update Table 3-6 Update Table 4-4 Update Figure 4-2 Update Table 4-5 Update Table 4-6 Update Table 4-7 Update Table 4-8 Update Table 4-9 Update Table 4-10 | 8, 9 12 12 13 16 18 19 19 19 20 20 20 21 |
| 1.1 | 2017. 11. 03 | Add power connector information Update Table 3-5 Add comm. connector information PDHU data packet structure was updated Mechanical interface added Thermal interface added | 14 16 16 20 23 25 |
| 1.2 | 2018. 01. 10 | Change document title Change section title from [other document] to [KARI document] Add operation duty | All 8 13 |
| 1.3 | 2018. 09. 13 | Add PDHU interface timing diagram | 21 |
| 1.4 | 2018. 10. 26 | Update the role of MOC in Chapter 2.3 Update Figure 4-1 Update Table 4-2 Update Table 4-3 Update Table 4-5 Update Table 4-8 Update Table 4-9 |  |
| 1.5 | 2019. 03. 13 | Change UART interface ACK mode from [echo beck] to [OK check] Update Table 4-3 Update Table 4-9 | 19 20 24 |
| 1.6 | 2019. 04. 18 | Mechanical interface was updated Thermal interface was updated | 26-29 30 |
| 1.7 | 2019. 04. 25 | ACK method update (echo back to OK check) Update Table 4-3 | 21 22 |
| 1.8 | 2019. 06. 05 | Update chapter 4.1.2 Update chapter 4.1.3 | 21 22 |
| F.00 | 2020. 01. 20 | Delete [TBC] Delete <echo request> TC Delete <SW initialize> TC Delete <echo reply> TM Delete RFU in TC/TM | 15, 19 21, 24 21, 24 21, 25 21, 24 |
| F.01 | 2020. 05. 20 | Define ACK packet in the UART protocol Delete ACK, NACK in the TM list Update script1 command Delete ACK, NACK in the TM list Add Appendix | 20 21 21 24 30 |

Documents Tree

Contents

1.	Introduction	10

1.1.	Purpose and Scope	10

1.2.	Applicable Documents	10

1.2.1.	ETRI Documents	10

1.2.2.	KARI Documents	10

1.3.	Abbreviations	10

2.	Overview of KPLO DTN	12

2.1.	DTN and Space Communications	12

2.2.	Mission of DTN Operation	12

2.3.	DTN Configurations	12

2.4.	Communication Nodes	13

2.4.1.	DTN nodes	13

2.4.2.	Non-DTN nodes	13

3.	Electrical Interface	14

3.1.	Power Interface	15

3.2.	TM/TC (DMA UART, Bi-level) Interface	16

3.3.	Data Downlink Interface	18

4.	Communication Protocols	20

4.1.	TM/TC Protocols	20

4.1.1.	UART Protocol	20

4.1.2.	Packet Format	20

4.1.3.	TM/TC Data Structure4	21

4.2.	Datalink Protocol	21

4.2.1.	Datalink protocol for downlink	21

4.2.2.	Datalink Data Structure	22

4.2.3.	Interface Timing Diagram	23

4.3.	TM/TC Lists	23

4.3.1.	Command Lists	23

4.3.2.	Telemetry Lists	24

5.	Mechanical Interface	25

5.1.	Mechanical Configuration	25

5.2.	Spacecraft Interface	26

5.3.	Mass Budget	26

5.4.	Mass Properties	27

5.4.1.	Center of gravity	27

5.4.2.	Moment of inertia	28

6.	Thermal Interface	29

6.1.	Thermal Contact Area	29

6.2.	Thermal Characteristic	29

7.	Appendix	30

Figures

Figure 2.1 DTN test configuration in the first phase	13

Figure 3-1 Interface diagram of DTN payload	14

Figure 3-2 Electrical block diagram of DTN payload	14

Figure 3-3 Power scheme of DTNPL	15

Figure 3-4 DTNPL DC power connector location	16

Figure 3-5 Interface diagram for UART TM/TC	17

Figure 3-6 Interface diagram for bi-level TM/TC	18

Figure 3-7 DTNPL interface connector location	18

Figure 3-8 Interface diagram for data downlink	19

Figure 4-1 TM/TC packet structure	20

Figure 4-2 Downlink datalink packet structure	22

Figure 4-3 Datalink interface timing diagram	23

Figure 5-1 DTNPL Mechanical Configuration	26

Figure 5-2 DTNPL Exploded Configuration	26

Figure 5-3 Center of gravity	28

Figure 6-1 Thermal Contact Area	30

Tables

Table 3-1 Electrical interface summary of DTNPL	15

Table 3-2 Power connector (J01) pin assignment of DTNPL	16

Table 3-3 UART TM/TC Interface of DTNPL	16

Table 3-4 Bi-level TM/TC interface of DTNPL	17

Table 3-5 TM/TC (DMA UART, bi-level) interface connector(J02) pin assignment	18

Table 3-6 Data downlink interface of DTNPL	18

Table 4-1 DTNPL Protocols	20

Table 4-2 TM/TC Packet Format	20

Table 4-3 TM/TC data structure	21

Table 4-4 Datalink packet structure	22

Table 4-5 Contents of User Data Field	22

Table 4-6 First frame datalink structure	23

Table 4-7 Last frame datalink structure	23

Table 4-8 DTNPL Command Lists	24

Table 4-9 DTNPL Telemetry Lists	24

Table 5-1 Mass Budjet	27

Table 5-2 Center of gravity	28

Table 5-3 Moment of inertia	29

Introduction

Purpose and Scope

This document defines the electrical and mechanical interfaces between KPLO bus and DTN payload.

Applicable Documents

ETRI Documents

| Document Number | Document Title |
| --- | --- |
| KPLO-D0-550-000 | System Requirement of DTN Payload |
| KPLO-SP-550-001 | System Specification of DTN Payload |

KARI Documents

| Document Number | Document Title |
| --- | --- |
| KPLO-SP-310-001 | KPLO System Specification Document |
| KPLO-D0-500-001 | KPLO Payload Interface Requirements |
|  | DMAUART Interface Requirement for Command and Telemetry Operation of Payload (Appendix) |

Abbreviations

| AMS | Asynchronous Message Service |
| --- | --- |
| BP | Bundle Protocol |
| BSS | Bundle Streaming Service |
| CCSDS | Consultative Committee for Space Data Systems |
| CFDP | CCSDS File Delivery Protocol |
| CRC | Cyclical Redundancy Check |
| DCC | DTN Control Center |
| DMA | Direct Memory Access |
| DSA | DTN Service Adaptor |
| DTN | Delay(Disruption) Tolerant Network |
| DTNPL | DTN Payload |
| EOD | End of Data |
| EOF | End of File |
| FW | Firm Ware |
| FSW | Flight Software |
| ION | Interplanetary Overlay Network |
| KDSA | Korea Deep Space Antenna |
| KPLO | Korea Pathfinder Lunar Orbiter |
| LCM | Lander Communication Model |
| LTP | Licklider Transmission Protocol |
| MOC | Mission Operation Center |
| OBC | On Board Computer |
| OBT | On Board Time |
| PDHU | Payload Data Handling Unit |
| RCM | Rover Communication Model |
| RFU | Reserved for Future Use |
| SBC | Single Board Computer |
| SBMU | Spacecraft Bus Management Unit |
| SOD | Start of Data |
| SOF | Start of File |
| SOH | Start of Head |
| TBC | To Be Confirmed |
| TBD | To Be Defined |
| TC | Tele-command |
| TCP | Transmission Control Protocol |
| TM | Telemetry |
| UART | Universal Asynchronous Receiver Transmitter |
| USRT | Universal Synchronous Receiver Transmitter |

Overview of KPLO DTN

DTN and Space Communications

Delay/disruption tolerant networking (DTN) technology offers a new solution to highly stressed communications in space environments, especially those with long link delay and frequent link disruptions in deep space missions. Delay/disruption tolerant networking (DTN) technology offers a new solution that use a store-and-forward mechanism to combat long link delay and frequent link disruptions generally characterizing space communications. DTN communications use a bundle protocol (BP), to construct a store-and-forward overlay network. The BP, situated at the application layer of the Internet model, utilizes conventional Internet protocols to send and receive data bundles. The benefits of DTN communications are as follows.

- Interoperability and Reuse: A standardized DTN protocol suite enables interoperability of multi-agency communication assets and also allows NASA to use the same communication stack for future missions (LEO, NEO or Deep Space).

- Space Link Efficiency, Utilization and Robustness: DTN enables more reliable and efficient data transmissions resulting in more usable bandwidth. DTN also improves link reliability by having multiple network paths and assets for potential communication hops.

- Security: The DTN Bundle Security Protocol (BSP) allows for integrity checks, authentication and encryption, even on links where not previously used.

- Quality of Service: The DTN protocol suite allows for many priority levels to be set for different data types, ensuring that the most important data is received ahead of less important data.

- Improved Operations and Situational Awareness: The DTN store-and-forward mechanism along with automatic retransmission provides more insight into events during communication outages and significantly reduces the need for ground-based scheduling.

Mission of DTN Operation

The mission of DTN is to validate DTN communication protocols by space link testing through KPLO. So DTN operations are focused on testing the DTN protocols using KPLO space links.

DTN Configurations

In the first phase of lunar exploration, test lunar orbiter called Korea Pathfinder Lunar Orbiter (KPLO) will be launched without lunar lander. Figure 2.1 shows DTN configuration in first phase of lunar exploration. There are no Lander and Rover on the moon in first phase. So Lander Communication Model (LCM) and Rover Communication Model (RCM) will be located on earth for the DTN communication testing.

DTN Payload (DTNPL) is a DTN node in the space. DTNPL will be mounted on the KPLO for DTN communication. DTN Control Center (DCC) is a control center of DTN communication nodes. Lander Communication Model (LCM), Rover Communication Model (RCM), and Mission Operation Center (MOC) are DTN nodes on ground. All DTN nodes on ground are interconnected by the terrestrial network such as Internet. However, Korea Deep Space Antenna (KDSA) is not DTN node. It just transfers DTN packets to next communication nodes.

Figure 2.1 DTN test configuration in the first phase

Communication Nodes

Many communication nodes are needed for the DTN operation. Some of communication nodes in the network are DTN nodes whereas otherwise nodes are non-DTN nodes. The function of communication nodes used in the DTN operation is as follows.

DTN nodes

- DCC is a ground DTN node which monitor and controls DTN nodes. DCC send commands to DTN nodes such as DTNPL, LCM and RCM. It also receives telemetries or acquired data from some DTN nodes such as RCM or LCM.

- DTNPL is a DTN node mounted on the KPLO. DTNPL can be operated as a DTN end point. And it also can be operated as relay nodes which relay DTN data between DCC and RCM or LCM.

- LCM is a ground DTN node. LCM relays DTN data between MOC and RCM.

- RCM is a ground DTN node and a DTN end point. The commands sent by the DCC is terminated at the RCM and the RCM returns telemetry or data to DCC through KPLO.

- MOC is a ground DTN node which monitors and controls KPLO and payloads. MOC also receives and processes data generated from the payloads and transfers to each payload control center.

Non-DTN nodes

- KDSA is a ground station for deep space communication. KDSA will have a big size of antenna and RF equipment for space link interface. KDSA also has network equipment for terrestrial communication interface such as SLE link. KDSA is a non-DTN node.

Electrical Interface

The DTN payload (DTNPL) shall have interface with KPLO for the operational power supply and for the communications. The following figure shows DTN payload interface diagram with KPLO.

Figure 3-1 Interface diagram of DTN payload

There are four kinds of interface types between DTNPL and KPLO. Each interface types are described as following.

- TM/TC (bi-level): One bi-level TC and one bi-level TM for DTNPL emergency operation.

- Bi-level TC: System reset command in the emergence condition

- Bi-level TM: Telemetry showing that DTNPL is in alive

- TM/TC (UART): Serial communication channel for TM/TC communication with KPLO SBMU

- Data downlink (USRT): Serial data downlink channel to KPLO PDHU

- Power: Operation DC power +28V from the KPLO PCDU

The detailed electrical configuration of DTNPL is shown below figure.

Figure 3-2 Electrical block diagram of DTN payload

The following table shows characteristics of electrical interface of DTNPL.

Table 3-1 Electrical interface summary of DTNPL

|  | Type | Mnemonic | Physical I/F | Direction | Description | Connector |
| --- | --- | --- | --- | --- | --- | --- |
| Power | +28V | DTNPL_+28V | Nominal +28V (24~32.8)Vdc | Bus(PCDU) DTNPL(SBC) | DTN Power | CON_PWR |
| TM/TC (TLM / CMD) | Bi-level TM | DTNPL_STATUS | Bi-level TM - Logic LOW : 0V - Logic HIGH : 5V | Bus(SBMU) DTNPL(SBC) | 0.25 Hz Heartbeat | CON_COMM |
|  | Bi-level TC | DTNPL_RESET | Bi-level TC - Logic LOW : 0V - Logic HIGH : 5V | Bus(SBMU) DTNPL(SBC) | DTN Run (0V, Default) DTN Reset (5V) Duration>1msec. |  |
| TM/TC (SBMU) | DMA_UART | DTNPL_SBMU_UART | RS422 Differential | Bus(SBMU) DTNPL(SBC) | 115.2 kbps |  |
| TM/Data downlink (PDHU) | USRT | DTNPL_PDHU_UART | RS422 Differential | Bus(PDHU) DTNPL(SBC) | 2.0 Mbps |  |

Power Interface

The DTNPL has the DC power interface to get operation power from the KPLO (+28 V). Secondary power values of DTNPL are +5V, +3.3V and +1.2V..

Figure 3-3 Power scheme of DTNPL

- Power consumption: The maximum power consumption of DTNPL does not exceed 12 W.

- Operation duty: DTNPL will be operated in orbit irregularly. So, it is difficult to define operation duty. For example, DTNPL can be operated as following;

- SOM + 2 weeks: DTNPL operation time is 40 minutes twice per day

- Until MOM, DTNPL will be non-operated

- MOM + 2 weeks: DTNPL operation time is 40 minutes twice per day

- Until EOM, DTNPL will be non-operated

- EOM + 2 weeks: DTNPL operation time is 40 minutes twice per day

- SOM: Start of Mission

- MOM: Middle of Mission

- EOM: End of Mission

Table 3-2 Power connector (J01) pin assignment of DTNPL

| Pin | Description | Pin | Description | Pin | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | DTNPL_+28V | 6 | (Reserved) | 11 | DTNPL_+28V_rtn |
| 2 | DTNPL_+28V | 7 | (Reserved) | 12 | DTNPL_+28V_rtn |
| 3 |  | 8 | (Reserved) | 13 |  |
| 4 |  | 9 | (Reserved) | 14 |  |
| 5 |  | 10 | (Reserved) | 15 |  |

Detailed Connector Information is as follow.

- Part Type : HD15 Male

- Part Number : 280-018P1H15Z2GP

- Vendor : Glenair

The DC power connector location is shown below.

Figure 3-4 DTNPL DC power connector location

TM/TC (DMA UART, Bi-level) Interface

TM/TC (DMA UART) Interface is as below.

Table 3-3 UART TM/TC Interface of DTNPL

| Type | DMA_UART |
| --- | --- |
| Mnemonic | DTNPL_SBMU_UART |
| Physical I/F | RS422 Differential |
| Direction | Bus(SBMU) DTNPL(SBC) |
| Description | 115.2 kbps |

Figure 3-5 Interface diagram for UART TM/TC

TM/TC (bi-level) Interface is as below.

Table 3-4 Bi-level TM/TC interface of DTNPL

| Type | Bi-level TM | Bi-level TC |
| --- | --- | --- |
| Mnemonic | DTNPL_STATUS | DTNPL_RESET |
| Physical I/F | Logic LOW: 0V Logic HIGH: 5V | Logic LOW: 0V Logic HIGH: 5V |
| Direction | Bus(SBMU) DTNPL(SBC) | Bus(SBMU) DTNPL(SBC) |
| Description | 0.25 Hz (Duty 50:50) Heartbeat | DTN Run (0V, Default) DTN Reset (5V) |

Figure 3-6 Interface diagram for bi-level TM/TC

TM/TC (DMA UART, bi-level) Interface Connector Pin map is as below.

Table 3-5 TM/TC (DMA UART, bi-level) interface connector(J02) pin assignment

| Pin | Description | Pin | Description | Pin | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | DTNPL_UART_SBMU_Tx_DTNPL_Rx_P | 10 | DTNPL_PDHUp_Rx_DTNPL_TxEn_P | 19 | (Reserved) |
| 2 | DTNPL_UART_SBMU_Tx_DTNPL_Rx_N | 11 | DTNPL_PDHUp_Rx_DTNPL_TxEn_N | 20 | (Reserved) |
| 3 | DTNPL_UART_SBMU_Rx_DTNPL_Tx_P | 12 | (Reserved) | 21 | (Reserved) |
| 4 | DTNPL_UART_SBMU_Rx_DTNPL_Tx_N | 13 | DTNPL_PDHUr_Rx_DTNPL_TxData_P | 22 | (Reserved) |
| 5 | (Reserved) | 14 | DTNPL_PDHUr_Rx_DTNPL_TxData_N | 23 | DTNPL_STATUS |
| 6 | DTNPL_PDHUp_Rx_DTNPL_TxData_P | 15 | DTNPL_PDHUr_Rx_DTNPL_TxClk_P | 24 | DTNPL_STATUS_rtn |
| 7 | DTNPL_PDHUp_Rx_DTNPL_TxData_N | 16 | DTNPL_PDHUr_Rx_DTNPL_TxClk_N | 25 | DTNPL_RESET |
| 8 | DTNPL_PDHUp_Rx_DTNPL_TxClk_P | 17 | DTNPL_PDHUr_Rx_DTNPL_TxEn_P | 26 | DTNPL_RESET_rtn |
| 9 | DTNPL_PDHUp_Rx_DTNPL_TxClk_N | 18 | DTNPL_PDHUr_Rx_DTNPL_TxEn_N |  |  |

Detailed Connector Information is as follow.

- Part Type : HD26 Female

- Part Number : 280-019S2H26Z2GP

- Vendor : Glenair

Figure 3-7 DTNPL interface connector location

Data Downlink Interface

Data Downlink Interface with PDHU is as below.

Table 3-6 Data downlink interface of DTNPL

| Type | USRT |
| --- | --- |
| Mnemonic | DTNPL_PDHU_USRT |
| Physical I/F | RS422 Differential |
| Direction | Bus(PDHU) DTNPL(SBC) |
| Description | 2.0 Mbps |

Figure 3-8 Interface diagram for data downlink

Communication Protocols

There are two kinds of communication protocols between DTNPL and KPLO as following.

Table 4-1 DTNPL Protocols

|  | TM/TC Protocol | Datalink Protocol |
| --- | --- | --- |
| Communication link | DTNPL ↔ SBMU(KPLO) | DTNPL → PDHU(KPLO) |
| Application | TM/TC communication (SBMU ↔ DTNPL) File transfer using DTN (SBMU → DTNPL) | File transfer using DTN (DTNPL → PDHU) Video streaming using DTN (DTNPL → PDHU) |
| ACK | OK check | None |
| Single/Multiple | Single packet: TM/TC Multiple packets: File transfer | Multiple packets |

TM/TC Protocols

UART Protocol

DMA UART will be used for the communication between DTNPL and KPLO SBMU.

- Baud rate: 115.2 kbps

- None-Parity, 8 Data bit and 1 Start/Stop bit (N, 8, 1)

- ACK method: OK check

  - ACK: 0x12 (1 byte)

  - ACK response time: within 100us

- ID pattern: 0xAA

Time interval between the commands are 125ms as a minimum.

So, DTNPL have to handle sequential commands at 125ms interval.

Packet Format

The maximum packet size is 70 byte. The TM/TC packet is consists of following fields.

Figure 4-1 TM/TC packet structure

The packet fields are defined in following table.

Table 4-2 TM/TC Packet Format

| Field name | Size | Description | Value |
| --- | --- | --- | --- |
| Sync | 1 byte | Sync pattern for UART communication | 0xAA |
| Length | 2 byte | Length of data field (byte) | 0 (0x0000) ~ 65 (0x0041) |
| Command | 1 byte | Transmit Channel (1 bit) | SBC (0), DTN (1) |
|  |  | Response Channel (1 bit) | SBMU (0), PDHU (1) |
|  |  | Type (2 bits) | SBC TC (00 b), SBC TM (01 b) |
|  |  | Command number (4 bits) | 0 (0000 b) ~ 15 (1111 b) |
| Pkt no | 1 byte | Sequence number of DTN Segment (only used in DTN data traffic, not in TC/TM) | 0 (0x00) ~ 255 (0xFF) in DTN |
|  |  |  | 0 (0x00) in TC/TM |
| Data | Variable (max. 65 byte) | - DTN data from DCC to DTNPL - TC from MOC to DTNPL - TM from DTNPL to SBMU/MOC |  |

TM/TC Data Structure4

TM/TC data structure is as following.

Table 4-3 TM/TC data structure

| Type | Name | Sync | Length | Command | Packet No | Payload Data | Corresponding Action |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TC | Time setting |  | 0x0006 | 0x02 | 0x00 | OBT - week since 2000-01-01 12:00:00 UTC (2 byte) - octant second (4 byte) | ACK or NACK |
|  | SOH request |  | 0x0000 | 0x03 | 0x00 | None | SOH data or NACK |
|  | Script1 (ionstart, killm) |  | Variable (max.) - ionstart : 0x000E - killm : 0x0009 | 0x04 | 0x00 | Script command - ionstart: max. 14 byte - killm: max. 9 byte | ACK or NACK |
|  | Script2 (DCC data) |  | Variable - 0x0000~0x0041 | 0xC6 | 0x00 ~ 0xFF | DCC TC/data (0 ~ 65 byte) | ACK or NACK |
| SBC TM | SOH data | 0xAA | 0x41 | 0x14 | 0x00 | SOH data (65 byte) |  |
|  | NACK |  | 0x41 | 0x15 | 0x00 | Reserved (65 byte) | Delete |
|  | ACK |  | 0x41 | 0x16 | 0x00 | Reserved (65 byte) | Delete |

Datalink Protocol

Datalink protocol is used for payload data transfer from DTNPL to PDHU. USRT will be used for the datalink protocol and characteristics are as following.

Datalink protocol for downlink

The downlink datalink packet structure can be expressed logically as following.

- Packet structure = VN + Type + SHF + APID + SF + PSC + PL + SP + User data + CRC

  - Where; VN: Version Number, SHF: Secondary Header Flag, APID: Application Process ID, SF: Sequence Flags, PSC: Packet Sequence Count, PL: Packet Length, SP: Sync Pattern, CRC: Cyclic Redundancy Check

The downlink datalink packet structure can be expressed as following figure.

Figure 4-2 Downlink datalink packet structure

The datalink packet structure is as following.

Table 4-4 Datalink packet structure

| Field name | Size (bit) | Description | Comments |
| --- | --- | --- | --- |
| VN | 3 | 000b | Version 1 |
| Type | 1 | 0b | Telemetry |
| SHF | 1 | 1b |  |
| APID | 11 | 11 bits APID is determined by the destination EID of DTN segment received from DTNPL-ION |  |
| SF | 2 | 11b (standalone) or combination of 01b / 00b / 10b (first / continue / last) |  |
| PSC | 14 | Modulo-16384, 0 ~ 16383 | The sequential binary count of each packet |
| PL | 16 | (Length of SP)+(Length of User Data Field)+(Length of CRC) -1 |  |
| SP | 32 | 0x352EF853 |  |
| User Data Field | Variable | Max 65536 Octets. Refer to Table 4-5. |  |
| CRC | 16 | CRC-16-CCITT (Polynomial: x16+x12+x5+1) Calculation field: (SP)+(User Data Filed) |  |

The contents of User Data Field are as following.

Table 4-5 Contents of User Data Field

| Sub-field | Size (byte) | Description | Comments |
| --- | --- | --- | --- |
| Sync | 1 | Sync pattern for USRT communication |  |
| Length | 2 | Length of Data field in User Data Field (byte) |  |
| Command | 1 | Same to command field of TM/TC packet. Refer to Table 4-2 |  |
| Packet No. | 1 | Same to Pkt no field of TM/TC packet. Refer to Table 4-2 |  |
| Data | Variable (Max. 2043) | Variable. Max. 2043 Octets. | - message - File data - Streaming data |

Datalink Data Structure

Datalink data structure is as following.

Table 4-6 First frame datalink structure

| VN | Type | SHF | APID | SF | PSC | PL | SP | User Data Field | CRC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 000b | 0b | 1b | xxx xxxx xxxxb | 01b | 00 0000 0000 0000b | xxxx xxxx …. | 0x352EF853 | Variable |  |

Table 4-7 Last frame datalink structure

| VN | Type | SHF | APID | SF | PSC | PL | SP | User Data Field | CRC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 000b | 0b | 1b | xxx xxxx xxxxb | 10b | xx xxxx xxxx xxxxb | xxxx xxxx …. | 0x352EF853 | Variable |  |

Interface Timing Diagram

The interface timing diagram for datalink is as following.

Figure 4-3 Datalink interface timing diagram

Clock-synchronized data should be transferred during the enable signal is in high state.

CLK signal should be continue 16 cycles after enable signal is low state for the PDHU post processing.

Fill pattern should be transferred, when data is absent.

Data would be sampled at clock rising-edge.

TM/TC Lists

Command Lists

There are two kinds of command types in the DTNPL as following.

- SBC commands: DTNPL control

- DTN commands: ION-DTN control. They are transmitted by AMS from DCC to DTNPL.

The DTNPL command lists are shown in following table.

Table 4-8 DTNPL Command Lists

| Type | TC name | Description |
| --- | --- | --- |
|  | Time setting | Set On Board Time |
|  | SOH request | Request SOH TM |
|  | Script (ion start) | ION Start |
|  | Script (killm) | Close all process |
| DTN | DCC data | BPM TC / CFDP file data transfer |

Telemetry Lists

There are three kinds of telemetry types as following.

- SBC telemetries: The SOH (State Of Health) of DTN payload to be reported to the KPLO FSW to monitor DTNPL health condition.

- DTN telemetries: Response to DTN command. They are transmitted by BP message from DTNPL to DCC.

The detailed telemetry lists are shown in below.

Table 4-9 DTNPL Telemetry Lists

| Type | TM name | Description | Size (byte) | Start time (sec.) | Update interval | Transmit interval to FSW |
| --- | --- | --- | --- | --- | --- | --- |
| SBC | SOH Data | temp_board (Board temperature) | 4 | Auto count after DTNPL power ON | <1 sec. | 32 sec. (FSW send SOH request TC to DTNPL at every 32 sec.) |
|  |  | temp_fpga (FPGA temperature) | 4 |  | <1 sec. |  |
|  |  | volt_28 (28.0V input voltage level) | 2 |  | <1 sec. |  |
|  |  | volt_3.3 (3.3V voltage level) | 2 |  | <1 sec. |  |
|  |  | volt_1.2 (1.2V voltage level) | 2 |  | <1 sec. |  |
|  |  | curr_28 (28.0V input current level) | 2 |  | <1 sec. |  |
|  |  | time (Current DTNPL time) | 6 |  | 1 sec. |  |
|  |  | run_time (Time(second) count after DTNPL power on) | 4 |  | 1 sec. |  |
|  |  | fpga_ver (SBC FPGA version) | 1 |  | Fixed |  |
|  |  | fw_ver (SBC firmware version) | 1 |  | Fixed |  |
|  |  | edac (EDAC count) | 4 |  | Event occurred |  |
|  |  | tmtc_rx_pkt_cnt (SBMU→DTNPL received packet count) | 4 |  | When packet received |  |
|  |  | tmtc_tx_pkt_cnt (DTNPL→SBMU send packet count) | 4 |  | When packet sent |  |
|  |  | downlink_pkt_cnt (DTNPL→PDHU send packet count) | 4 |  | When packet sent |  |
|  |  | dtn_state (ION start/stop state) | 1 |  | 1 sec. |  |
|  |  | ion_ver (ION version) | 2 |  | Fixed |  |
|  | NACK | NACK response | 0 | - | NA | Delete |
|  | ACK | ACK response | 0 | - | NA |  |

Mechanical Interface

Mechanical Configuration

The DTNPL consisted of 1 SBC Board, 1 Housing and 1 Top Cover.

Total Mass: 0.79kg (Tolerance ±5%)

Dimension: W x L x H = 120 x 170 x 39 mm

Figure 5-1 DTNPL Mechanical Configuration

Figure 5-2 DTNPL Exploded Configuration

Spacecraft Interface

The DTNPL is fixed by six M4 bolts on Spacecraft

Mass Budget

Table 5-1  Mass Budjet

| Name | Mass [kg] | Remark |
| --- | --- | --- |
| Housing | 0.37 | Measured weight |
| Top Cover | 0.23 |  |
| SBC PCB+EEE Parts | 0.15 |  |
| etc (bolts…) | 0.04 |  |
| Total Sum | 0.79 | Tolerance ±5% |

Mass Properties

Center of gravity

Figure 5-3 Center of gravity

Table 5-2 Center of gravity

| Axis | Location |
| --- | --- |
| X | 85.35 |
| Y | 60.66 |
| Z | 19.61 |

Moment of inertia

Table 5-3 Moment of inertia

| Moment of inertia |  |
| --- | --- |
| Ixx [ kg-m2 ] | 0.004 |
| Iyy [ kg-m2 ] | 0.008 |
| Izz [ kg-m2 ] | 0.011 |
| Ixy [ kg-m2 ] | -0.004 |
| Iyz [ kg-m2 ] | -0.001 |
| Ixz [ kg-m2 ] | -0.00874 |

Thermal Interface

The DTNPL monitor temperature of FPGA (Hot point) and PCB (Cold point) using its temperature sensing circuit.

No thermistors are required from the spacecraft side for the temperature monitor.

No thermal fillers are required between the bottom side of DTNPL and spacecraft panel during the equipment mounting.

Thermal Contact Area

The area of the thermal contact surface of DTN PL is 0.018m2, and the shape is shown in the figure below.

Figure 6-1 Thermal Contact Area

Thermal Characteristic

Emissivity[%] : 0%

Thermal Capacity : 540[J/K]

Thermal Dissipation : 4.768[W]

Appendix

The following document is a DMAUART interface requirement for the command and telemetry operation of payload in the KPLO. For the full document view, click the page.