Korea Lunar Exploration Program

DTNPL Technical Description

| Date: 29 Oct. 2018 | Doc. No: KPLO-D0-550-100 Issue: 1.0 Total page: 23 |
| --- | --- |

Superseding     Revised

| Prepared By: June-Tae Park Date DTN SBC H/W Engineer, Lumir Min-Woo Park Date DTN SBC S/W Engineer, Lumir |  | Jeong-Chan Lee Date DTN SBC Mechanical Engineer, Lumir |
| --- | --- | --- |
| Reviewed By: Chang-Soo Lee Date DTN SBC Product Assurance, Lumir |  | Dae-Soo Oh Date DTN SBC Project Manager, Lumir |
| Kyung-Rak Lee Date DTN Payload Engineer, ETRI |  |  |
| Approval Signature: Choel-Oh Jeong Date DTN Payload Product Assurance, ETRI |  | Jin-Ho Jo Date DTN Payload Project Leader, ETRI |

Original CDMO Release__________________

| Revision History |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version | Author(s) |  | Date | Description |
| 0.1 | Jae-Won Jang |  | 2017. 12. 28 | Draft |
| 1.0 | June-Tae Park Jeong-Chan Lee Min-Woo Park |  | 2018. 10. 29 | Final |

| Revision/Change Records |  |  |  |
| --- | --- | --- | --- |
| Version | Date | Revision/Change Description | Pages |
| 0.1 | 2017. 12. 28 | Initial | All |
| 1.0 | 2018. 10. 29 | Final | All |

Documents Tree

Contents

1.	Introduction	8

1.1.	Purpose and Scope	8

1.2.	Applicable Documents	8

1.2.1.	ETRI Documents	8

1.2.2.	Other Documents	8

1.3.	Abbreviations	8

2.	Operation Description	9

2.1.	DTN and Space Communications	9

2.2.	Mission of DTN Operation	9

2.3.	DTN Configurations	9

2.4.	Communication Nodes	10

2.4.1.	DTN nodes	10

2.4.2.	Non-DTN nodes	10

3.	System Configuration of DTN Payload	11

4.	Technical Description	12

4.1.	Functional Block diagram	12

4.2.	Major Performance	12

4.3.	Electrical Interface	13

4.3.1.	Power Interface	13

4.3.2.	Communication Interface	14

4.3.3.	House Keeping Design	17

4.4.	EEE Parts List	18

4.5.	FPGA Design	19

4.6.	Schematic Design	21

4.7.	PCB artwork	22

Figures

Figure 2-1 DTN test configuration in the first phase	10

Figure 3-1 Interface diagram of DTN payload	11

Figure 4-1 DTNPL Functional Block Daigram	12

Figure 4-2 Power Interface	14

Figure 4-3 Power Connetor Location	14

Figure 4-4 TM/TC UART Interface Scheme	15

Figure 4-5 TM Bi-level Interface Scheme	15

Figure 4-6 TC Bi-level Interface Scheme	16

Figure 4-7 Data Downlink Interface Scheme	16

Figure 4-8 Communication Connector Location	17

Figure 4-9 House Keeping Scheme	18

Figure 4-10 FPGA Block Diagram	19

Figure 4-11 Schematic Design	21

Figure 4-12 Artwork	22

Tables

Table 4-1 DTNPL Major Performance	12

Table 4-2 DTNPL Electrical Interface	13

Table 4-3 Power Connector Pin Map	14

Table 4-4 TM/TC UART Interface Description	15

Table 4-5 TM/TC Bi-level Interface Description	15

Table 4-6 Data Downlink Interface Description	16

Table 4-7 Communication Connector Pin Map	17

Table 4-8 DTNPL Parts List	19

Table 4-9 FPGA Function Description	20

Table 4-10 FPGA Gate Estimation	21

Table 4-11 Schematic Description	22

Table 4-12 PCB Stack Up Information	23

Introduction

Purpose and Scope

This document describes the system specifications of DTN payload for Korea lunar exploration program.

Applicable Documents

ETRI Documents

| Document Number | Document Title |
| --- | --- |
| KPLO-D0-551-002 | Operation Scenario of DTN |
| KPLO-D0-550-000 | System Requirements of DTN Payload |

Other Documents

| Document Number | Document Title |
| --- | --- |
| KPLO-SP-300-002 | KPLO System Requirements Document |

Abbreviations

| AMS | Asynchronous Message Service |
| --- | --- |
| AOS | Advanced Orbiting System |
| BP | Bundle Protocol |
| BSS | Bundle Streaming Service |
| CCSDS | Consultative Committee for Space Data Systems |
| CFDP | CCSDS File Delivery Protocol |
| DCC | DTN Control Center |
| DTN | Delay(Disruption) Tolerant Network |
| DTNPL | DTN Payload |
| IP | Internet Protocol |
| KPLO | Korea Pathfinder Lunar Orbiter |
| LCM | Lander Communication Model |
| LTP | Licklider Transmission Protocol |
| M&C | Monitor & Control |
| RCM | Rover Communication Model |
| SLE | Space Link Extension |
| TBC | To Be Confirmed |
| TBD | To Be Defined |
| TCP | Transmission Control Protocol |
| UDP | User Datagram Protocol |

Operation Description

DTN and Space Communications

Delay/disruption tolerant networking (DTN) technology offers a new solution to highly stressed communications in space environments, especially those with long link delay and frequent link disruptions in deep space missions. Delay/disruption tolerant networking (DTN) technology offers a new solution that use a store-and-forward mechanism to combat long link delay and frequent link disruptions generally characterizing space communications. DTN communications use a bundle protocol (BP), to construct a store-and-forward overlay network. The BP, situated at the application layer of the Internet model, utilizes conventional Internet protocols to send and receive data bundles. The benefits of DTN communications are as follows.

- Interoperability and Reuse: A standardized DTN protocol suite enables interoperability of multi-agency communication assets and also allows NASA to use the same communication stack for future missions (LEO, NEO or Deep Space).

- Space Link Efficiency, Utilization and Robustness: DTN enables more reliable and efficient data transmissions resulting in more usable bandwidth. DTN also improves link reliability by having multiple network paths and assets for potential communication hops.

- Security: The DTN Bundle Security Protocol (BSP) allows for integrity checks, authentication and encryption, even on links where not previously used.

- Quality of Service: The DTN protocol suite allows for many priority levels to be set for different data types, ensuring that the most important data is received ahead of less important data.

- Improved Operations and Situational Awareness: The DTN store-and-forward mechanism along with automatic retransmission provides more insight into events during communication outages and significantly reduces the need for ground-based scheduling.

Mission of DTN Operation

The mission of DTN is to validate DTN communication protocols by space link testing through KPLO. So DTN operations are focused on testing the DTN protocols using KPLO space links.

DTN Configurations

In the first phase of lunar exploration, test lunar orbiter called Korea Pathfinder Lunar Orbiter (KPLO) will be launched without lunar lander. Figure 2.1 shows DTN configuration in first phase of lunar exploration. There are no Lander and Rover on the moon in first phase. So Lander Communication Mode (LCM) and Rover Communication Mode (RCM) will be located on earth for the DTN communication testing.

DTN Payload (DTNPL) is a DTN node in the space. DTNPL will be mounted on the KPLO for DTN communication. DTN Control Center (DCC) is a control center of DTN communication nodes. LCM and RCM are DTN nodes on ground. All DTN nodes on ground are interconnected by the terrestrial network such as Internet. MCC and DSN are not DTN nodes. They just transfer DTN packets to next communication nodes.

Figure 2-1 DTN test configuration in the first phase

Communication Nodes

Many communication nodes are needed for the DTN operation. Some of communication nodes in the network are DTN nodes whereas otherwise nodes are non-DTN nodes. The function of communication nodes used in the DTN operation is as follows.

DTN nodes

- DCC is a ground DTN node which monitor and controls DTN nodes. DCC send commands to DTN nodes such as DTNPL, LCM and RCM. It also receives telemetries or acquired data from some DTN nodes such as RCM or LCM.

- DTNPL is a DTN node mounted on the KPLO. DTNPL can be operated as a DTN end point. And it also can be operated as relay nodes which relay DTN data between DCC and RCM or LCM.

- LCM is a ground DTN node. LCM relays DTN data between MCC and RCM.

- RCM is a ground DTN node and a DTN end point. The commands sent by the DCC is terminated at the RCM and the RCM returns telemetry or data to DCC through KPLO.

Non-DTN nodes

- MCC is a non-DTN node. MCC monitors and controls KPLO and payloads. MCC also receives and processes data generated from the payloads and transfers to each payload control center.

- DSN is a ground station for deep space communication. DSN will have a big size of antenna and RF equipment for space link interface. DSN also have network equipment for terrestrial communication interface such as SLE link. DSN is a non-DTN node.

System Configuration of DTN Payload

The DTN payload shall have interface with KPLO for the operational power supply and for the communications. The following figure shows DTN payload interface diagram with KPLO.

Figure 3-1 Interface diagram of DTN payload

There are four kinds of interface types between DTNPL and KPLO. Each interface types are described as following.

- TM/TC (bi-level): One bi-level TC and one bi-level TM for DTNPL emergency operation.

- Bi-level TC: System reset command in the emergence condition

- Bi-level TM: Telemetry showing that DTNPL is in alive

- TM/TC (UART): Serial communication channel for TM/TC communication with KPLO SBMU

- Data downlink: Serial data downlink channel to KPLO PDHU

- Power: Operation DC power +28V from the KPLO PCDU

Technical Description

Functional Block diagram

Figure 4-1 DTNPL Functional Block Daigram

Major Performance

Table 4-1 DTNPL Major Performance

| Item | Design Specification | Specifications |
| --- | --- | --- |
| Processor | 32-bit Leon3 |  |
| OS | RTEMS v4.11 |  |
| Memory | 64 Mbits for Booting and Program |  |
|  | 2.5 Gbits for Code Execution |  |
|  | 4 GB (Non Volatile) for Mission Data | > 2 GB |
| TM/TC (SBMU I/F) | DMA_UART (RS422) | DMA_UART (RS422) |
| Downlink (PDHU I/F) | USRT (RS422) | USRT (RS422) |
| Analog I/F | 8 ch | 8 ch |
| Input Power | 28 V Unregulated (20 VDC ~ 40 VDC) | 28 V Unregulated (24 VDC ~ 32.8 VDC) |
| Power Consumption | <6 W | < 12 W |
| Mass | < 1 kg | < 2 kg |
| Size (WxLxH) | 120 x 170 x 39 mm | < 120 x 170 x 50 mm |
| Reliability | 0.878063 (Analyzed) | > 0.7 |
| Debug | 1EA: JTAG (Firmware Uploader) 1EA: RS422 (Debugging UART) 1EA: SpaceWire (Image Uploader) 4EA: GPIO (Test) 1EA: SPI (S/W 동적테스트 장비(DT10) 연동) |  |

Electrical Interface

Table 4-2 DTNPL Electrical Interface

|  | Type | Mnemonic | Physical I/F | Direction | Description | Connector |
| --- | --- | --- | --- | --- | --- | --- |
| Power | +28V | DTNPL_+28V | Nominal +28Vdc | Bus (PCDU) DTNPL | DTN Power | CON_PWR |
| TM/TC | Bi-level TM | DTNPL_STATUS | Bi-level TM - Logic LOW : 0V - Logic HIGH : 5V | Bus (SBMU) DTNPL | 0.25 Hz Heartbeat | CON_COMM |
|  | Bi-level TC | DTNPL_RESET | Bi-level TC - Logic LOW : 0V - Logic HIGH : 5V | Bus (SBMU) DTNPL | DTN Run (0V) DTN Reset (5V) |  |
| TM/TC (SBMU) | DMA_UART | DTNPL_SBMU_UART | RS422 Differential | Bus (SBMU) DTNPL | 115.2 kbps |  |
| Data downlink (PDHU) | USRT | DTNPL_PDHU_USRT | RS422 Differential | Bus (PDHU) DTNPL | 2.0 Mbps |  |

Power Interface

The DTNPL has the DC power interface to get operation power from the KPLO (nominal +28Vdc). Secondary power values of DTNPL are 3.3V, 2.5V, 1.8V, 1.2V and 5V for precision voltage reference.

Figure 4-2 Power Interface

Table 4-3 Power Connector Pin Map

| High Density D-SUB 15-Pin (Male) Allocation List |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Pin | Description | Pin | Description | Pin | Description |
| 1 | DTNPL_+28V | 6 | (Reserved) | 11 | DTNPL_+28V_rtn |
| 2 | DTNPL_+28V | 7 | (Reserved) | 12 | DTNPL_+28V_rtn |
| 3 | DTNPL_+28V | 8 | (Reserved) | 13 | DTNPL_+28V_rtn |
| 4 | DTNPL_+28V | 9 | (Reserved) | 14 | DTNPL_+28V_rtn |
| 5 | DTNPL_+28V | 10 | (Reserved) | 15 | DTNPL_+28V_rtn |

Figure 4-3 Power Connetor Location

CON_PWR

Communication Interface

The DTNPL has 1 UART for SBMU, 1 Bi-level for Status, 1 Bi-level for Command and 2 USART for PDHU(p/r).

TM/TC (DMC UART) interface is as below.

Table 4-4 TM/TC UART Interface Description

| Type | DMA_UART |
| --- | --- |
| Mnemonic | DTNPL_SBMU_UART |
| Physical I/F | RS422 Differential |
| Direction | Bus(SBMU) DTNPL(SBC) |
| Description | 115.2 kbps Echo back |

Figure 4-4 TM/TC UART Interface Scheme

TM/TC (Bi-level) interface is as below.

Table 4-5 TM/TC Bi-level Interface Description

| Type | Bi-level TM | Bi-level TC |
| --- | --- | --- |
| Mnemonic | DTNPL_STATUS | DTNPL_RESET |
| Physical I/F | Logic LOW: 0V Logic HIGH: 5V | Logic LOW: 0V Logic HIGH: 5V |
| Direction | Bus(SBMU) DTNPL(SBC) | Bus(SBMU) DTNPL(SBC) |
| Description | 0.25 Hz Heartbeat | DTN Run (0V, Default) DTN Reset (5V) |

Figure 4-5 TM Bi-level Interface Scheme

Figure 4-6 TC Bi-level Interface Scheme

Data downlink interface with PDHU is as below.

Table 4-6 Data Downlink Interface Description

| Type | USART |
| --- | --- |
| Mnemonic | DTNPL_PDHU_UART |
| Physical I/F | RS422 Differential |
| Direction | Bus(PDHU) DTNPL(SBC) |
| Description | 2.0 Mbps |

Figure 4-7 Data Downlink Interface Scheme

Table 4-7 Communication Connector Pin Map

| Pin | Description | Pin | Description |
| --- | --- | --- | --- |
| 1 | DTNPL_JTAG_TDI | 14 | DTNPL_DBG_SpW_RxDp |
| 2 | DTNPL_JTAG_TDO | 15 | DTNPL_DBG_SpW_RxDn |
| 3 | DTNPL_JTAG_TCK | 16 | DTNPL_DBG_SpW_RxSp |
| 4 | DTNPL_JTAG_TMS | 17 | DTNPL_DBG_SpW_RxSn |
| 5 | DTNPL_JTAG_TRSTB | 18 | DTNPL_GPIO |
| 6 | DTNPL_JTAG_GND | 19 | DTNPL_UART422_TXp |
| 7 | DTNPL_JTAG_GND | 20 | DTNPL_UART422_TXn |
| 8 | DTNPL_JTAG_+3.3V | 21 | DTNPL_UART422_RXp |
| 9 | DTNPL_JTAG_+3.3V | 22 | DTNPL_UART422_RXn |
| 10 | DTNPL_DBG_SpW_TxDp | 23 | DTNPL_DT10_SCK |
| 11 | DTNPL_DBG_SpW_TxDn | 24 | DTNPL_DT10_SDI |
| 12 | DTNPL_DBG_SpW_TxSp | 25 | DTNPL_DT10_nCS |
| 13 | DTNPL_DBG_SpW_TxSn | 26 | DTNPL_DT10_CNG |

Figure 4-8 Communication Connector Location

CON_COMM

House Keeping Design

The DTNPL has a function of the House Keeping as below.

- Power consumption monitoring (+28V current)

- Input voltage monitoring (+28V voltage)

- Secondary voltage monitoring (+3.3V voltage)

- FPGA core voltage monitoring (+1.2V voltage)

- FPGA temperature monitoring

- Board temperature monitoring

If abnormal case, DTNPL does

- Stores H.K data to NAND flash(Black box area)

- Goes to Standby mode

- Does NOT send heartbeat to SBMU (0.25Hz  0Hz)

Figure 4-9 House Keeping Scheme

EEE Parts List

Table 4-8 DTNPL Parts List

FPGA Design

Figure 4-10 FPGA Block Diagram

Table 4-9 FPGA Function Description

| FPGA IP | Description |
| --- | --- |
| LEON3 | 32-bit Leon3 Processor |
| AHBCTRL | AHB Controller |
| APBCTRL | APB Controller |
| APBUART | UART Controller using APB Bus |
| DSU | Debugging Support Unit |
| DMA_UART | UART Controller for TM/TC I/F |
| USRT | USRT Controller for PDHU I/F |
| Memory Controller | Memory Controller |
| Watchdog, IRQ CTRL | System monitoring and status informing |
| GPIO | General Purpose I/O Control |
| Timer | Timing Synchronization |
| A/D Logic | Convert Analog to Digital Block |
| Firmware Uploader | LEON3 S/W Uploader |

Table 4-10 FPGA Gate

| IP Core | Vendor | Description | 크기 |  | 개 수 | 크기합계 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell | RAM 24K |  | Cell | RAM 24K |
| LEON3 | Gaisler | 32-bit Leon3 Processor | 5900 | 75 | 1 | 5900 | 75 |
| AHBCTRL | Gaisler | AHB Controller | 600 | 0 | 1 | 600 | 0 |
| APBCTRL | Gaisler | APB Controller | 400 | 0 | 1 | 400 | 0 |
| APBUART | Gaisler | UART Controller using APB BUS | 350 | 0 | 1 | 350 | 0 |
| DSU | Gailser | Debugging Support Unit | 1100 | 2 | 1 | 1100 | 2 |
| DMA_UART | Lumir | DMA Controller for TM/TC I/F | 450 | 0 | 1 | 450 | 0 |
| USRT | Lumir | USRT Controller for PDHU I/F | 450 | 0 | 1 | 450 | 0 |
| Memory Controller | Gaisler | Memory controller | 3,200 | 50 | 1 | 3,200 | 50 |
| Watchdog, IRQ CTRL | Gaisler | System monitoring and status informing | 340 | 0 | 1 | 340 | 0 |
| GPIO | Gaisler | General Purpose I/O Control | 390 | 0 | 1 | 390 | 0 |
| Time Sync | Lumir | Timing Synchronization according to PPS | 220 | 0 | 1 | 220 | 0 |
| A/D Logic | Lumir | Convert Analog to Digital Block | 1610 | 2 | 1 | 1610 | 2 |
| Firmware Uploader | Lumir | LEON3 S/W Uploader | 2040 | 5 | 1 | 2040 | 5 |
| 합계 |  |  |  |  |  | 17,950 | 134 |
| RT4G150 Cell Capacity |  |  |  |  |  | 180,728 | 239 |
| FPGA 전체 점유율 |  |  |  |  |  | 10% | 56% |

Schematic Design

- Design tool : Altium 16.1.7

- Total 15 sheets

Figure 4-11 Schematic Design

Table 4-11 Schematic Description

| Sheet # | Title | Description |
| --- | --- | --- |
| 0 | Cover | Board Information |
| 1 | Connectors | External connectors and Buffer |
| 2 | POWERCLK | Regulator, OSC |
| 3 | FPGA_B0B1 | FPGA Bank0, Bank1 |
| 4 | FPGA_B2B4 | FPGA Bank2, Bank4 |
| 5 | FPGA_B5B6 | FPGA Bank5, Bank6 |
| 6 | FPGA_B7B8 | FPGA Bank7, Bank8 |
| 7 | FPGA_B9JTAG | FPGA Bank9 JTAG Reset |
| 8 | FPGA_SERDES | FPGA SERDES |
| 9 | FPGA_PWR | FPGA PWR for Core, SERDES, PLL |
| 10 | FPGA_PWR2 | FPGA PWR for Bank IO |
| 11 | FPGA_GNDNC | FPGA GND and NC |
| 12 | MEMORY | SRAM, MRAM, Flash |
| 13 | IF_INT | LVDS, 422 |
| 14 | ALGMUX_ADC | ADC MUX, OPAMP |

PCB artwork

- Design tool : Altium 16.1.7

- PCB size : 158mm X 88mm X 2T

- 10 layers

Figure 4-12 Artwork

Polyimide

Table 4-12 PCB Stack Up Information

| STACK UP |  |  | 50옴 임피던스 |  | 100옴 임피던스 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 층 | 적층 | 예상두께 | 회로폭(㎛) | 보정후 | 회로폭(㎛) | 간격(㎛) | 보정회로폭 | 보정후 간격 |
| 1 | 1/2 OZ COPPER | 42 ㎛ | 100 | 130 | 90 | 260 | 120 | 230 |
|  | 0.06X1 | 60 ㎛ |  |  |  |  |  |  |
| 2 | 1 OZ COPPER | 35 ㎛ | REFERENCE |  | REFERENCE |  |  |  |
|  | 0.3T | 300 ㎛ |  |  |  |  |  |  |
| 3 | 1 OZ COPPER | 17 ㎛ | 114 | 144 | 92 | 220 | 122 | 190 |
|  | 0.11X1 | 110 ㎛ |  |  |  |  |  |  |
| 4 | 1 OZ COPPER | 35 ㎛ | REFERENCE |  | REFERENCE |  |  |  |
|  | 0.3T | 300 ㎛ |  |  |  |  |  |  |
| 5 | 1 OZ COPPER | 35 ㎛ | 114 | 144 | 92 | 220 | 122 | 190 |
|  | 0.11X1 | 110 ㎛ |  |  |  |  |  |  |
| 6 | 1 OZ COPPER | 35 ㎛ | REFERENCE |  | REFERENCE |  |  |  |
|  | 0.3T | 300 ㎛ |  |  |  |  |  |  |
| 7 | 1 OZ COPPER | 35 ㎛ | REFERENCE |  | REFERENCE |  |  |  |
|  | 0.11X1 | 110 ㎛ |  |  |  |  |  |  |
| 8 | 1 OZ COPPER | 35 ㎛ | 114 | 144 | 92 | 220 | 122 | 190 |
|  | 0.3T | 300 ㎛ |  |  |  |  |  |  |
| 9 | 1 OZ COPPER | 35 ㎛ | REFERENCE |  | REFERENCE |  |  |  |
|  | 0.06X1 | 60 ㎛ |  |  |  |  |  |  |
| 10 | 1/2 OZ COPPER | 42 ㎛ | 100 | 130 | 90 | 260 | 120 | 230 |
| TOTAL |  | 1,996 ㎛ |  |  |  |  |  |  |