Korea Lunar Exploration Program

DTNPL Qualification Model

Thermal Cycling Test Procedure

| Date: 19 Feb. 2019 | Doc. No: KPLO-D0-550-450 Issue: 1.0 Total page: 43 |
| --- | --- |

Superseding     Revised

| Reviewed By: Chang-Soo Lee Date DTN Product Assurance, Lumir Kyung-Rak Lee Date DTN System Engineer, ETRI |  | Prepared By: Sun-Ku Kim Date DTN Engineer, Lumir Dae-Soo Oh Date DTN Project Manager, Lumir |
| --- | --- | --- |
| Reviewed By: Chol-Oh Jeong Date DTN Product Assurance, ETRI |  | Jin-Ho Jo Date DTN Project Leader, ETRI |

Original CDMO Release__________________

| Revision/Change Records |  |  |  |
| --- | --- | --- | --- |
| Version | Date | Revision/Change Description | Pages |
| P-00 | 2017. 11. 06. | Draft | All |
| P-01 | 2017. 11. 24. | Test Spec Updated | All |
| 1.0 | 2019. 02. 19 | QM Test Spec Updated | All |

Documents Tree

Contents

1.	Introduction	8

1.1.	Purpose and Scope	8

1.2.	Applicable Documents	8

1.2.1.	ETRI Documents	8

1.2.2.	Reference Documents	8

1.3.	Abbreviations	8

2.	Test Article	10

2.1.	DTNPL Configurations	10

2.1.1.	CON_PWR	10

2.1.2.	CON_COMM	10

2.1.3.	CON_DEBUG	11

2.2.	Verification Cross Reference	12

2.3.	Notification for Qualification and Acceptance Tests	12

3.	Conditions	12

3.1.	Test personnel	12

3.2.	Precaution and Warnings	13

4.	Test Setup	14

4.1.	Test facility	14

4.2.	Thermal Chamber Setup	14

4.3.	Test Software	15

4.4.	Test Preparation	15

4.4.1.	Control and Measuring Equipment	15

5.	Test Description	16

5.1.	Thermal Test Levels	16

5.2.	Temperature Profile	16

5.3.	Test Parameters	17

6.	Test Procedures	17

6.1.	Test preparation	23

6.2.	Ambient Test Before Thermal Cycling Test	24

6.3.	Highest Non-Operational Temperature	25

6.4.	First Hot Plateau	26

6.5.	Lowest Non-Operational Temperature	27

6.6.	First Cold Plateau	28

6.7.	Intermediate Cycle	29

6.7.1.	2nd Cycle	29

6.7.2.	3rd Cycle	31

6.7.3.	4th Cycle	33

6.8.	Last Hot Plateau	35

6.9.	Last Cold Plateau	36

6.10.	Back to Ambient Temperature	37

6.11.	Functional Test After Thermal Cycling	38

6.12.	Remove DTNPL From Chamber	39

7.	Test Conclusion	39

Figures

Figure 2-1 DTNPL Configurations	10

Figure 2-2 Power Connector	10

Figure 2-3 Commend Connector	11

Figure 2-4 Debug Connector	12

Figure 4-1 Test Room Layout	14

Figure 4-2 Thermal Chamber Layout	14

Figure 4-3 Location of Thermal Sensors	15

Figure 5-1 Temperature Design and Test Limits	16

Figure 5-2 Timeline for Thermal Cycle Profile	17

Tables

Table 4-1 Control Thermal Sensors	15

Table 4-2 Measurement Thermal Sensors	15

Table 5-1 DTNPL Test Temperature Limits	16

Table 6-1 Thermal Cycle Procedure and Setting Prameters	18

Introduction

Purpose and Scope

This document defines the Thermal Cycling test procedure for the DTN payload.

The purpose of this is to demonstrate that the DTN payload is working as expected under the required environmental conditions. During the Thermal Test a representative collection of functionality and performance test will be performed to check interfaces and hardware related functions.

Applicable Documents

ETRI Documents

|  | Document Number | Document Title |
| --- | --- | --- |
| AD01 | KPLO-D0-550-000 | System Requirement of DTN Payload |
| AD02 | KPLO-D0-550-001 | System Specification of DTN Payload |
| AD03 | KPLO-SP-320-002 | KPLO Environmental Design and Test Specification |

Reference Documents

|  | Document Number | Document Title |
| --- | --- | --- |
| RD01 | MIL-STD-1540C | Test Requirements for Launch, Upper-Stage, and Space Vehicles |
| RD02 | PSS-01-801 | Test Requirements Specification for Space Equipment |
| RD03 | ECSS-E-10-03-A | ECSS Space Engineering-Testing |

Abbreviations

| AMS | Asynchronous Message Service |
| --- | --- |
| BP | Bundle Protocol |
| BSS | Bundle Streaming Service |
| CCSDS | Consultative Committee for Space Data Systems |
| CFDP | CCSDS File Delivery Protocol |
| DCC | DTN Control Center |
| CRC | Cyclical Redundancy Check |
| DMA | Direct Memory Access |
| DTN | Delay(Disruption) Tolerant Network |
| DTNPL | DTN Payload |
| EOD | End of Data |
| EOF | End of File |
| FW | Firm Ware |
| FSW | Flight Software |
| ION | Interplanetary Overlay Network |
| KPLO | Korea Pathfinder Lunar Orbiter |
| LCM | Lander Communication Model |
| LTP | Lickliter Transmission Protocol |
| OBC | On Board Computer |
| PDHU | Payload Data Handling Unit |
| RCM | Rover Communication Model |
| DTNPL | Single Board Computer |
| SOD | Start of Data |
| SOF | Start of File |
| SOH | Start of Head |
| TBC | To Be Confirmed |
| TBD | To Be Defined |
| TC | Tele-command |
| TCP | Transmission Control Protocol |
| TM | Telemetry |
| UART | Universal Asynchronous Receiver Transmitter |

Test Article

The objective of this Thermal Cycling Test is to demonstrate compliance of the DTNPL with the requirements from the KPLO Requirements Specification [AD 03]

This test procedure shall be used for test of the DTNPL QM.

DTNPL Configurations

The unit under test is shown in the following figures:

Figure 2-1 DTNPL Configurations

Dimension: 120 x 170 x 39 ㎜

Weight: 1.23 kg

CON_PWR

- Power Input (2 lines for +28V, 2 lines for RTN)

Figure 2-2 Power Connector

CON_COMM

- 1 UART for OBC

- 2 USRT for PDHU(P/R)

- 1 Bi-level Status for OBC

- 1 Bi-level Command for OBC

Figure 2-3 Commend Connector

CON_DEBUG

- 1 JTAG for Firmware Uploading

- 1 RS422 for Debugging UART

- 1 SpaceWire for Image Uploading, 40 Mbps (TBR)

- 1 GPIO for Debugging

- 1 SPI for DT10 (S/W Test Platform)

Figure 2-4 Debug Connector

Verification Cross Reference

The requirements which have to be verified by a successful performance in the test and its relevant test steps are given below:

| Applicable Doc. | Requirement | Verified by Test Step | Compliance |
| --- | --- | --- | --- |
| KPLO-SP-320-002 | Environmental Design | All | C |

Notification for Qualification and Acceptance Tests

This test procedure shall be approved by the ETRI prior to performance of the test, either by signature on cover page of by performance of a dedicated Test Readiness Review(TRR).

Conditions

Test personnel

The personnel required for the performance of the tests is listed below:

Test Engineers

The test engineers are responsible for:

- Operation of measuring instrumentation, data acquisition, data handling, data recording

- Identification and compilation of data sheet

- Safety precaution for test articles

- Test personnel to be present in the test area

- The accuracy of the measured values & Data report

Test Manager

The test manager is responsible for:

- Test articles and technical aspects of the test performance

- Record the correct software version for test article and EGSE

- Examination of disturbances

- Test article integration and handling

- Test performance

- Evaluation of test results & Providing of test reports

Quality Assurance Engineer

The QA engineer is responsible for:

- Surveillance of test equipment according to regulations as well as the test procedure

- Statement that the test articles to be tested have passed all checks before testing

- Checking the identification markings on the test articles

- Supervision of the proceeding with respect to quality assurance performance

The customer has the fight to participate or observe the test

| Responsibility | Company /Dep. | Name |
| --- | --- | --- |
| Test Engineer | Lumir / R&D | Kim, Sun-Ku |
| Test Manager | Lumir / R&D | Oh, Dae-Soo |
| Quality Assurance Engineer | Lumir / PA | Lee, Chang-Soo |
| Facility Engineer / Operator | Lumir / QA | Choi, Jae-woong |

Precaution and Warnings

For all testing and handling, ESD requirements concerning personnel, facilities and test equipment shall be regarded.

No special hazard exists during the test.

The following precautions apply:

- Only trained personnel are allowed to handle the test specimen.

- A test cable or breakout box shall never be connected of disconnected when the test specimen is powered.

- Correct set-up of the test equipment has to be checked carefully prior to the connection/switch-on of flight hardware, e.g. bonding, grounding, voltages, current limiters, etc.

- The test specimen shell be protected by a clean, antistatic bag all the time no activities are performed at the hardware.

Test Setup

Test facility

The ambient conditions within the test rooms shall be in compliance with the clean room class 100,000 or less.

Figure 4-1 Test Room Layout

Thermal Chamber Setup

For Thermal Cycling Test the following setup will be used:

Figure 4-2 Thermal Chamber Layout

Test Software

Software version used:

Test Preparation

The test harness shall be connected from DTNPL ESGE to the DTNPL through the Thermal Chamber feed-thru. During the test, the DTNPL shall be operated in a representative manner.

Control and Measuring Equipment

The thermal plate and the DTNPL unit will be equipped with temperature sensors(Thermocouples) on the positions defined in the descriptions below. The actual sensor positions will be noted after fixing on the equipment as given in the sensor location plan at the following tables:

Table 4-1 Control Thermal Sensors

| No. | Position | Remark |
| --- | --- | --- |
| C1 | Base Plate | Defined by Test Engineer |
| C2 | Base Plate (optional) | Defined by Test Engineer |

Table 4-2 Measurement Thermal Sensors

| No | Position | Remark |
| --- | --- | --- |
| T1 | DTNPL Top Cover |  |
| T2 | DTNPL Case Side |  |

Figure 4-3 Location of Thermal Sensors

Test Description

Thermal Test Levels

The nominal sequence of the thermal cycles is depicted in and temperature range for tests are summarized in

Table 5-1 DTNPL Test Temperature Limits

|  | Temperature Limits (degree C) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Equipment | Proto-flight (Operating) Range |  | Cold Turn-On | Acceptance (Operating) Range |  | Survival (Non-Operating) Range |  |
|  | Min | Max |  | Min | Max | Min | Max |
| DTNE_DTNPL | -30 | 60 | -30 | -20 | 50 | -35 | 70 |

Figure 5-1 Temperature Design and Test Limits

Temperature Profile

The DTNPL shall be subjected to temperature cycles as described in the following figure. DTNPL operation over the range of temperatures shall be demonstrated at specific points during the thermal cycling test according to the following figure.

Figure 5-2 Timeline for Thermal Cycle Profile

Test Parameters

The following test parameters shall apply:

Temperature range : As defined in the applicable equipment specification and Table 5-1

Temperature exposure : As described in Table 5-1 and Figure 5-2

Temperature change rate : 3 to 5°C/minute

Test Procedures

The DTNPL shall have a full functional test performed at ambient temperature prior to beginning the thermal cycling test. Thermal soak duration shall be at least 6 hours at the hot temperatures and 6 hours at

the cold temperatures during the first and last cycle (Figure 4-1). Intermediate cycles shall give at least 1 hour soak at the hot and cold temperatures. During the thermal soak, the unit shall be turned off until the temperature stabilizes and turn on. Remaining on until the next soak period off on sequence. Measurement of thermal soak duration shall begin at the time of unit turn on (Figure 4-1).

Functional tests shall be conducted after the DTNPL temperature have stabilized at hot and cold temperatures during the first and last thermal cycle, and after return to ambient. During the remainder of the test, electrical and electronic units, including all redundant circuits and paths, shall be cycled through various operation modes.

DTNPL shall meet their performance requirements within specification over the maximum expected temperature range extended at both temperatures by 10°C. The final thermal cycle should employ a sufficiently slow temperature transition to permit a complete functional check to be repeated at essentially all temperatures.

Moisture condensation in side of DTNPL shall be prevented.

Table 6-1 Thermal Cycle Procedure and Setting Prameters

|  | Setting Tempe (℃) | Setting Time (H.M.S) | Power Control | Remark |
| --- | --- | --- | --- | --- |
| Start | Ambient |  | Turn on for function test | Ambient |
| 1 | 70 | 00.15.00 | Turn off @ After 10minutes | First Cycle |
| 2 | 70 | 01.00.00 |  |  |
| 3 | 60 | 00.06.00 |  |  |
| 4 | 60 | 06.30.00 | Turn on @ After 30minutes |  |
| 5 | -35 | 00.35.00 | Turn off @ After 20minutes |  |
| 6 | -35 | 01.00.00 |  |  |
| 7 | -30 | 00.05.00 | Turn on @ After 3minutes |  |
| 8 | -30 | 06.00.00 |  |  |
| 9 | 60 | 00.23.00 | Turn off @ After 23minutes | 2nd Intermediate Cycle |
| 10 | 60 | 01.30.00 | Turn on @ After 10minutes |  |
| 11 | -30 | 00.23.00 | Turn off @ After 23minutes |  |
| 12 | -30 | 01.30.00 | Turn on @ After 10minutes |  |
| 13 | 60 | 00.23.00 | Turn off @ After 23minutes | 3rd Intermediate Cycle |
| 14 | 60 | 01.30.00 | Turn on @ After 10minutes |  |
| 15 | -30 | 00.23.00 | Turn off @ After 23minutes |  |
| 16 | -30 | 01.30.00 | Turn on @ After 10minutes |  |
| 17 | 60 | 00.23.00 | Turn off @ After 23minutes | 4th Intermediate Cycle |
| 18 | 60 | 01.30.00 | Turn on @ After 10minutes |  |
| 19 | -30 | 00.23.00 | Turn off @ After 23minutes |  |
| 20 | -30 | 01.30.00 | Turn on @ After 10minutes |  |
| 21 | 60 | 00.23.00 | Turn off @ After 23minutes | Last Cycle |
| 22 | 60 | 06.30.00 | Turn on @ After 30minutes |  |
| 23 | -30 | 00.23.00 | Turn off @ After 23minutes |  |
| 24 | -30 | 06.30.00 | Turn on @ After 30minutes |  |
| 25 | 35 | 00.18.00 |  | Ambient |
| 26 | 35 | 00.30.00 |  |  |
| 27 | 20 | 00.05.00 |  |  |
| End | Ambient |  | Turn off After function test |  |

Table 6-2 Test Procedure Sequence

| Step | Test Procedure | Remark |
| --- | --- | --- |
| 1 | Configure the test environment as shown in the ‘Figure 4-2’. |  |
| 2 | Power Input - Power Supply Setting : 28V / 0.50A - Nominal Voltage : 28V - Nominal Current : <0.50A |  |
| 2 | Scan Chain Pass | O |
| 4 | GRMON Connection PASS |  |
| 5 | F/W execution PASS | O |
| 6 | MATLAB DTN_EGSE.m execution PASS | O |
| 7 | Set up environment to receive data by running ‘Data Center’ Software. |  |
| 8 | After COSMOS Software execution,, In the 층 packet window, select 1_MULTI_Test and click the send button. | O |
| 9 | Storing data when data comes in ‘Data center’ Software. | O |

Test preparation

Start Date : ________________________   /  Time : ________________________

| (1) | Fill out test environment conditions | Pass / Fail | Signe |
| --- | --- | --- | --- |
| (2) | Fill out test personnel | Pass / Fail | Signe |
| (3) | Check test articles and fill out, check, and sign Test Article list above | Pass / Fail | Signe |
| (4) | Check test equipment and fill out, check, Test Equipment list above setup complete | Pass / Fail | Signe |
| (5) | Power off for each test article and test equipment switched off Install DTNPL in Thermal Cycling chamber | Pass / Fail | Signe |
| (6) | Mount DTNPL onto the base plate by fixing of screws. | Pass / Fail | Signe |
| (7) | Attach temperature sensors according to chapter 4.4.1 | Pass / Fail | Signe |
| (8) | Check that connector savers installed on all used interfaces | Pass / Fail | Signe |
| (9) | Connect harness to DTNPL and connect harness to chamber feed- through | Pass / Fail | Signe |
| (10) | Take pictures of the mounted DTNPL when ready for thermal cycling test | Pass / Fail | Signe |
| (11) | Start recording temperature measurement points and chamber temperature continuously | Pass / Fail | Signe |
| (12) | Correlate time, Temperature and test results exactly | Pass / Fail | Signe |

End Date:                        /  Time:

Test Engineer:                     (Signature)

Ambient Test Before Thermal Cycling Test

Start Date:                        /  Time:

Temperature:                   ℃ (19 ~ 25℃)

Humidity:                    % (35 ~ 55℃)

With feed assembly in the thermal chamber, the functional test should be performed at the ambient condition in accordance with the following sequence and record the test results per the indicated method.

|  | Test Description | Nominal Value | Actual Value |
| --- | --- | --- | --- |
| (1) | Measure input current | Under 0.5A |  |
| (2) | Measure input voltage | 28V |  |
| (3) | Perform Functional test in ‘Table 6-2’ | Pass / Fail |  |

End Date:                        /  Time:

Test Engineer:                     (Signature)

Highest Non-Operational Temperature

Start Date:                        /  Time:

- Increase the temperature in the T/C chamber to the maximum test temperature(+70℃)

- Turn off DTNPL at + 60℃ during thermal transition

Turn off time:

- Wait until the temperature of Thermal Chamber reaches +70℃

|  | Temperature |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | C1 | C2 | T1 | T2 | T3 |
| Start |  |  |  |  |  |
| End |  |  |  |  |  |

End Date:                        /  Time:

Test Engineer:                     (Signature)

First Hot Plateau

Start Date:                        /  Time:

- Start thermal transition setting to Temperature(+60℃)

|  | Temperature |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | C1 | C2 | T1 | T2 | T3 |
| Start |  |  |  |  |  |
| End |  |  |  |  |  |

- Wait until the temperature of the Thermal Chamber reaches +60℃ and switch DTNPL on.

Turn on time:

| Test Description | Nominal Value | Actual Value |
| --- | --- | --- |
| Measure input current | Under 0.5A |  |
| Measure input voltage | 28V |  |
| Perform full Functional Test of DTNPL | Pass / Fail |  |

- Hold temperature for at least 6 hours.

End Date:                        /  Time:

Test Engineer:                     (Signature)

Lowest Non-Operational Temperature

Start Date:                        /  Time:

- Start thermal transition by decreasing temp setting down to -35℃.

- Turn off DTNPL at -30℃ during thermal transition

Turn off time:

- Wait until the temperature of Thermal Chamber reaches -35℃

|  | Temperature |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | C1 | C2 | T1 | T2 | T3 |
| Start |  |  |  |  |  |
| End |  |  |  |  |  |

- Hold temperature for at least 1 hour

End Date:                        /  Time:

Test Engineer:                     (Signature)

First Cold Plateau

Start Date:                        /  Time:

- Start thermal transition by increasing temp setting up to -30℃.

- Turn on DTNPL at -30℃ during thermal transition

Turn on time:

- Wait until the temperature of the Thermal Chamber reaches -30℃

|  | Temperature |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | C1 | C2 | T1 | T2 | T3 |
| Start |  |  |  |  |  |
| End |  |  |  |  |  |

- Perform short Functional Test of DTNPL

| Test Description | Nominal Value | Actual Value |
| --- | --- | --- |
| Measure input current | Under 0.5A |  |
| Measure input voltage | 28V |  |
| Perform full Functional Test of DTNPL | Pass / Fail |  |

- Hold temperature for at least 5 hours.

End Date:                        /  Time:

Test Engineer:                     (Signature)

Intermediate Cycle

2nd Cycle

Start Date:                        /  Time:

- Start thermal transition by increasing temp setting up to +60℃.

|  | Temperature |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | C1 | C2 | T1 | T2 | T3 |
| +55℃. |  |  |  |  |  |
| -25℃. |  |  |  |  |  |

- When base plate temperature reaches +60℃ switch DTNPL off

Turn off time:

- Hold temperature for at least 10 minutes and Turn on DTNPL

Turn on time:

- Perform short Functional Test of DTNPL

| Test Description | Nominal Value | Actual Value |
| --- | --- | --- |
| Measure input current | Under 0.5A |  |
| Measure input voltage | 28V |  |
| Perform full Functional Test of DTNPL | Pass / Fail |  |

- Hold temperature for at least 1 hour.

- Start thermal transition by decreasing temp setting down to -30℃.

- When Thermal Chamber reaches -30℃ switch DTNPL off

Turn off time:

- Hold temperature for 10 minutes and switch DTNPL on

Turn on time:

- Perform short Functional Test of DTNPL

| Test Description | Nominal Value | Actual Value |
| --- | --- | --- |
| Measure input current | Under 0.5A |  |
| Measure input voltage | 28V |  |
| Perform full Functional Test of DTNPL | Pass / Fail |  |

- Hold temperature for at least 1 hour.

End Date:                        /  Time:

Test Engineer:                     (Signature)

3rd Cycle

Start Date:                        /  Time:

- Start thermal transition by increasing temp setting up to +60℃.

|  | Temperature |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | C1 | C2 | T1 | T2 | T3 |
| +55℃. |  |  |  |  |  |
| -25℃. |  |  |  |  |  |

- When base plate temperature reaches +60℃ switch DTNPL off

Turn off time:

- Hold temperature for at least 10 minutes and Turn on DTNPL

Turn on time:

- Perform short Functional Test of DTNPL

| Test Description | Nominal Value | Actual Value |
| --- | --- | --- |
| Measure input current | Under 0.5A |  |
| Measure input voltage | 28V |  |
| Perform full Functional Test of DTNPL | Pass / Fail |  |

- Hold temperature for at least 1 hour.

- Start thermal transition by decreasing temp setting down to -30℃.

- When Thermal Chamber reaches -30℃ switch DTNPL off

Turn off time:

- Hold temperature for 10 minutes and switch DTNPL on

Turn on time:

- Perform short Functional Test of DTNPL

| Test Description | Nominal Value | Actual Value |
| --- | --- | --- |
| Measure input current | Under 0.5A |  |
| Measure input voltage | 28V |  |
| Perform full Functional Test of DTNPL | Pass / Fail |  |

- Hold temperature for at least 1 hour.

End Date:                        /  Time:

Test Engineer:                     (Signature)

4th Cycle

Start Date:                        /  Time:

- Start thermal transition by increasing temp setting up to +60℃.

|  | Temperature |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | C1 | C2 | T1 | T2 | T3 |
| +55℃. |  |  |  |  |  |
| -25℃. |  |  |  |  |  |

- When base plate temperature reaches +60℃ switch DTNPL off

Turn off time:

- Hold temperature for at least 10 minutes and Turn on DTNPL

Turn on time:

- Perform short Functional Test of DTNPL

| Test Description | Nominal Value | Actual Value |
| --- | --- | --- |
| Measure input current | Under 0.5A |  |
| Measure input voltage | 28V |  |
| Perform full Functional Test of DTNPL | Pass / Fail |  |

- Hold temperature for at least 1 hour.

- Start thermal transition by decreasing temp setting down to -30℃.

- When Thermal Chamber reaches -30℃ switch DTNPL off

Turn off time:

- Hold temperature for 10 minutes and switch DTNPL on

Turn on time:

- Perform short Functional Test of DTNPL

| Test Description | Nominal Value | Actual Value |
| --- | --- | --- |
| Measure input current | Under 0.5A |  |
| Measure input voltage | 28V |  |
| Perform full Functional Test of DTNPL | Pass / Fail |  |

- Hold temperature for at least 1 hour.

End Date:                        /  Time:

Test Engineer:                     (Signature)

Last Hot Plateau

Start Date:                        /  Time:

- Start thermal transition setting to Temperature(+60℃)

|  | Temperature |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | C1 | C2 | T1 | T2 | T3 |
| Start |  |  |  |  |  |
| End |  |  |  |  |  |

- Wait until the temperature of the Thermal Chamber reaches +60℃ and switch DTNPL on.

Turn on time:

| Test Description | Nominal Value | Actual Value |
| --- | --- | --- |
| Measure input current | Under 0.5A |  |
| Measure input voltage | 28V |  |
| Perform full Functional Test of DTNPL | Pass / Fail |  |

- Hold temperature for at least 6 hours.

End Date:                        /  Time:

Test Engineer:                     (Signature)

Last Cold Plateau

Start Date:                        /  Time:

- Start thermal transition by increasing temp setting up to -30℃.

- Turn on DTNPL at -30℃ during thermal transition

Turn on time:

- Wait until the temperature of the Thermal Chamber reaches -30℃

|  | Temperature |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | C1 | C2 | T1 | T2 | T3 |
| Start |  |  |  |  |  |
| End |  |  |  |  |  |

- Perform short Functional Test of DTNPL

| Test Description | Nominal Value | Actual Value |
| --- | --- | --- |
| Measure input current | Under 0.5A |  |
| Measure input voltage | 28V |  |
| Perform full Functional Test of DTNPL | Pass / Fail |  |

- Hold temperature for at least 5 hours.

End Date:                        /  Time:

Test Engineer:                     (Signature)

Back to Ambient Temperature

Start Date:                        /  Time:

- Start thermal transition by increasing temp setting up to +35℃.

- Check the temperature as it reaches +35℃.

- Hold temperature for at least 30 minutes.

- Start thermal transition by decreasing temp setting down to +25℃.

- Check the temperature as it reaches +25℃.

|  | Temperature |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | C1 | C2 | T1 | T2 | T3 |
| +35℃. |  |  |  |  |  |
| +25℃. |  |  |  |  |  |

End Date:                        /  Time:

Test Engineer:                     (Signature)

Functional Test After Thermal Cycling

Start Date:                        /  Time:

- Perform Full Functional Test of DTNPL

| Test Description | Nominal Value | Actual Value |
| --- | --- | --- |
| Measure input current | Under 0.5A |  |
| Measure input voltage | 28V |  |
| Perform full Functional Test of DTNPL | Pass / Fail |  |

- Switch DTNPL off.

Turn off time:

- Stop recording chamber data

End Date:                        /  Time:

Test Engineer:                     (Signature)

Remove DTNPL From Chamber

Start Date:                        /  Time:

- Disconnect Test Harness from DTNPL EGSE and chamber feed-through.

- Disconnect TC Harness from DTNPL EGSE and chamber feed-through.

- Remove temperature sensors.

- Dismount DTNPL Base plate fixations.

- Remove DTNPL from Chamber.

End Date:                        /  Time:

Test Engineer:                     (Signature)

Test Conclusion

The PA shall review the test data for acceptance of the thermal Cycling test.

The following sign off is verification that the test article has satisfactorily passed thermal Cycling test.

PA:                            Date: