Korea Lunar Exploration Program

DTNPL Qualification Model

Vibration Test Procedure

| Date: 11 Mar. 2019 | Doc. No: KPLO-D0-550-470 Issue: D-01 Total page: 29 |
| --- | --- |

Superseding     Revised

| Reviewed By: Chang-Soo Lee Date DTN Product Assurance, Lumir Kyung-Rak Lee Date DTN System Engineer, ETRI |  | Prepared By: Jeong-Chan Lee Date DTN Engineer, Lumir Dae-Soo Oh Date DTN Project Manager, Lumir |
| --- | --- | --- |
| Reviewed By: Chol-Oh Jeong Date DTN Product Assurance, ETRI |  | Jin-Ho Jo Date DTN Project Leader, ETRI |

Original CDMO Release__________________

| Revision History |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version | Author(s) |  | Date | Description |
| D-01 | Jin-Ho Jo |  | 2017. 11. 07 | Draft |
| 1.0 | Sun-Ku Kim |  | 2019. 03.11 | QM Test Spec Updated |

| Revision/Change Records |  |  |  |
| --- | --- | --- | --- |
| Version | Date | Revision/Change Description | Pages |
| D-01 | 2017. 11. 07 | Initial | All |
| 1.0 | 2019. 03. 01 | QM Test Spec Updated | All |

Documents Tree

Contents

1.	Introduction	9

1.1.	Purpose and Scope	9

1.2.	Applicable & Reference Documents	9

1.2.1.	Applicable Documents	9

1.2.1.	Reference Documents	9

1.3.	Abbreviations	9

2.	Test Article	11

2.1.	Verification Cross Reference	12

2.2.	Notification for Qualification and Acceptance Tests	12

2.3.	Precaution and Warnings	12

3.	. Test Specifications	13

3.1.	Sine vibration	13

3.2.	Random vibration	13

3.3.	DTNPL Function test item	13

3.4.	Test condition tolerances	14

3.5.	Pass/fail criteria	14

4.	Test Equipment	14

5.	Vibration test configuration	15

5.1.	Sensor location	15

5.2.	Vibration test setup	15

5.2.1.	Vibration Test setup for x Axis	15

5.2.2.	Vibration Test setup for Y Axis	16

5.2.3.	Vibration Test setup for Z Axis	17

6.	Test Environments	18

6.1.	equipment to be tested	18

6.2.	test location	18

6.3.	Test personnel	18

6.4.	Vibration test equipment	19

7.	Test Description	19

7.1.	Vibration test	19

8.	Test Procedure	20

8.1.	X-axis test set up	20

8.1.1.	Sine vibration test for x axis	21

8.1.2.	Random vibration test for x axis	22

8.2.	Y-axis test	23

8.2.1.	Sine vibration test for Y axis	24

8.2.2.	Random vibration test for Y axis	25

8.3.	Y axis test	26

8.3.1.	vibration test for z axis	27

8.3.2.	Random vibration test for z axis	28

9.	Test conclusions	29

Figures

Figure 2-2-1 DTNPL Configurations	11

Figure 5-1 Location for Sensors for Test	15

Figure 7-1 Vibration Test Flow for DTNPL	19

Tables

Table 6-1 Vibration Test Equipment List	19

Introduction

Purpose and Scope

This document establishes the conditions, requirements and test procedures for vibration testing of the DTNPL payload.

Applicable & Reference Documents

Applicable Documents

|  | Document Number | Document Title |
| --- | --- | --- |
| AD-01 | C1-SP-320-003 | Components Environmental Design and Test Specification |
| AD-02 | KPLO-SP-320-002 | KPLO Environmental Design and Test Specification |

    - Reference Documents

|  | Document Number | Document Title |
| --- | --- | --- |
| RD-01 | MIL-STD-1540C | Test Requirements for Space Launch, Upper-Stage, and Space Vehicle |
| RD-02 | ECSS-E-10-0 | Space Engineering, Testing |

  - Abbreviations

| AMS | Asynchronous Message Service |
| --- | --- |
| BP | Bundle Protocol |
| BSS | Bundle Streaming Service |
| CCSDS | Consultative Committee for Space Data Systems |
| CFDP | CCSDS File Delivery Protocol |
| DCC | DTN Control Center |
| CRC | Cyclical Redundancy Check |
| DMA | Direct Memory Access |
| DTN | Delay(Disruption) Tolerant Network |
| DTNPL | DTN Payload |
| EOD | End of Data |
| EOF | End of File |
| FW | Firm Ware |
| FSW | Flight Software |
| ION | Interplanetary Overlay Network |
| CAS500-1 | Korea Pathfinder Lunar Orbiter |
| LCM | Lander Communication Model |
| LTP | Licklider Transmission Protocol |
| OBC | On Board Computer |
| PDHU | Payload Data Handling Unit |
| RCM | Rover Communication Model |
| DTNPL | Single Board Computer |
| SOD | Start of Data |
| SOF | Start of File |
| SOH | Start of Head |
| TBC | To Be Confirmed |
| TBD | To Be Defined |
| TC | Tele-command |
| TCP | Transmission Control Protocol |
| TM | Telemetry |
| UART | Universal Asynchronous Receiver Transmitter |

Test Article

DTNPL is composed by major functional modules as follows :

  - SBC (Single Board Computer)

Figure 2-1 shows the DTNPL QM external view and its coordinate system. Size and mass of test article is given in Table 2-1.

The coordinate system is defined as follows.

- The plane defined by the unit coordinate system axes X and Y is parallel to the mounting surface.

- The unit Z axis is normal to the mounting plane.

- The X axis with the dimension along X (length) being greater than the dimension along Y(width).

L

W

H

Figure 2-2-1 DTNPL Configurations

Table 2-1 Dimension Criteria

| Description | Value |
| --- | --- |
| Length measurement | 170 mm |
| Width measurement | 120 mm |
| Height measurement | 39 mm |
| Weighing | 0.98 kg |

Verification Cross Reference

The requirements which have to be verified by a successful performance in the test and its relevant test steps are given below:

| Applicable Doc. | Requirement | Verified by Test Step | Compliance |
| --- | --- | --- | --- |
| KPLO-SP-320-002 | Vibration | All | C |

Notification for Qualification and Acceptance Tests

This test procedure shall be approved by the KARI prior to performance of the test, either by signature on cover page of by performance of a dedicated Test Readiness Review(TRR).

Precaution and Warnings

For all testing and handling, ESD requirements concerning personnel, facilities and test equipment shall be regarded.

No special hazard exists during the test.

The following precautions apply:

- Only trained personnel are allowed to handle the test specimen.

- A test cable or breakout box shall never be connected of disconnected when the test specimen is powered.

- Correct set-up of the test equipment has to be checked carefully prior to the connection/switch-on of flight hardware, e.g. bonding, grounding, voltages, current limiters, etc.

- The test specimen shell be protected by a clean, antistatic bag all the time no activities are performed at the hardware.

. Test Specifications

Sine vibration

Table 3-1 is the qualification sinusoidal vibration specifications for the DTNPL in all three axes.

Table 3-1 Sine Vibration Level

| Frequency(Hz) | Acceleration(g’s) | ETC |
| --- | --- | --- |
| 5-20 | Max Shaker Limit | - Sweep Rate: 2 Oct./min |
| 20-100 | 20g |  |

Random vibration

Table 3-2 is the qualification random vibration specifications for the DTNPL in all three axes.

Table 3-2 Random Vibration Level (Lateral Axis, X-Y)

| Frequency(Hz) | Acceleration(g’s) | ETC |
| --- | --- | --- |
| 20 | 0.026 | - For testing: 2 minute per axis |
| 50 | 0.16 |  |
| 800 | 0.16 |  |
| 2000 | 0.026 |  |
| Overall | 14.1 grms |  |

Table 3-3Random Vibration Level (Longitudinal, X-Y)

| Frequency(Hz) | Acceleration(g’s) | ETC |
| --- | --- | --- |
| 20 | 0.00236 | - For testing: 2 minute per axis |
| 50 | 0.5 |  |
| 800 | 0.5 |  |
| 2000 | 0.0202 |  |
| Overall | 18.4 grms |  |

DTNPL Function test item

- Initialization

- Operation Test

Test condition tolerances

Table 3-3 Test Condition Tolerances

| Items | Tolerances |
| --- | --- |
| Sine Vibration Amplitude Frequency | ±10% ±2% |
| Random Vibration Accel. Spectral Density RMS Level | ±3 dB +10% |

Pass/fail criteria

The pass/fail for the Vibration test is verified by the detailed visual check, function test. The pass/fail criteria are shown in the Table 3-4.

| Item | Requirement |
| --- | --- |
| Function | No anomaly |

Test Equipment

The vibration excitation system and data acquisition system in QRT will be used for the vibration test.

The specifications of the shaker system and data measurement system are as shown in Table 4-1.

| Item | Manufacturer | Description |
| --- | --- | --- |
| Vibration Accelerometer | FAMTECH | 2 ton |
| Test Fixture | Lumir |  |
| Sensor | - |  |

Vibration test configuration

Sensor location

Location and information of the sensors are indicated in Figure 5-1. Normally two sensors will be attached beside mounting points and the maximum value is going to be used as criteria to know the responsive. The test sensor descriptions will be recorded on the Test Report.

Figure 5-1 Location for Sensors for Test

| No. | Position | Sensor S/N | Coordinate |  | Remark |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Sensor | Equip |  |
| 1 | C1 |  |  |  | Control |
| 2 | D1 |  |  |  | Measure |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |

Vibration test setup

Vibration Test setup for x Axis

Figure 5-2 Test Configuration for X Axis

Vibration Test setup for Y Axis

Figure 5-3 Test Configuration for Y Axis

Vibration Test setup for Z Axis

Figure 5-4 Test Configuration for Z Axis

Test Environments

The following test environment conditions shall be recorded prior to measurement.

equipment to be tested

Model Name:

test location

Test personnel

The personnel required for the performance of the tests is listed below:

Test Engineers

The test engineers are responsible for:

- Operation of measuring instrumentation, data acquisition, data handling, data recording

- Identification and compilation of data sheet

- Safety precaution for test articles

- Test personnel to be present in the test area

- The accuracy of the measured values & Data report

Test Manager

The test manager is responsible for:

- Test articles and technical aspects of the test performance

- Record the correct software version for test article and EGSE

- Examination of disturbances

- Test article integration and handling

- Test performance

- Evaluation of test results & Providing of test reports

Quality Assurance Engineer

The QA engineer is responsible for:

- Surveillance of test equipment according to regulations as well as the test procedure

- Statement that the test articles to be tested have passed all checks before testing

- Checking the identification markings on the test articles

- Supervision of the proceeding with respect to quality assurance performance

The customer has the fight to participate or observe the test

| Responsibility | Company /Dep. | Name |
| --- | --- | --- |
| Test Engineer | Lumir / R&D |  |
| Test Manager | Lumir / R&D |  |
| Quality Assurance Engineer | Lumir / PA |  |
| Facility Engineer | SaTRec |  |

Vibration test equipment

Table 6-1 Vibration Test Equipment List

| Name | Model No. | Manufacturer |
| --- | --- | --- |

Test Description

Vibration test

The test shall be performed in accordance with the following sequences and the test results are recorded per the indicated method.

Figure 7-1 Vibration Test Flow for DTNPL

Test Procedure

The test will be performed in accordance with Figure 5-2 and 7-1.

X-axis test set up

| (a) | For the X axis evaluation, check and install the shaker. | (√) |
| --- | --- | --- |
| (b) | Assemble the test fixture and sensors on the shaker. | (√) |
| (c) | Perform the fixture evaluation per the test specification that is indicated in Table 3-1 & 3-2. | (√) |

| Start Date / Time |  |
| --- | --- |
| End Date / Time |  |
| Temperature ℃ (16 to 32℃) |  |
| Relative Humidity % (20 to 70%) |  |

| (d) | Generate plot for the verification compliance with the spectrum level and tolerance for each channel, and verify the results. | (√) |
| --- | --- | --- |

Sine vibration test for x axis

| (a) | Check equipment mounting and sensor status before the sine test for X axis. | (√) |
| --- | --- | --- |
| (b) | Input the test condition data in the controller of the shaker and verify. | (√) |
| (c) | Perform the fixture evaluation per the test specification that is indicated in Table 3-1. | (√) |

| Start Date / Time |  |
| --- | --- |
| End Date / Time |  |
| Temperature ℃ (16 to 32℃) |  |
| Relative Humidity % (20 to 70%) |  |

| (d) | Generate plot for the verification compliance with the spectrum level and tolerance for each channel, and verify the results. | (√) |
| --- | --- | --- |

| (e) | After the sine test is finished, perform the visual inspection of the equipment. | (√) |
| --- | --- | --- |

Random vibration test for x axis

| (a) | Check equipment mounting and sensor status before the random test for X axis. | (√) |
| --- | --- | --- |
| (b) | Input the test condition data in the controller of the shaker and verify. | (√) |
| (c) | Perform the fixture evaluation per the test specification that is indicated in Table 3-2. | (√) |

| Start Date / Time |  |
| --- | --- |
| End Date / Time |  |
| Temperature ℃ (16 to 32℃) |  |
| Relative Humidity % (20 to 70%) |  |

| (d) | Generate plot for the verification compliance with the spectrum level and tolerance for each channel, and verify the results. | (√) |
| --- | --- | --- |

| (e) | After the random test is finished, perform the visual inspection of the equipment. | (√) |
| --- | --- | --- |

Y-axis test

| (a) | For the Y axis evaluation, check and install the shaker. | (√) |
| --- | --- | --- |
| (b) | Assemble the test fixture and sensors on the shaker. | (√) |
| (c) | Perform the fixture evaluation per the test specification that is indicated in Table 3-1 & 3-2. | (√) |

| Start Date / Time |  |
| --- | --- |
| End Date / Time |  |
| Temperature ℃ (16 to 32℃) |  |
| Relative Humidity % (20 to 70%) |  |

| (d) | Generate plot for the verification compliance with the spectrum level and tolerance for each channel, and verify the results. | (√) |
| --- | --- | --- |

Sine vibration test for Y axis

| (a) | Check equipment mounting and sensor status before the sine test for X axis. | (√) |
| --- | --- | --- |
| (b) | Input the test condition data in the controller of the shaker and verify. | (√) |
| (c) | Perform the fixture evaluation per the test specification that is indicated in Table 3-1. | (√) |

| Start Date / Time |  |
| --- | --- |
| End Date / Time |  |
| Temperature ℃ (16 to 32℃) |  |
| Relative Humidity % (20 to 70%) |  |

| (d) | Generate plot for the verification compliance with the spectrum level and tolerance for each channel, and verify the results. | (√) |
| --- | --- | --- |

| (e) | After the sine test is finished, perform the visual inspection of the equipment. | (√) |
| --- | --- | --- |

Random vibration test for Y axis

| (a) | Check equipment mounting and sensor status before the random test for Y axis. | (√) |
| --- | --- | --- |
| (b) | Input the test condition data in the controller of the shaker and verify. | (√) |
| (c) | Perform the fixture evaluation per the test specification that is indicated in Table 3-2. | (√) |

| Start Date / Time |  |
| --- | --- |
| End Date / Time |  |
| Temperature ℃ (16 to 32℃) |  |
| Relative Humidity % (20 to 70%) |  |

| (d) | Generate plot for the verification compliance with the spectrum level and tolerance for each channel, and verify the results. | (√) |
| --- | --- | --- |

| (e) | After the random test is finished, perform the visual inspection of the equipment. | (√) |
| --- | --- | --- |

Y axis test

| (a) | For the Z axis evaluation, check and install the shaker. | (√) |
| --- | --- | --- |
| (b) | Assemble the test fixture and sensors on the shaker. | (√) |
| (c) | Perform the fixture evaluation per the test specification that is indicated in Table 3-1 & 3-2. | (√) |

| Start Date / Time |  |
| --- | --- |
| End Date / Time |  |
| Temperature ℃ (16 to 32℃) |  |
| Relative Humidity % (20 to 70%) |  |

| (d) | Generate plot for the verification compliance with the spectrum level and tolerance for each channel, and verify the results. | (√) |
| --- | --- | --- |

vibration test for z axis

| (a) | Check equipment mounting and sensor status before the sine test for Z axis. | (√) |
| --- | --- | --- |
| (b) | Input the test condition data in the controller of the shaker and verify. | (√) |
| (c) | Perform the fixture evaluation per the test specification that is indicated in Table 3-1. | (√) |

| Start Date / Time |  |
| --- | --- |
| End Date / Time |  |
| Temperature ℃ (16 to 32℃) |  |
| Relative Humidity % (20 to 70%) |  |

| (d) | Generate plot for the verification compliance with the spectrum level and tolerance for each channel, and verify the results. | (√) |
| --- | --- | --- |

| (e) | After the sine test is finished, perform the visual inspection of the equipment. | (√) |
| --- | --- | --- |

Random vibration test for z axis

| (a) | Check equipment mounting and sensor status before the random test for Z axis. | (√) |
| --- | --- | --- |
| (b) | Input the test condition data in the controller of the shaker and verify. | (√) |
| (c) | Perform the fixture evaluation per the test specification that is indicated in Table 3-2. | (√) |

| Start Date / Time |  |
| --- | --- |
| End Date / Time |  |
| Temperature ℃ (16 to 32℃) |  |
| Relative Humidity % (20 to 70%) |  |

| (d) | Generate plot for the verification compliance with the spectrum level and tolerance for each channel, and verify the results. | (√) |
| --- | --- | --- |

| (e) | After the random test is finished, perform the visual inspection of the equipment. | (√) |
| --- | --- | --- |

Test conclusions

The PA shall review the test data for acceptance of the Vibration test.

The following sign off is verification that the test article has satisfactorily passed Vibration test.

PA:                            Date: