DTNPL Flight Model Special Test Procedure (1/2)

Test Procedure : (KPLO-D0-550-460) KPLO-DTN Thermal Vacuum Test Procedures

Test Date :

Test No. :

Cycle No. :

Health Report

The DTNPL shall report its health status to KPLO SBMU for the reliable operation. The health status shall be included following information.

- Payload time, FW version

- Payload 1st voltage, Payload 1st current, Payload 2nd voltage

- Board temperature, FPGA temperature

- DTN status (enable/disabled)

- ION version

Health Report Demonstration Procedure

| No | Demonstration Procedure | Remark |
| --- | --- | --- |
| 1 | Send SOH request by using EGSE SW |  |
| 2 | Check the value of SOH and Record actual value below Pass/Fail Criteria |  |
| 3 | Save EGSE SW status screenshot and record file name at below Pass/Fail Criteria |  |

Health Report Pass/Fail Criteria

| Test Identifier | Demonstration Description | Expected Value | Actual Value | Pass/Fail |
| --- | --- | --- | --- | --- |
| STP #1 | Payload time | Current time |  |  |
|  | FW version | V0.5 |  |  |
|  | Board Temperature |  |  |  |
|  | FPGA Temperature |  |  |  |
|  | 28V | 28V |  |  |
|  | 3.3V | 3.3V |  |  |
|  | 1.2V | 1.2V |  |  |
|  | DTN status | Enable |  |  |
|  | ION version | 3.6.1 |  |  |

| Save File name |  |
| --- | --- |

DTNPL Flight Model Special Test Procedure (2/2)

DTN End Point Mode

In the DTN end point mode, DTNPL shall be operated as a source or destination node for the DTN communications. This mode necessary for the one-way trip test defined in the document ‘Operation Scenario of DTN’ (KPLO-D0-551-002)

DTN End Point Mode Procedure

| No | Demonstration Procedure | Remark |
| --- | --- | --- |
| 1 | Setup SBC EGSE PC & ION EGSE PC Network IP address |  |
| 2 | ION EGSE SW setup at standby mode |  |
| 3 | SBC EGSE SW Operation |  |
| 4 | Go on a mission command at ION EGSE SW |  |
| 5 | Check the same message is displayed in ION EGSE & SBC EGSE and record actual value below Pass/Fail Criteria |  |
| 6 | Perform 3 time mission |  |

DTN End Point Mode Pass/Fail Criteria

| Test Identifier | Demonstration Description | Expected Value | Actual Value | Pass/Fail |
| --- | --- | --- | --- | --- |
| STP #2 | Check the same message is displayed in ION EGSE & SBC EGSE | Operation |  |  |