Korea Lunar Exploration Program

DTNPL Software Technical Description

| Date: 28 Dec. 2017 | Doc. No: KPLO-D0-550-201 Issue: 1.0 Total page: 18 |
| --- | --- |

Superseding     Revised

| Prepared By: Hyun-Woong Choi Date DTN SBC S/W Engineer, Lumir Dae-Soo Oh Date DTN SBC Engineer, Lumir |  | Kyung-Rak Lee Date DTN Payload Engineer, ETRI |
| --- | --- | --- |
| Reviewed By: Jin-Ho Jo Date DTN System Engineer, ETRI |  | Sin-Ae Ji Date Proximity-1 System Engineer, ETRI |
| Sang-Man Moon Date RLE KPLO Electrical System, KARI |  | Hyun-Jin Shin Date RLE KPLO Mechanical System, KARI |
| Approval Signature: Byoung-Sun Lee Date DTN Project Leader, ETRI |  | Il-Young Kim Date RLE KPLO Product Assurance, KARI |
| Seong-Bin Lim Date Head, Lunar Exploration System Team, KARI |  | Seok-Weon Choi Date Director, Lunar Exploration Program Office, KARI |

Original CDMO Release__________________

| Revision History |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version | Author(s) |  | Date | Description |
| 0.1 | Hyun-Woong Choi |  | 2017. 12. 28 | Draft |

| Revision/Change Records |  |  |  |
| --- | --- | --- | --- |
| Version | Date | Revision/Change Description | Pages |
| 0.1 | 2017. 12. 28 | Initial | All |

Documents Tree

Contents

1.	Introduction	8

1.1.	Purpose and Scope	8

1.2.	Applicable Documents	8

1.2.1.	ETRI Documents	8

1.2.2.	Other Documents	8

1.3.	Abbreviations	8

2.	Overview of KPLO DTN	10

2.1.	DTN and Space Communications	10

2.2.	Mission of DTN Operation	10

2.3.	DTN Configurations	10

2.4.	Communication Nodes	11

2.4.1.	DTN nodes	11

2.4.2.	Non-DTN nodes	11

3.	Operations and Maintenance Manual	12

4.	Communication Protocols	13

4.1.	TM/TC Protocols	13

4.1.1.	UART Protocol	13

4.1.2.	Packet Format	13

4.1.3.	TM/TC Data Structure	14

4.2.	Datalink Protocol	14

4.2.1.	Datalink protocol for downlink	15

4.2.2.	Datalink Data Structure	16

4.3.	TM/TC Lists	16

4.3.1.	Command Lists	16

4.3.2.	Telemetry Lists	16

Figures

Figure 2.1 DTN test configuration in the first phase	11

Figure 3-1 Interface diagram of DTN payload	12

Figure 3-2 Electrical block diagram of DTN payload	12

Figure 3 Operational Concept	13

Figure 5-1 TM/TC packet structure	14

Figure 5-2 Downlink datalink packet structure	16

Tables

Table 5-1 DTNPL Protocols	13

Table 5-2 TM/TC Packet Format	14

Table 5-3 Data Field Format	14

Table 5-4 TM/TC data structure	15

Table 5-5 Datalink packet structure	16

Table 5-6 Contents of User Data Field	16

Table 5-7 First frame datalink structure	17

Table 5-8 Last frame datalink structure	17

Table 5-9 DTNPL Command Lists	17

Table 5-10 DTNPL Telemetry Lists	18

Introduction

Purpose and Scope

This document defines the electrical and mechanical interfaces between KPLO bus and DTN payload.

Applicable Documents

ETRI Documents

| Document Number | Document Title |
| --- | --- |
| KPLO-D0-550-000 | System Requirement of DTN Payload |
| KPLO-D0-550-001 | System Specification of DTN Payload |

Other Documents

| Document Number | Document Title |
| --- | --- |
| KPLO-SP-310-001 | KPLO System Specification Document |
| KPLO-SP-340-001 | KPLO Payload/Spacecraft Bus ICD |

Abbreviations

| AMS | Asynchronous Message Service |
| --- | --- |
| BP | Bundle Protocol |
| BSS | Bundle Streaming Service |
| CCSDS | Consultative Committee for Space Data Systems |
| CFDP | CCSDS File Delivery Protocol |
| DCC | DTN Control Center |
| CRC | Cyclical Redundancy Check |
| DMA | Direct Memory Access |
| DTN | Delay(Disruption) Tolerant Network |
| DTNPL | DTN Payload |
| EOD | End of Data |
| EOF | End of File |
| FW | Firm Ware |
| FSW | Flight Software |
| ION | Interplanetary Overlay Network |
| KDSA | Korea Deep Space Antenna |
| KPLO | Korea Pathfinder Lunar Orbiter |
| LCM | Lander Communication Model |
| LTP | Licklider Transmission Protocol |
| MOC | Mission Operation Center |
| OBC | On Board Computer |
| OBT | On Board Time |
| PDHU | Payload Data Handling Unit |
| RCM | Rover Communication Model |
| RFU | Reserved for Future Use |
| SBC | Single Board Computer |
| SOD | Start of Data |
| SOF | Start of File |
| SOH | Start of Head |
| TBC | To Be Confirmed |
| TBD | To Be Defined |
| TC | Tele-command |
| TCP | Transmission Control Protocol |
| TM | Telemetry |
| UART | Universal Asynchronous Receiver Transmitter |
| USRT | Universal Synchronous Receiver Transmitter |

Overview of KPLO DTN

DTN and Space Communications

Delay/disruption tolerant networking (DTN) technology offers a new solution to highly stressed communications in space environments, especially those with long link delay and frequent link disruptions in deep space missions. Delay/disruption tolerant networking (DTN) technology offers a new solution that use a store-and-forward mechanism to combat long link delay and frequent link disruptions generally characterizing space communications. DTN communications use a bundle protocol (BP), to construct a store-and-forward overlay network. The BP, situated at the application layer of the Internet model, utilizes conventional Internet protocols to send and receive data bundles. The benefits of DTN communications are as follows.

- Interoperability and Reuse: A standardized DTN protocol suite enables interoperability of multi-agency communication assets and also allows NASA to use the same communication stack for future missions (LEO, NEO or Deep Space).

- Space Link Efficiency, Utilization and Robustness: DTN enables more reliable and efficient data transmissions resulting in more usable bandwidth. DTN also improves link reliability by having multiple network paths and assets for potential communication hops.

- Security: The DTN Bundle Security Protocol (BSP) allows for integrity checks, authentication and encryption, even on links where not previously used.

- Quality of Service: The DTN protocol suite allows for many priority levels to be set for different data types, ensuring that the most important data is received ahead of less important data.

- Improved Operations and Situational Awareness: The DTN store-and-forward mechanism along with automatic retransmission provides more insight into events during communication outages and significantly reduces the need for ground-based scheduling.

Mission of DTN Operation

The mission of DTN is to validate DTN communication protocols by space link testing through KPLO. So DTN operations are focused on testing the DTN protocols using KPLO space links.

DTN Configurations

In the first phase of lunar exploration, test lunar orbiter called Korea Pathfinder Lunar Orbiter (KPLO) will be launched without lunar lander. Figure 2.1 shows DTN configuration in first phase of lunar exploration. There are no Lander and Rover on the moon in first phase. So Lander Communication Model (LCM) and Rover Communication Model (RCM) will be located on earth for the DTN communication testing.

DTN Payload (DTNPL) is a DTN node in the space. DTNPL will be mounted on the KPLO for DTN communication. DTN Control Center (DCC) is a control center of DTN communication nodes. LCM and RCM are DTN nodes on ground. All DTN nodes on ground are interconnected by the terrestrial network such as Internet. Mission Operation Center (MOC) and Korea Deep Space Antenna (KDSA) are not DTN nodes. They just transfer DTN packets to next communication nodes.

Figure 2.1 DTN test configuration in the first phase

Communication Nodes

Many communication nodes are needed for the DTN operation. Some of communication nodes in the network are DTN nodes whereas otherwise nodes are non-DTN nodes. The function of communication nodes used in the DTN operation is as follows.

DTN nodes

- DCC is a ground DTN node which monitor and controls DTN nodes. DCC send commands to DTN nodes such as DTNPL, LCM and RCM. It also receives telemetries or acquired data from some DTN nodes such as RCM or LCM.

- DTNPL is a DTN node mounted on the KPLO. DTNPL can be operated as a DTN end point. And it also can be operated as relay nodes which relay DTN data between DCC and RCM or LCM.

- LCM is a ground DTN node. LCM relays DTN data between MOC and RCM.

- RCM is a ground DTN node and a DTN end point. The commands sent by the DCC is terminated at the RCM and the RCM returns telemetry or data to DCC through KPLO.

Non-DTN nodes

- MOC is a non-DTN node. MOC monitors and controls KPLO and payloads. MOC also receives and processes data generated from the payloads and transfers to each payload control center.

- KDSA is a ground station for deep space communication. KDSA will have a big size of antenna and RF equipment for space link interface. KDSA also has network equipment for terrestrial communication interface such as SLE link. KDSA is a non-DTN node.

Operations and Maintenance Manual

The DTN payload (DTNPL) shall have interface with KPLO for the operational power supply and for the communications. The following figure shows DTN payload interface diagram with KPLO.

Figure 3-1 Interface diagram of DTN payload

There are four kinds of interface types between DTNPL and KPLO. Each interface types are described as following.

- TM/TC (bi-level): One bi-level TC and one bi-level TM for DTNPL emergency operation.

- Bi-level TC: System reset command in the emergence condition

- Bi-level TM: Telemetry showing that DTNPL is in alive

- TM/TC (UART): Serial communication channel for TM/TC communication with KPLO OBC

- Data downlink (USRT): Serial data downlink channel to KPLO PDHU

- Power: Operation DC power +28V from the KPLO PCDU

The detailed electrical configuration of DTNPL is shown below figure.

Figure 3-2 Electrical block diagram of DTN payload

Communication Protocols

There are two kinds of communication protocols between DTNPL and KPLO as following.

Table 4-1 DTNPL Protocols

|  | TM/TC Protocol | Datalink Protocol |
| --- | --- | --- |
| Communication link | DTNPL ↔ OBC(KPLO) | DTNPL → PDHU(KPLO) |
| Application | TM/TC communication (OBC ↔ DTNPL) File transfer using DTN (OBC → DTNPL) | File transfer using DTN (DTNPL → PDHU) Video streaming using DTN (DTNPL → PDHU) |
| ACK | Echo back | None |
| Single/Multiple | Single packet: TM/TC Multiple packets: File transfer | Multiple packets |

TM/TC Protocols

UART Protocol

DMA UART will be used for the communication between DTNPL and KPLO OBC.

- Baud rate: 115.2 kbps

- None-Parity, 8 Data bit and 1 Start/Stop bit (N, 8, 1)

- ACK method: Echo back

- ID pattern: 0xAA

Packet Format

The maximum packet size is 70 byte. The TM/TC packet is consists of following fields.

Figure 4-1 TM/TC packet structure

The packet fields are defined in following table.

Table 4-2 TM/TC Packet Format

| Field name | Size | Description | Value |
| --- | --- | --- | --- |
| Sync | 1 byte | Sync pattern for UART communication | 0xAA |
| Length | 1 byte | Length of data field (byte) | 2(0x02) ~ 67(0x43) |
| Command | 1 byte | Type (1 bit) | SBC(0), DTN(1) |
|  |  | Response Channel (1 bit) | OBC(0), PDHU(1) |
|  |  | Single/Multiple (1 bits) | Single packet(0) Multiple packet(1) |
|  |  | Write/Read (1 bits) | Write(0), Read(1) |
|  |  | Command number (4 bits) | 0(0x00) ~ 15(0x0F) |
| Data | Variable | Data (max. 67 byte) | Refer to Table 4-3 |

Table 4-3 Data Field Format

| Sub-field name | Size | Description | Value |
| --- | --- | --- | --- |
| Packet No | 2 byte | Single packet Multiple packets serial number | 0 0(0x00) ~ 65535(0xFFFF) |
| Data | 65 byte | Payload data |  |

TM/TC Data Structure

TM/TC data structure is as following.

Table 4-4 TM/TC data structure

| Type | Name | Sync | Length | Command | Data |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Packet No | Payload Data |
| SBC TC | Echo test | 0xAA | 0x02 + Payload Data length | 0x00 | 0x0000 | variable |
|  | SW initialize |  | 0x02 | 0x01 | 0x0000 | None |
|  | Time setting |  | 0x08 | 0x02 | 0x0000 | OBT - week since 2000-01-01 12:00:00 UTC (2 byte) - octant second (4 byte) |
|  | SOH request |  | 0x02 | 0x13 | 0x0000 | None |
|  | Script (ionstart) |  | 0x0E | 0x04 | 0x0000 | Script command (12 byte) |
|  | Script (killm) |  | 0x09 | 0x05 | 0x0000 | Script command (7 byte) |
|  | DCC data |  | 0x02~0x43 | 0x06 | 0x0000 ~ 0xFFFF | DCC TC/data (0 ~ 65 byte) |
|  | RFU1 |  | 0x2A | 0x07 | 0x0000 | Reserved (40 byte) |
|  | RFU2 |  | 0x2A | 0x08 | 0x0000 | Reserved (40 byte) |
|  | RFU3 |  | 0x2A | 0x09 | 0x0000 | Reserved (40 byte) |
|  | RFU4 |  | 0x2A | 0x0A | 0x0000 | Reserved (40 byte) |
|  | RFU5 |  | 0x2A | 0x0B | 0x0000 | Reserved (40 byte) |
| SBC TM | SOH data | 0xAA | 0x3C | 0x14 | 0x0000 | SOH data (58 byte) |
|  | RFU1 |  | 0x06 | 0x15 | 0x0000 | Reserved (4 byte) |
|  | RFU2 |  | 0x06 | 0x16 | 0x0000 | Reserved (4 byte) |
|  | RFU3 |  | 0x06 | 0x17 | 0x0000 | Reserved (4 byte) |
|  | RFU4 |  | 0x06 | 0x18 | 0x0000 | Reserved (4 byte) |
|  | RFU5 |  | 0x06 | 0x19 | 0x0000 | Reserved (4 byte) |

Datalink Protocol

Datalink protocol is used for payload data transfer from DTNPL to PDHU. USRT will be used for the datalink protocol and characteristics are as following.

Datalink protocol for downlink

The downlink datalink packet structure can be expressed logically as following.

- Packet structure = VN + Type + SHF + APID + SF + PSC + PL + SP + User data + CRC

  - Where; VN: Version Number, SHF: Secondary Header Flag, APID: Application Process ID, SF: Sequence Flags, PSC: Packet Sequence Count, PL: Packet Length, SP: Sync Pattern, CRC: Cyclic Redundancy Check

The downlink datalink packet structure can be expressed as following figure.

Figure 4-2 Downlink datalink packet structure

The datalink packet structure is as following.

Table 4-5 Datalink packet structure

| Field name | Size (bit) | Description | Comments |
| --- | --- | --- | --- |
| VN | 3 | 000b | Version 1 |
| Type | 1 | 0b | Telemetry |
| SHF | 1 | 1b |  |
| APID | 11 | 11 bits |  |
| SF | 2 | 11b (standalone) or combination of 01b / 00b / 10b (first / continue / last) |  |
| PSC | 14 | Modulo-16384, 0 ~ 16383 | The sequential binary count of each packet |
| PL | 16 | (Length of User Data Field) - 1 |  |
| SP | 32 | 0x352EF853 |  |
| User Data Field | Variable | Max 65536 Octets. Refer to Table 4-6. |  |
| CRC | 16 | CRC-16-CCITT (Polynomial: x16+x12+x5+1) |  |

The contents of User Data Field are as following.

Table 4-6 Contents of User Data Field

| Sub-field | Size (bit) | Description | Comments |
| --- | --- | --- | --- |
| Sync | 8 | Sync pattern for USRT communication |  |
| Length | 8 | Refer to Table 4-2 and Table 4-3 |  |
| Command | 8 |  |  |
| Packet No. | 16 |  |  |
| Data | Variable | Variable. Max 65531 Octets. | DTN TM - AMS message - CFDP file data |

Datalink Data Structure

Datalink data structure is as following.

Table 4-7 First frame datalink structure

| VN | Type | SHF | APID | SF | PSC | PL | SP | User Data Field | CRC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 000b | 0b | 1b | xxx xxxx xxxxb | 01b | 00 0000 0000 0000b | xxxx xxxx …. | 0x352EF853 | Variable | [TBD] |

Table 4-8 Last frame datalink structure

| VN | Type | SHF | APID | SF | PSC | PL | SP | User Data Field | CRC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 000b | 0b | 1b | xxx xxxx xxxxb | 10b | xx xxxx xxxx xxxxb | xxxx xxxx …. | 0x352EF853 | Variable | [TBD] |

TM/TC Lists

Command Lists

There are two kinds of command types in the DTNPL as following.

- SBC commands: DTNPL control

- DTN commands: ION-DTN control. They are transmitted by AMS from DCC to DTNPL.

The DTNPL command lists are shown in following table.

Table 4-9 DTNPL Command Lists

| Type | TC name | Description |
| --- | --- | --- |
| SBC | Echo test | UART communication checking |
|  | SW initialize | Initialize system |
|  | Time setting | Set On Board Time |
|  | SOH request | Request SOH TM |
|  | RFU_1 | Reserved for future use |
|  | RFU_2 | Reserved for future use |
|  | RFU_3 | Reserved for future use |
|  | RFU_4 | Reserved for future use |
|  | RFU_5 | Reserved for future use |
| DTN | Script (ion start) | ION Start |
|  | Script (killm) | Close all process |
|  | DCC data | AMS TC / CFDP file data transfer |

Telemetry Lists

There are two kinds of telemetry types as following.

- SBC telemetries: The SOH (State Of Health) of DTN payload to be reported to the KPLO FSW to monitor DTNPL health condition.

- DTN telemetries: Response to DTN command. They are transmitted by AMS from DTNPL to DCC.

The detailed telemetry lists are shown in below.

Table 4-10 DTNPL Telemetry Lists

| Type | TM name | Description | Size (byte) | Start time (sec.) | Update interval | Transmit interval to FSW |
| --- | --- | --- | --- | --- | --- | --- |
| SBC | SOH Data | temp_board (Board temperature) | 4 | Auto count after DTNPL power ON | <1 sec. | After receiving SOH request (32 sec). |
|  |  | temp_fpga (FPGA temperature) | 4 |  | <1 sec. |  |
|  |  | volt_28 (28.0V input voltage level) | 2 |  | <1 sec. |  |
|  |  | volt_3.3 (3.3V voltage level) | 2 |  | <1 sec. |  |
|  |  | volt_1.0 (1.0V voltage level) | 2 |  | <1 sec. |  |
|  |  | curr_28 (28.0V input current level) | 2 |  | <1 sec. |  |
|  |  | time (Current DTNPL time) | 6 |  | 1 sec. |  |
|  |  | run_time (Time(second) count after DTNPL power on) | 4 |  | 1 sec. |  |
|  |  | fpga_ver (SBC FPGA version) | 4 |  | Fixed |  |
|  |  | fw_ver (SBC firmware version) | 4 |  | Fixed |  |
|  |  | edac (EDAC count) | 4 |  | Event occurred |  |
|  |  | tmtc_rx_pkt_cnt (OBC→DTNPL received packet count) | 4 |  | When packet received |  |
|  |  | tmtc_tx_pkt_cnt (DTNPL→OBC send packet count) | 4 |  | When packet sent |  |
|  |  | downlink_pkt_cnt (DTNPL→PDHU send packet count) | 4 |  | When packet sent |  |
|  |  | dtn_state (ION start/stop state) | 4 |  | 1 sec. |  |
|  |  | ion_ver (ION version) | 4 |  | Fixed |  |
|  | RFU_1 | Reserved for future use | 4 | - | [TBD] | [TBD] |
|  | RFU_2 | Reserved for future use | 4 | - | [TBD] | [TBD] |
|  | RFU_3 | Reserved for future use | 4 | - | [TBD] | [TBD] |
|  | RFU_4 | Reserved for future use | 4 | - | [TBD] | [TBD] |
|  | RFU_5 | Reserved for future use | 4 | - | [TBD] | [TBD] |
| DTN | [TBD] | [TBD] | - | - | - | - |