Korea Lunar Exploration Program

DTNPL Software Test Plan & Procedures

| Date: 28 Dec. 2017 | Doc. No: KPLO-D0-550-202 Issue: 1.0 Total page: 25 |
| --- | --- |

Superseding     Revised

| Prepared By: Hyun-Woong Choi Date DTN SBC S/W Engineer, Lumir Dae-Soo Oh Date DTN SBC Engineer, Lumir |  | Kyung-Rak Lee Date DTN Payload Engineer, ETRI |
| --- | --- | --- |
| Reviewed By: Jin-Ho Jo Date DTN System Engineer, ETRI |  | Sin-Ae Ji Date Proximity-1 System Engineer, ETRI |
| Sang-Man Moon Date RLE KPLO Electrical System, KARI |  | Hyun-Jin Shin Date RLE KPLO Mechanical System, KARI |
| Approval Signature: Byoung-Sun Lee Date DTN Project Leader, ETRI |  | Il-Young Kim Date RLE KPLO Product Assurance, KARI |
| Seong-Bin Lim Date Head, Lunar Exploration System Team, KARI |  | Seok-Weon Choi Date Director, Lunar Exploration Program Office, KARI |

Original CDMO Release__________________

| Revision History |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version | Author(s) |  | Date | Description |
| 0.1 | Hyun-Woong Choi |  | 2017. 12. 28 | Draft |

| Revision/Change Records |  |  |  |
| --- | --- | --- | --- |
| Version | Date | Revision/Change Description | Pages |
| 0.1 | 2017. 12. 28 | Initial | All |

Documents Tree

Contents

1.	Introduction	8

1.1.	Purpose and Scope	8

1.2.	Applicable Documents	8

1.2.1.	ETRI Documents	8

1.2.2.	Other Documents	8

1.3.	Abbreviations	8

2.	Overview of KPLO DTN	10

2.1.	DTN and Space Communications	10

2.2.	Mission of DTN Operation	10

2.3.	DTN Configurations	10

2.4.	Communication Nodes	11

2.4.1.	DTN nodes	11

2.4.2.	Non-DTN nodes	11

3.	Software Test Plan	12

3.1.	Test Target	12

3.1.1.	ION	12

3.1.2.	DTNPL Management	12

3.1.3.	SCH	12

3.1.4.	DMA_UART	12

3.1.5.	USRT	13

3.2.	Approach	14

3.3.	Pass/Fail Criteria	14

3.3.1.	TC Verification	14

3.3.2.	Functional Test	14

4.	Software Test Procedure	15

4.1.	Test Setting	15

4.2.	Test Steps	15

4.2.1.	Power supply	15

4.2.3.	DTNPL EGSE PC Set Up	16

4.2.4.	Telecommand Test	19

4.2.5.	Telemetry Test	23

Figures

Figure 2.1 DTN test configuration in the first phase	11

Figure 3.1 DTNPL Software Layer	12

Figure 3.2 DTNPL Software Archetecture	12

Figure 3.3 DTNPL Test Configuration	14

Figure 4.1 run time test result	15

Figure 4.2 DTNPL EGSE GUI	16

Figure 4.3 Command and Telemetry Server Window	17

Figure 4.4 Command Sender Window	17

Figure 4.5 Packet Viewer Window	18

Figure 4.6 Command and Telemetry Server Window	18

Figure 4.7 TC DDC DATA	19

Figure 4.8 TC ECHO TEST	19

Figure 4.9 TC SW INITIALIZE	20

Figure 4.10 TC ION START	20

Figure 4.11 TC ION STOP	20

Figure 4.12 TC SOH REQUEST	21

Figure 4.13 TC TIME SETTING	21

Figure 4.14 SOH Telemetry	23

Tables

Table 4-1 DTNPL Test Configuration	15

Table 4-1 DTNPL Command Data Structure	22

Table 4-2 DTNPL Command Data Structure	22

Table 4-2 DTNPL Telemetry Lists	24

Introduction

Purpose and Scope

This document defines the electrical and mechanical interfaces between KPLO bus and DTN payload.

Applicable Documents

ETRI Documents

| Document Number | Document Title |
| --- | --- |
| KPLO-D0-550-000 | System Requirement of DTN Payload |
| KPLO-D0-550-001 | System Specification of DTN Payload |

Other Documents

| Document Number | Document Title |
| --- | --- |
| KPLO-SP-310-001 | KPLO System Specification Document |
| KPLO-SP-340-001 | KPLO Payload/Spacecraft Bus ICD |

Abbreviations

| AMS | Asynchronous Message Service |
| --- | --- |
| BP | Bundle Protocol |
| BSS | Bundle Streaming Service |
| CCSDS | Consultative Committee for Space Data Systems |
| CFDP | CCSDS File Delivery Protocol |
| DCC | DTN Control Center |
| CRC | Cyclical Redundancy Check |
| DMA | Direct Memory Access |
| DTN | Delay(Disruption) Tolerant Network |
| DTNPL | DTN Payload |
| EOD | End of Data |
| EOF | End of File |
| FW | Firm Ware |
| FSW | Flight Software |
| ION | Interplanetary Overlay Network |
| KDSA | Korea Deep Space Antenna |
| KPLO | Korea Pathfinder Lunar Orbiter |
| LCM | Lander Communication Model |
| LTP | Licklider Transmission Protocol |
| MOC | Mission Operation Center |
| OBC | On Board Computer |
| OBT | On Board Time |
| PDHU | Payload Data Handling Unit |
| RCM | Rover Communication Model |
| RFU | Reserved for Future Use |
| SBC | Single Board Computer |
| SOD | Start of Data |
| SOF | Start of File |
| SOH | Start of Head |
| TBC | To Be Confirmed |
| TBD | To Be Defined |
| TC | Tele-command |
| TCP | Transmission Control Protocol |
| TM | Telemetry |
| UART | Universal Asynchronous Receiver Transmitter |
| USRT | Universal Synchronous Receiver Transmitter |

Overview of KPLO DTN

DTN and Space Communications

Delay/disruption tolerant networking (DTN) technology offers a new solution to highly stressed communications in space environments, especially those with long link delay and frequent link disruptions in deep space missions. Delay/disruption tolerant networking (DTN) technology offers a new solution that use a store-and-forward mechanism to combat long link delay and frequent link disruptions generally characterizing space communications. DTN communications use a bundle protocol (BP), to construct a store-and-forward overlay network. The BP, situated at the application layer of the Internet model, utilizes conventional Internet protocols to send and receive data bundles. The benefits of DTN communications are as follows.

- Interoperability and Reuse: A standardized DTN protocol suite enables interoperability of multi-agency communication assets and also allows NASA to use the same communication stack for future missions (LEO, NEO or Deep Space).

- Space Link Efficiency, Utilization and Robustness: DTN enables more reliable and efficient data transmissions resulting in more usable bandwidth. DTN also improves link reliability by having multiple network paths and assets for potential communication hops.

- Security: The DTN Bundle Security Protocol (BSP) allows for integrity checks, authentication and encryption, even on links where not previously used.

- Quality of Service: The DTN protocol suite allows for many priority levels to be set for different data types, ensuring that the most important data is received ahead of less important data.

- Improved Operations and Situational Awareness: The DTN store-and-forward mechanism along with automatic retransmission provides more insight into events during communication outages and significantly reduces the need for ground-based scheduling.

Mission of DTN Operation

The mission of DTN is to validate DTN communication protocols by space link testing through KPLO. So DTN operations are focused on testing the DTN protocols using KPLO space links.

DTN Configurations

In the first phase of lunar exploration, test lunar orbiter called Korea Pathfinder Lunar Orbiter (KPLO) will be launched without lunar lander. Figure 2.1 shows DTN configuration in first phase of lunar exploration. There are no Lander and Rover on the moon in first phase. So Lander Communication Model (LCM) and Rover Communication Model (RCM) will be located on earth for the DTN communication testing.

DTN Payload (DTNPL) is a DTN node in the space. DTNPL will be mounted on the KPLO for DTN communication. DTN Control Center (DCC) is a control center of DTN communication nodes. LCM and RCM are DTN nodes on ground. All DTN nodes on ground are interconnected by the terrestrial network such as Internet. Mission Operation Center (MOC) and Korea Deep Space Antenna (KDSA) are not DTN nodes. They just transfer DTN packets to next communication nodes.

Figure 2.1 DTN test configuration in the first phase

Communication Nodes

Many communication nodes are needed for the DTN operation. Some of communication nodes in the network are DTN nodes whereas otherwise nodes are non-DTN nodes. The function of communication nodes used in the DTN operation is as follows.

DTN nodes

- DCC is a ground DTN node which monitor and controls DTN nodes. DCC send commands to DTN nodes such as DTNPL, LCM and RCM. It also receives telemetries or acquired data from some DTN nodes such as RCM or LCM.

- DTNPL is a DTN node mounted on the KPLO. DTNPL can be operated as a DTN end point. And it also can be operated as relay nodes which relay DTN data between DCC and RCM or LCM.

- LCM is a ground DTN node. LCM relays DTN data between MOC and RCM.

- RCM is a ground DTN node and a DTN end point. The commands sent by the DCC is terminated at the RCM and the RCM returns telemetry or data to DCC through KPLO.

Non-DTN nodes

- MOC is a non-DTN node. MOC monitors and controls KPLO and payloads. MOC also receives and processes data generated from the payloads and transfers to each payload control center.

- KDSA is a ground station for deep space communication. KDSA will have a big size of antenna and RF equipment for space link interface. KDSA also has network equipment for terrestrial communication interface such as SLE link. KDSA is a non-DTN node.

Software Test Plan

Test Target

NASA CFS is used as a DTNPL framework which was used for satellite in history. Because of this, the applications layer will be focused on during the test. The layer of DTNPL Software is shown below.

Figure 3.1 DTNPL Software Layer

All DTNPL software will be test. The target software apps are shown on below.

Figure 3.2 DTNPL Software Archetecture

ION

DTNPL Mission Processing will be tested.

DTNPL Management

Operation mode, FDIR will be tested.

SCH

Schedule management will be tested.

DMA_UART

Telecommand and Telemetry will be tested.

USRT

Datalink with PDHU will be tested.

Approach

DTN EGSE will be used to test all the software functionalities and performances. The test configuration is shown on below.

Figure 3.3 DTNPL Test Configuration

Pass/Fail Criteria

Pass/Fail Criteria will set up to test DTNPL. The criteria is divided into two parts TC verification and functional test.

TC Verification

TC verification is used to define whether the telecommand is valid or not. DTNPL EGSE can be used as an OBC and send telecommands. The DTNPL EGSE must receive expected response from DTNPL.

Functional Test

The functional test can be performed after telecommand verification is done. During this test, the task execution time will be measured.

Software Test Procedure

Test Setting

The DTNPL software test configuration is introduced on table below

Table 4-1 DTNPL Test Configuration

| Name of items | Number of items | Description |
| --- | --- | --- |
| DTNPL | 1 | DTNPL is a Unit Under Test(UUT). |
| Power Supply | 1 | This power supply is used to provide enough power to DTNPL.. The power must set to 28V. |
| DTNPL EGSE BOX | 1 | DTNPL EGSE BOX is directly connected to DTNPL and provide all equipment to run test.. |
| DTNPL EGSE PC | 1 | The EGSE PC is also directly connected to DTNPL and provide user interface for software test.. |

This test can be set by following figure.

Figure 4.1 run time test result

Test Steps

Power supply

The DTNPL power supply is used to supply power to DTNPL.

- Voltage setting value: 28V

- Current limit setting value: 0.4A

DTNPL EGSE PC Set Up

- The “DTN EGSE.exe” file is clicked to enter DTNPL EGSE software test GUI

Figure 4.2 DTNPL EGSE GUI

- Command and Telemetry Server is clicked to initiate test connection between DTNPL EGSE and DTNPL

  - The green “true” indicates the connection status

Figure 4.3 Command and Telemetry Server Window

- Telecommand can be enabled by clicking Cmd Packets icon on command and telemetry server window.

  - Command sender window will be shown.

Figure 4.4 Command Sender Window

- Telemetry packets can be monitored by clicking “Tlm Packets” bar

Figure 4.5 Packet Viewer Window

- The log of test process is show on “Logging” bar section.

Figure 4.6 Command and Telemetry Server Window

Telecommand Test

Command sender windows shows the available TCs for test.

DDC_DATA

DDC_DATA can be selected to test and the setting values for DDC_DATA input values can be manually entered or loaded. The interface of DDC_DATA is shown on below.

Figure 4.7 TC DDC DATA

ECHO_TEST

ECHO TEST checked the reliability of communication connection between DTNPL and DTNPL EGSE. ECHO_TEST can be selected to test and the setting values for ECHO_TEST input values can be manually entered or loaded. The interface of ECHO_TEST is shown on below.

Figure 4.8 TC ECHO TEST

.

SW_INITIALIZE

The SW_INITIALIZE TC is used to initialize software and the command does not include any user inputs. The interface of software initialization TC is shown below.

Figure 4.9 TC SW INITIALIZE

ION_START and ION_STOP

The ION START is used to start ION and ION STOP is used to stop ION operation. The interface of ION start and stop TC is shown below.

| Figure 4.10 TC ION START | Figure 4.11 TC ION STOP |
| --- | --- |

SOH_REQUEST

The SOH REQUEST TC requests state of health information. The DTNPL is designed to send state of health information every 32 seconds but state of health information can be requested anytime by this telecommand. The interface of SOH request is shown below.

Figure 4.12 TC SOH REQUEST

TIME_SETTING

The TIME_SETTING command is used to control time in DTNPL. The week and second time can be set. The interface of Time setting is shown below.

Figure 4.13 TC TIME SETTING

The summary of telecommand test list is shown below.

Table 4-1 DTNPL Command Data Structure

| Type | Name | Sync | Length | Command | Data |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Packet No | Payload Data |
| SBC TC | Echo test | 0xAA | 0x02 + Payload Data length | 0x00 | 0x0000 | variable |
|  | SW initialize |  | 0x02 | 0x01 | 0x0000 | None |
|  | Time setting |  | 0x08 | 0x02 | 0x0000 | OBT - week since 2000-01-01 12:00:00 UTC (2 byte) - octant second (4 byte) |
|  | SOH request |  | 0x02 | 0x13 | 0x0000 | None |
|  | Script (ionstart) |  | 0x0E | 0x04 | 0x0000 | Script command (12 byte) |
|  | Script (killm) |  | 0x09 | 0x05 | 0x0000 | Script command (7 byte) |
|  | DCC data |  | 0x02~0x43 | 0x06 | 0x0000 ~ 0xFFFF | DCC TC/data (0 ~ 65 byte) |
|  | RFU1 |  | 0x2A | 0x07 | 0x0000 | Reserved (40 byte) |
|  | RFU2 |  | 0x2A | 0x08 | 0x0000 | Reserved (40 byte) |
|  | RFU3 |  | 0x2A | 0x09 | 0x0000 | Reserved (40 byte) |
|  | RFU4 |  | 0x2A | 0x0A | 0x0000 | Reserved (40 byte) |
|  | RFU5 |  | 0x2A | 0x0B | 0x0000 | Reserved (40 byte) |
| SBC TM | SOH data | 0xAA | 0x3C | 0x14 | 0x0000 | SOH data (58 byte) |
|  | RFU1 |  | 0x06 | 0x15 | 0x0000 | Reserved (4 byte) |
|  | RFU2 |  | 0x06 | 0x16 | 0x0000 | Reserved (4 byte) |
|  | RFU3 |  | 0x06 | 0x17 | 0x0000 | Reserved (4 byte) |
|  | RFU4 |  | 0x06 | 0x18 | 0x0000 | Reserved (4 byte) |
|  | RFU5 |  | 0x06 | 0x19 | 0x0000 | Reserved (4 byte) |

Table 4-2 DTNPL Command Data Structure

| Type | TC name | Description |
| --- | --- | --- |
| SBC | Echo test | UART communication checking |
|  | SW initialize | Initialize system |
|  | Time setting | Set On Board Time |
|  | SOH request | Request SOH TM |
|  | RFU_1 | Reserved for future use |
|  | RFU_2 | Reserved for future use |
|  | RFU_3 | Reserved for future use |
|  | RFU_4 | Reserved for future use |
|  | RFU_5 | Reserved for future use |
| DTN | Script (ion start) | ION Start |
|  | Script (killm) | Close all process |
|  | DCC data | AMS TC / CFDP file data transfer |

Telemetry Test

Telemetry can be transmitted every 32 seconds since DTNPL is operating or by a SOH request command. The telemetry interface is shown on below.

Figure 4.14 SOH Telemetry

The detail of tested telemetry is shown below.

Table 4-2 DTNPL Telemetry Lists

| Type | TM name | Description | Size (byte) | Start time (sec.) | Update interval | Transmit interval to FSW |
| --- | --- | --- | --- | --- | --- | --- |
| SBC | SOH Data | temp_board (Board temperature) | 4 | Auto count after DTNPL power ON | <1 sec. | After receiving SOH request (32 sec). |
|  |  | temp_fpga (FPGA temperature) | 4 |  | <1 sec. |  |
|  |  | volt_28 (28.0V input voltage level) | 2 |  | <1 sec. |  |
|  |  | volt_3.3 (3.3V voltage level) | 2 |  | <1 sec. |  |
|  |  | volt_1.0 (1.0V voltage level) | 2 |  | <1 sec. |  |
|  |  | curr_28 (28.0V input current level) | 2 |  | <1 sec. |  |
|  |  | time (Current DTNPL time) | 6 |  | 1 sec. |  |
|  |  | run_time (Time(second) count after DTNPL power on) | 4 |  | 1 sec. |  |
|  |  | fpga_ver (SBC FPGA version) | 4 |  | Fixed |  |
|  |  | fw_ver (SBC firmware version) | 4 |  | Fixed |  |
|  |  | edac (EDAC count) | 4 |  | Event occurred |  |
|  |  | tmtc_rx_pkt_cnt (OBC→DTNPL received packet count) | 4 |  | When packet received |  |
|  |  | tmtc_tx_pkt_cnt (DTNPL→OBC send packet count) | 4 |  | When packet sent |  |
|  |  | downlink_pkt_cnt (DTNPL→PDHU send packet count) | 4 |  | When packet sent |  |
|  |  | dtn_state (ION start/stop state) | 4 |  | 1 sec. |  |
|  |  | ion_ver (ION version) | 4 |  | Fixed |  |
|  | RFU_1 | Reserved for future use | 4 | - |  |  |
|  | RFU_2 | Reserved for future use | 4 | - |  |  |
|  | RFU_3 | Reserved for future use | 4 | - |  |  |
|  | RFU_4 | Reserved for future use | 4 | - |  |  |
|  | RFU_5 | Reserved for future use | 4 | - |  |  |
| DTN |  |  | - | - |  |  |