Korea Lunar Exploration Program

DTNPL Mass Properties Report

| Date: 29 Oct. 2018 | Doc. No: KPLO-D0-550-101 Issue: 1.0 Total page: 15 |
| --- | --- |

Superseding     Revised

| Prepared By: Jeong-Chan Lee Date DTN SBC Mechanical Engineer, Lumir |  |  |
| --- | --- | --- |
| Reviewed By: Chang-Soo Lee Date DTN SBC Product Assurance, Lumir |  | Dae-Soo Oh Date DTN SBC Project Manager, Lumir |
| Kyung-Rak Lee Date DTN Payload Engineer, ETRI |  |  |
| Approval Signature: Choel-Oh Jeong Date DTN Payload Product Assurance, ETRI |  | Jin-Ho Jo Date DTN Payload Project Leader, ETRI |

Original CDMO Release__________________

| Revision History |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version | Author(s) |  | Date | Description |
| 0.1 | Jeong-Chan Lee |  | 2017. 12. 28 | Draft |
| 1.0 | Jeong-Chan Lee |  | 2018. 10. 29 | Final |

| Revision/Change Records |  |  |  |
| --- | --- | --- | --- |
| Version | Date | Revision/Change Description | Pages |
| 0.1 | 2017. 12. 28 | Initial | All |
| 1.0 | 2018. 10. 29 | Final | All |

Documents Tree

Contents

1.	Introduction	8

1.1.	Purpose and Scope	8

1.2.	Applicable Documents	8

1.2.1.	ETRI Documents	8

1.2.2.	Other Documents	8

1.3.	Abbreviations	8

2.	Operation Description	9

2.1.	DTN and Space Communications	9

2.2.	Mission of DTN Operation	9

2.3.	DTN Configurations	9

2.4.	Communication Nodes	10

2.4.1.	DTN nodes	10

2.4.2.	Non-DTN nodes	10

3.	System Configuration of DTN Payload	11

4.	Mass Property	12

4.1.	Mechnical Configuration	12

4.2.	Mechanical Interface	13

4.3.	Mass Budget	13

4.4.	Center of gravity	14

4.5.	Moment of inertia	15

Figures

Figure 2-1 DTN test configuration in the first phase	10

Figure 3-1 Interface diagram of DTN payload	11

Figure 3 Mechanical Configuration	12

Figure 4 exploded configuration	12

Figure 5 DTN PL interface design	13

Figure 6 Center of gravity	14

Tables

Table 1 Mass Budjet	13

Table 2 Center of gravity	14

Table 3 Moment of Inertia	15

Introduction

Purpose and Scope

This document describes the system specifications of DTN payload for Korea lunar exploration program.

Applicable Documents

ETRI Documents

| Document Number | Document Title |
| --- | --- |
| KPLO-D0-551-002 | Operation Scenario of DTN |
| KPLO-D0-550-000 | System Requirements of DTN Payload |

Other Documents

| Document Number | Document Title |
| --- | --- |
| KPLO-SP-300-002 | KPLO System Requirements Document |

Abbreviations

| AMS | Asynchronous Message Service |
| --- | --- |
| AOS | Advanced Orbiting System |
| BP | Bundle Protocol |
| BSS | Bundle Streaming Service |
| CCSDS | Consultative Committee for Space Data Systems |
| CFDP | CCSDS File Delivery Protocol |
| DCC | DTN Control Center |
| DTN | Delay(Disruption) Tolerant Network |
| DTNPL | DTN Payload |
| IP | Internet Protocol |
| KPLO | Korea Pathfinder Lunar Orbiter |
| LCM | Lander Communication Model |
| LTP | Licklider Transmission Protocol |
| M&C | Monitor & Control |
| RCM | Rover Communication Model |
| SLE | Space Link Extension |
| TBC | To Be Confirmed |
| TBD | To Be Defined |
| TCP | Transmission Control Protocol |
| UDP | User Datagram Protocol |

Operation Description

DTN and Space Communications

Delay/disruption tolerant networking (DTN) technology offers a new solution to highly stressed communications in space environments, especially those with long link delay and frequent link disruptions in deep space missions. Delay/disruption tolerant networking (DTN) technology offers a new solution that use a store-and-forward mechanism to combat long link delay and frequent link disruptions generally characterizing space communications. DTN communications use a bundle protocol (BP), to construct a store-and-forward overlay network. The BP, situated at the application layer of the Internet model, utilizes conventional Internet protocols to send and receive data bundles. The benefits of DTN communications are as follows.

- Interoperability and Reuse: A standardized DTN protocol suite enables interoperability of multi-agency communication assets and also allows NASA to use the same communication stack for future missions (LEO, NEO or Deep Space).

- Space Link Efficiency, Utilization and Robustness: DTN enables more reliable and efficient data transmissions resulting in more usable bandwidth. DTN also improves link reliability by having multiple network paths and assets for potential communication hops.

- Security: The DTN Bundle Security Protocol (BSP) allows for integrity checks, authentication and encryption, even on links where not previously used.

- Quality of Service: The DTN protocol suite allows for many priority levels to be set for different data types, ensuring that the most important data is received ahead of less important data.

- Improved Operations and Situational Awareness: The DTN store-and-forward mechanism along with automatic retransmission provides more insight into events during communication outages and significantly reduces the need for ground-based scheduling.

Mission of DTN Operation

The mission of DTN is to validate DTN communication protocols by space link testing through KPLO. So DTN operations are focused on testing the DTN protocols using KPLO space links.

DTN Configurations

In the first phase of lunar exploration, test lunar orbiter called Korea Pathfinder Lunar Orbiter (KPLO) will be launched without lunar lander. Figure 2.1 shows DTN configuration in first phase of lunar exploration. There are no Lander and Rover on the moon in first phase. So Lander Communication Mode (LCM) and Rover Communication Mode (RCM) will be located on earth for the DTN communication testing.

DTN Payload (DTNPL) is a DTN node in the space. DTNPL will be mounted on the KPLO for DTN communication. DTN Control Center (DCC) is a control center of DTN communication nodes. LCM and RCM are DTN nodes on ground. All DTN nodes on ground are interconnected by the terrestrial network such as Internet. MCC and DSN are not DTN nodes. They just transfer DTN packets to next communication nodes.

Figure 2-1 DTN test configuration in the first phase

Communication Nodes

Many communication nodes are needed for the DTN operation. Some of communication nodes in the network are DTN nodes whereas otherwise nodes are non-DTN nodes. The function of communication nodes used in the DTN operation is as follows.

DTN nodes

- DCC is a ground DTN node which monitor and controls DTN nodes. DCC send commands to DTN nodes such as DTNPL, LCM and RCM. It also receives telemetries or acquired data from some DTN nodes such as RCM or LCM.

- DTNPL is a DTN node mounted on the KPLO. DTNPL can be operated as a DTN end point. And it also can be operated as relay nodes which relay DTN data between DCC and RCM or LCM.

- LCM is a ground DTN node. LCM relays DTN data between MCC and RCM.

- RCM is a ground DTN node and a DTN end point. The commands sent by the DCC is terminated at the RCM and the RCM returns telemetry or data to DCC through KPLO.

Non-DTN nodes

- MCC is a non-DTN node. MCC monitors and controls KPLO and payloads. MCC also receives and processes data generated from the payloads and transfers to each payload control center.

- DSN is a ground station for deep space communication. DSN will have a big size of antenna and RF equipment for space link interface. DSN also have network equipment for terrestrial communication interface such as SLE link. DSN is a non-DTN node.

System Configuration of DTN Payload

The DTN payload shall have interface with KPLO for the operational power supply and for the communications. The following figure shows DTN payload interface diagram with KPLO.

Figure 3-1 Interface diagram of DTN payload

There are four kinds of interface types between DTNPL and KPLO. Each interface types are described as following.

- TM/TC (bi-level): One bi-level TC and one bi-level TM for DTNPL emergency operation.

- Bi-level TC: System reset command in the emergence condition

- Bi-level TM: Telemetry showing that DTNPL is in alive

- TM/TC (UART): Serial communication channel for TM/TC communication with KPLO OBC

- Data downlink [TBD]: Serial data downlink channel to KPLO PDHU

- Power: Operation DC power +28V from the KPLO PCDU

Mass Property

Mechnical Configuration

DTN PL consisted of 1 PCB modules, 1 Motherboard, 1 Top Cover.

Total Mass: 0.89±2% kg

Dimension: W x L x H = 120 x 170 x 39 mm

Figure 3 Mechanical Configuration

Figure 4 exploded configuration

Mechanical Interface

The DTN PL is fixed 6 holes by M4 bolts on spacecraft

Figure 5 DTN PL interface design

Mass Budget

Table 1 Mass Budget

| Name | Mass [kg] | Remark |
| --- | --- | --- |
| Housing | 0.37 | Measured weight |
| Top Cover | 0.23 |  |
| SBC PCB+EEE Parts | 0.15 |  |
| etc (bolts, bonding, coating) | 0.14 |  |
| Sum | 0.89 | Requirement: < 2 kg |

Center of gravity

Center of gravity is shown in the following figure and table.

Figure 6 Center of gravity

Table 2 Center of gravity

| Axis | Location |
| --- | --- |
| X | 85.35 |
| Y | 60.66 |
| Z | 19.61 |

Moment of inertia

Moment of inertia is shown in the following table.

Table 3 Moment of Inertia

| Moment of inertia |  |
| --- | --- |
| Ixx [ kg-m2 ] | 0.004 |
| Iyy [ kg-m2 ] | 0.008 |
| Izz [ kg-m2 ] | 0.011 |
| Ixy [ kg-m2 ] | -0.004 |
| Iyz [ kg-m2 ] | -0.001 |
| Ixz [ kg-m2 ] | -0.00874 |