Korea Lunar Exploration Program

DTNPL Thermal Analysis Report

| Date: 28 Dec. 2017 | Doc. No: KPLO-D0-550-104 Issue: 1.0 Total page: 17 |
| --- | --- |

Superseding     Revised

| Prepared By: Jeong-Chan Lee Date DTN SBC Mechanical Engineer, Lumir Dae-Soo Oh Date DTN SBC Engineer, Lumir |  | Kyung-Rak Lee Date DTN Payload Engineer, ETRI |
| --- | --- | --- |
| Reviewed By: Jin-Ho Jo Date DTN System Engineer, ETRI |  | Sin-Ae Ji Date Proximity-1 System Engineer, ETRI |
| Sang-Man Moon Date RLE KPLO Electrical System, KARI |  | Hyun-Jin Shin Date RLE KPLO Mechanical System, KARI |
| Approval Signature: Byoung-Sun Lee Date DTN Project Leader, ETRI |  | Il-Young Kim Date RLE KPLO Product Assurance, KARI |
| Seong-Bin Lim Date Head, Lunar Exploration System Team, KARI |  | Seok-Weon Choi Date Director, Lunar Exploration Program Office, KARI |

Original CDMO Release__________________

| Revision History |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version | Author(s) |  | Date | Description |
| 0.1 | Jeong-Chan Lee |  | 2017. 12. 28 | Draft |

| Revision/Change Records |  |  |  |
| --- | --- | --- | --- |
| Version | Date | Revision/Change Description | Pages |
| 0.1 | 2017. 12. 28 | Initial | All |

Documents Tree

Contents

1.	Introduction	8

1.1.	Purpose and Scope	8

1.2.	Applicable Documents	8

1.2.1.	ETRI Documents	8

1.2.2.	Other Documents	8

1.3.	Abbreviations	8

2.	Operation Description	9

2.1.	DTN and Space Communications	9

2.2.	Mission of DTN Operation	9

2.3.	DTN Configurations	9

2.4.	Communication Nodes	10

2.4.1.	DTN nodes	10

2.4.2.	Non-DTN nodes	10

3.	Thermal Analysis of DTN Payload	11

3.1.	Thermal analysis purposes and model definition	11

3.2.	Analysis Conditions	11

3.2.1.	Information Analysis	11

3.2.2.	Material property	12

3.2.3.	Contact condition	12

3.3.	Result of Analysis	13

3.3.1.	All Body	13

3.3.2.	PCB thermal analysis result	15

3.4.	Review results	16

Figures

Figure 2.1 DTN test configuration in the first phase	10

Figure 2 Thernal Analysis Target Model Configuration	11

Figure 3 FEM Model for Target Model	12

Figure 4 Temperature Distribution Contour for All Body (Housing)	13

Figure 5 Temperature Distribution Contour for All Body (Board)	13

Figure 6 PCB Calorific value information (Top)	14

Figure 7 PCB Calorific value information (Bottom)	14

Figure 8 Thermal analysis results for the PCB (Top)	15

Figure 9 Thermal analysis results for the PCB(Bottom)	15

Tables

Table 1  Material property	12

Table 2 Contaco Conditions	12

Table 3 PCB Thermal analysis result	16

Introduction

Purpose and Scope

This document describes the Thermal Analysis of DTN payload for Korea lunar exploration program.

Applicable Documents

ETRI Documents

| Document Number | Document Title |
| --- | --- |
| KPLO-D0-551-002 | Operation Scenario of DTN |
| KPLO-D0-550-000 | System Requirements of DTN Payload |

Other Documents

| Document Number | Document Title |
| --- | --- |
| KPLO-SP-300-002 | KPLO System Requirements Document |

Abbreviations

| AMS | Asynchronous Message Service |
| --- | --- |
| AOS | Advanced Orbiting System |
| BP | Bundle Protocol |
| BSS | Bundle Streaming Service |
| CCSDS | Consultative Committee for Space Data Systems |
| CFDP | CCSDS File Delivery Protocol |
| DCC | DTN Control Center |
| DTN | Delay(Disruption) Tolerant Network |
| DTNPL | DTN Payload |
| IP | Internet Protocol |
| KPLO | Korea Pathfinder Lunar Orbiter |
| M&C | Monitor & Control |
| ISTS | Integrated Satellite Test Software |
| TBC | To Be Confirmed |
| TBD | To Be Defined |
| TCP | Transmission Control Protocol |

Operation Description

DTN and Space Communications

Delay/disruption tolerant networking (DTN) technology offers a new solution to highly stressed communications in space environments, especially those with long link delay and frequent link disruptions in deep space missions. Delay/disruption tolerant networking (DTN) technology offers a new solution that use a store-and-forward mechanism to combat long link delay and frequent link disruptions generally characterizing space communications. DTN communications use a bundle protocol (BP), to construct a store-and-forward overlay network. The BP, situated at the application layer of the Internet model, utilizes conventional Internet protocols to send and receive data bundles. The benefits of DTN communications are as follows.

- Interoperability and Reuse: A standardized DTN protocol suite enables interoperability of multi-agency communication assets and also allows NASA to use the same communication stack for future missions (LEO, NEO or Deep Space).

- Space Link Efficiency, Utilization and Robustness: DTN enables more reliable and efficient data transmissions resulting in more usable bandwidth. DTN also improves link reliability by having multiple network paths and assets for potential communication hops.

- Security: The DTN Bundle Security Protocol (BSP) allows for integrity checks, authentication and encryption, even on links where not previously used.

- Quality of Service: The DTN protocol suite allows for many priority levels to be set for different data types, ensuring that the most important data is received ahead of less important data.

- Improved Operations and Situational Awareness: The DTN store-and-forward mechanism along with automatic retransmission provides more insight into events during communication outages and significantly reduces the need for ground-based scheduling.

Mission of DTN Operation

The mission of DTN is to validate DTN communication protocols by space link testing through KPLO. So DTN operations are focused on testing the DTN protocols using KPLO space links.

DTN Configurations

In the first phase of lunar exploration, test lunar orbiter called Korea Pathfinder Lunar Orbiter (KPLO) will be launched without lunar lander. Figure 2.1 shows DTN configuration in first phase of lunar exploration. There are no Lander and Rover on the moon in first phase. So Lander Communication Mode (LCM) and Rover Communication Mode (RCM) will be located on earth for the DTN communication testing.

DTN Payload (DTNPL) is a DTN node in the space. DTNPL will be mounted on the KPLO for DTN communication. DTN Control Center (DCC) is a control center of DTN communication nodes. LCM and RCM are DTN nodes on ground. All DTN nodes on ground are interconnected by the terrestrial network such as Internet. MCC and DSN are not DTN nodes. They just transfer DTN packets to next communication nodes.

Figure 2.1 DTN test configuration in the first phase

Communication Nodes

Many communication nodes are needed for the DTN operation. Some of communication nodes in the network are DTN nodes whereas otherwise nodes are non-DTN nodes. The function of communication nodes used in the DTN operation is as follows.

DTN nodes

- DCC is a ground DTN node which monitor and controls DTN nodes. DCC send commands to DTN nodes such as DTNPL, LCM and RCM. It also receives telemetries or acquired data from some DTN nodes such as RCM or LCM.

- DTNPL is a DTN node mounted on the KPLO. DTNPL can be operated as a DTN end point. And it also can be operated as relay nodes which relay DTN data between DCC and RCM or LCM.

- LCM is a ground DTN node. LCM relays DTN data between MCC and RCM.

- RCM is a ground DTN node and a DTN end point. The commands sent by the DCC is terminated at the RCM and the RCM returns telemetry or data to DCC through KPLO.

Non-DTN nodes

- MCC is a non-DTN node. MCC monitors and controls KPLO and payloads. MCC also receives and processes data generated from the payloads and transfers to each payload control center.

- DSN is a ground station for deep space communication. DSN will have a big size of antenna and RF equipment for space link interface. DSN also have network equipment for terrestrial communication interface such as SLE link. DSN is a non-DTN node.

Thermal Analysis of DTN Payload

Thermal analysis purposes and model definition

Research on thermal analysis was conducted to determine the operating temperature and design verification of DTN Payload analysis model for it is shown in the following figure.

Figure 2 Thernal Analysis Target Model Configuration

Analysis Conditions

Information Analysis

FEA Modeling

Through the model information in Figure 2, the 3D CAD model was subjected to the mesh generation with the thermal analysis FEA model. Information about the FEA model is as follows.

- FEA Solver: NX Space Systems Thermal V 10.0

- AL Structure (Housing, Cover) Modeling: Using the 3D Tetrahedron Element to Configure the FEA Model

- PCB Modeling: PCB modeling with Shell Element

- EEE parts 모델링

- IC chip uses Hexa element to construct FEA model

- Number of node : 78,898 EA

Figure 3 FEM Model for Target Model

Steady state analysis

- Base plate temperature : 50 ℃

- Consider thermal conduction only.

- The calorific value is calculated as the total calorific value for the entire volume.

- Consider contact thermal resistance and other thermal analysis conditions

Contact heat conduction

- In order to accurately model the actual phenomenon, contact thermal conductivity is applied

- If material properties such as heat conductivity and specific heat of individual parts are not known, uniform value application

- Refer to the reference thermal conductivity coefficient

- The contact thermal conductivity coefficient is very large due to surface roughness, contact pressure,

Material property

The material properties for thermal analysis are shown in the following table..

Table 1  Material property

|  | Density (kg/m3) | Conductivity (W/m•K) | Specific Heat (J/kg•K) |
| --- | --- | --- | --- |
| AI6061-T6 | 2,700 | 156 | 900 |
| PCB | 1,850 | 49 | 840 |
| EEE Parts | 2,330 | 149 | 794 |

Contact condition

The contact conditions of the materials are shown in the following table.

Table 2 Contaco Conditions

|  | Contact Conductance (W/m2•K) | Reference |
| --- | --- | --- |
| AI - AI | 250 | “Heat Transfer”, A. F. Mills (Moderate Pressure, usual finishes) |
| AI – PCB | 250 | “Heat Transfer”, A. F. Mills (Moderate Pressure, usual finishes) |
| EEE – PCB | 5,262 | “Heat Transfer”, A. F. Mills (Moderate Pressure, usual finishes) Space Craft Thermal Control Handbook Vol. 1 |

Result of Analysis

All Body

The thermal analysis results for the entire structure are shown in the following figure.

Figure 4 Temperature Distribution Contour for All Body (Housing)

Figure 5 Temperature Distribution Contour for All Body (Board)

The calorific value information for the PCB is shown in the following figure.

Figure 6 PCB Calorific value information (Top)

Figure 7 PCB Calorific value information (Bottom)

PCB thermal analysis result

Thermal analysis results for the PCB are shown in the following figure.

Figure 8 Thermal analysis results for the PCB (Top)

Figure 9 Thermal analysis results for the PCB(Bottom)

Table 3 PCB Thermal analysis result

TJ_derated applies ECSS-Q-60-11.

Review results

The thermal analysis results show that all components have a margin of at least 2.4 ° C based on the derating temperature and the maximum PCB temperature is around 87.4 ° C through Figure 8. There is no particular limitation on the thermal analysis. However, when the physical properties such as the heat conduction coefficient and specific heat of the individual components are not secured, a uniform value may be applied and there may be a difference from the actual results.