Korea Lunar Exploration Program

DTNPL EGSE Specification

| Date: 29 Oct. 2018 | Doc. No: KPLO-D0-550-300 Issue: 1.0 Total page: 12 |
| --- | --- |

Superseding     Revised

| Prepared By: Min-Woo Park Date DTN SBC S/W Engineer, Lumir |  |  |
| --- | --- | --- |
| Reviewed By: Chang-Soo Lee Date DTN SBC Product Assurance, Lumir |  | Dae-Soo Oh Date DTN SBC Project Manager, Lumir |
| Kyung-Rak Lee Date DTN Payload Engineer, ETRI |  |  |
| Approval Signature: Choel-Oh Jeong Date DTN Payload Product Assurance, ETRI |  | Jin-Ho Jo Date DTN Payload Project Leader, ETRI |

Original CDMO Release__________________

| Revision History |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version | Author(s) |  | Date | Description |
| 0.1 | Jong-Cheon Kim |  | 2017. 12. 28 | Draft |
| 1.0 | Min-Woo Kim |  | 2018. 10. 29 | Final |

| Revision/Change Records |  |  |  |
| --- | --- | --- | --- |
| Version | Date | Revision/Change Description | Pages |
| 0.1 | 2017. 12. 28 | Initial | All |
| 1.0 | 2018. 10. 29 | Final | All |

Documents Tree

Contents

1.	Introduction	7

1.1.	Purpose and Scope	7

1.2.	Applicable Documents	7

1.2.1.	ETRI Documents	7

1.2.2.	Other Documents	7

1.3.	Abbreviations	7

2.	Overview of KPLO DTN	9

2.1.	DTN and Space Communications	9

2.2.	Mission of DTN Operation	9

2.3.	DTN Configurations	9

2.4.	Communication Nodes	10

2.4.1.	DTN nodes	10

2.4.2.	Non-DTN nodes	10

3.	EGSE Requirement Specification	11

3.1.	Functional Requirement	11

3.1.1.	MMI	11

3.1.2.	DTN Protocol	11

3.1.3.	TC Test Mode	11

3.1.4.	TM Test Mode	11

3.1.5.	Logging	11

3.1.6.	De-logging	11

3.2.	Performance Requirements	11

3.2.1.	Data Interface Speed	11

3.2.2.	Memory	11

3.3.	Interface Requirements	11

3.3.1.	DC Power Interface	11

3.3.2.	Operating Voltage	12

3.3.3.	Data Interface	12

3.3.4.	Data Interface Format	12

4.	Verification Matrix	12

Figures

Figure 2.1 DTN test configuration in the first phase	10

Introduction

Purpose and Scope

This document defines the electrical and mechanical interfaces between KPLO bus and DTN payload.

Applicable Documents

ETRI Documents

| Document Number | Document Title |
| --- | --- |
| KPLO-D0-550-000 | System Requirement of DTN Payload |
| KPLO-D0-550-001 | System Specification of DTN Payload |

Other Documents

| Document Number | Document Title |
| --- | --- |
| KPLO-SP-310-001 | KPLO System Specification Document |
| KPLO-SP-340-001 | KPLO Payload/Spacecraft Bus ICD |

Abbreviations

| AMS | Asynchronous Message Service |
| --- | --- |
| BP | Bundle Protocol |
| BSS | Bundle Streaming Service |
| CCSDS | Consultative Committee for Space Data Systems |
| CFDP | CCSDS File Delivery Protocol |
| DCC | DTN Control Center |
| CRC | Cyclical Redundancy Check |
| DMA | Direct Memory Access |
| DTN | Delay(Disruption) Tolerant Network |
| DTNPL | DTN Payload |
| EOD | End of Data |
| EOF | End of File |
| FW | Firm Ware |
| FSW | Flight Software |
| ION | Interplanetary Overlay Network |
| KDSA | Korea Deep Space Antenna |
| KPLO | Korea Pathfinder Lunar Orbiter |
| LCM | Lander Communication Model |
| LTP | Licklider Transmission Protocol |
| MOC | Mission Operation Center |
| OBC | On Board Computer |
| OBT | On Board Time |
| PDHU | Payload Data Handling Unit |
| RCM | Rover Communication Model |
| RFU | Reserved for Future Use |
| SBC | Single Board Computer |
| SOD | Start of Data |
| SOF | Start of File |
| SOH | Start of Head |
| TBC | To Be Confirmed |
| TBD | To Be Defined |
| TC | Tele-command |
| TCP | Transmission Control Protocol |
| TM | Telemetry |
| UART | Universal Asynchronous Receiver Transmitter |
| USRT | Universal Synchronous Receiver Transmitter |

Overview of KPLO DTN

DTN and Space Communications

Delay/disruption tolerant networking (DTN) technology offers a new solution to highly stressed communications in space environments, especially those with long link delay and frequent link disruptions in deep space missions. Delay/disruption tolerant networking (DTN) technology offers a new solution that use a store-and-forward mechanism to combat long link delay and frequent link disruptions generally characterizing space communications. DTN communications use a bundle protocol (BP), to construct a store-and-forward overlay network. The BP, situated at the application layer of the Internet model, utilizes conventional Internet protocols to send and receive data bundles. The benefits of DTN communications are as follows.

- Interoperability and Reuse: A standardized DTN protocol suite enables interoperability of multi-agency communication assets and also allows NASA to use the same communication stack for future missions (LEO, NEO or Deep Space).

- Space Link Efficiency, Utilization and Robustness: DTN enables more reliable and efficient data transmissions resulting in more usable bandwidth. DTN also improves link reliability by having multiple network paths and assets for potential communication hops.

- Security: The DTN Bundle Security Protocol (BSP) allows for integrity checks, authentication and encryption, even on links where not previously used.

- Quality of Service: The DTN protocol suite allows for many priority levels to be set for different data types, ensuring that the most important data is received ahead of less important data.

- Improved Operations and Situational Awareness: The DTN store-and-forward mechanism along with automatic retransmission provides more insight into events during communication outages and significantly reduces the need for ground-based scheduling.

Mission of DTN Operation

The mission of DTN is to validate DTN communication protocols by space link testing through KPLO. So DTN operations are focused on testing the DTN protocols using KPLO space links.

DTN Configurations

In the first phase of lunar exploration, test lunar orbiter called Korea Pathfinder Lunar Orbiter (KPLO) will be launched without lunar lander. Figure 2.1 shows DTN configuration in first phase of lunar exploration. There are no Lander and Rover on the moon in first phase. So Lander Communication Model (LCM) and Rover Communication Model (RCM) will be located on earth for the DTN communication testing.

DTN Payload (DTNPL) is a DTN node in the space. DTNPL will be mounted on the KPLO for DTN communication. DTN Control Center (DCC) is a control center of DTN communication nodes. LCM and RCM are DTN nodes on ground. All DTN nodes on ground are interconnected by the terrestrial network such as Internet. Mission Operation Center (MOC) and Korea Deep Space Antenna (KDSA) are not DTN nodes. They just transfer DTN packets to next communication nodes.

Figure 2.1 DTN test configuration in the first phase

Communication Nodes

Many communication nodes are needed for the DTN operation. Some of communication nodes in the network are DTN nodes whereas otherwise nodes are non-DTN nodes. The function of communication nodes used in the DTN operation is as follows.

DTN nodes

- DCC is a ground DTN node which monitor and controls DTN nodes. DCC send commands to DTN nodes such as DTNPL, LCM and RCM. It also receives telemetries or acquired data from some DTN nodes such as RCM or LCM.

- DTNPL is a DTN node mounted on the KPLO. DTNPL can be operated as a DTN end point. And it also can be operated as relay nodes which relay DTN data between DCC and RCM or LCM.

- LCM is a ground DTN node. LCM relays DTN data between MOC and RCM.

- RCM is a ground DTN node and a DTN end point. The commands sent by the DCC is terminated at the RCM and the RCM returns telemetry or data to DCC through KPLO.

Non-DTN nodes

- MOC is a non-DTN node. MOC monitors and controls KPLO and payloads. MOC also receives and processes data generated from the payloads and transfers to each payload control center.

- KDSA is a ground station for deep space communication. KDSA will have a big size of antenna and RF equipment for space link interface. KDSA also has network equipment for terrestrial communication interface such as SLE link. KDSA is a non-DTN node.

EGSE Requirement Specification

Functional Requirement

MMI

The DTNPL shall provide Man Machine Interface(MMI).

DTN Protocol

The DTNPL EGSE shall support DTN Protocol.

TC Test Mode

The DTNPL EGSE shall provide TC Test Modes which follows DTNPL ICD.

TM Test Mode

The DTNPL EGSE shall receive telemetry from DTNPL and display readable information through MMI

Logging

The DTNPL shall be capable of save logging data which can be raw data.

De-logging

The DTNPL shall be capable of load saved logging data for troubleshooting.

Performance Requirements

Data Interface Speed

The DTNPL EGSE shall have enough speed of data interface for the communication with DTNPL.

Memory

The DTNPL EGSE shall have enough memory to perform its functions.

Interface Requirements

DC Power Interface

The DTNPL shall provide DC power to DTNPL.

Operating Voltage

The DTNPL shall be capable of providing the voltage range 24VDC ~ 32VDC.

Data Interface

The DTNPL EGSE shall have serial data bus interface for the communication with DTNPL.

Data Interface Format

The DTNPL EGSE transaction data shall be compatible with CCSDS DTN standards and RFC specifications.

.

Verification Matrix

The verification matrix is shown below.

| SRS ID | Title | Methods |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T | A | I | D | S |
| 3.1.1 | MMI |  |  |  | O |  |
| 3.1.2 | DTN Protocol | O |  |  |  |  |
| 3.1.3 | TC Test Mode | O |  |  |  |  |
| 3.1.4 | TM Test Mode | O |  |  |  |  |
| 3.1.5 | Logging | O |  |  |  |  |
| 3.1.6 | De-logging | O |  |  |  |  |
| 3.2.1 | Data Interface Speed |  |  |  | O |  |
| 3.2.2 | Memory |  |  |  | O |  |
| 3.3.1 | DC Power Interface |  |  |  | O |  |
| 3.3.2 | Operating Voltage |  |  |  | O |  |
| 3.3.3 | Data Interface | O |  |  |  |  |
| 3.3.4 | Data Interface Format | O |  |  |  |  |