Korea Lunar Exploration Program

DTNPL Flight Model

Vibration Test Procedure

| Date: 23 Apr. 2020 | Doc. No: KPLO-D0-550-422 Issue: F.00 Total page: 33 |
| --- | --- |

Superseding     Revised

| Reviewed By: Jae-Woon Choi Date DTN SBC Product Assurance, Lumir |  | Prepared By: Min-Seock Kim Date DTN SBC Mechanical Engineer, Lumir Dae-Soo Oh Date DTN SBC Project Manager, Lumir |
| --- | --- | --- |
| Approved By: Chol-Oh Jeong Date DTN Payload Product Assurance, ETRI |  | Jin-Ho Jo Date DTN Payload Project Leader, ETRI Byoung-Sun Lee Date DTN Program Manager, ETRI |

Original CDMO Release__________________

| Revision History |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version | Author(s) |  | Date | Description |
| F.00 | Min-Seock Kim |  | 2020. 04. 23 | Initial Released |

| Revision/Change Records |  |  |  |
| --- | --- | --- | --- |
| Version | Date | Revision/Change Description | Pages |
| F.00 | 2020. 04. 23 | Initial | All |

Documents Tree

Contents

1.	Introduction	9

1.1.	Purpose and Scope	9

1.2.	Applicable & Reference Documents	9

1.2.1.	Applicable Documents	9

1.2.1.	Reference Documents	9

1.3.	Abbreviations	9

2.	Test Article	11

2.1.	Verification Cross Reference	12

2.2.	Notification for Acceptance Tests	12

2.3.	Precaution and Warnings	12

3.	. Test Specifications	13

3.1.	Low level Sweep	13

3.2.	Random vibration	13

3.3.	DTNPL Function test item	13

3.4.	Test condition tolerances	14

3.5.	Pass/fail criteria	14

4.	Test Equipment	14

5.	Vibration test configuration	15

5.1.	Sensor location	15

5.2.	Vibration test setup	16

5.2.1.	Vibration Test setup for x Axis	16

5.2.2.	Vibration Test setup for Y Axis	16

5.2.3.	Vibration Test setup for Z Axis	17

6.	Test Environments	18

6.1.	equipment to be tested	18

6.2.	test location	18

6.3.	Test personnel	18

6.4.	Vibration test equipment	19

7.	Test Description	20

7.1.	Vibration test	20

8.	Test Procedure	21

8.1.	X-axis test set up	21

8.1.1.	Low level Sweep for x axis	22

8.1.2.	Random vibration test for x axis	23

8.1.3.	Low level Sweep for x axis	24

8.2.	Y-axis test	25

8.2.1.	Low level Sweep for Y axis	26

8.2.2.	Random vibration test for Y axis	27

8.2.3.	Low level Sweep for Y axis	28

8.3.	Z axis test	29

8.3.1.	Low level Sweep for z axis	30

8.3.2.	Random vibration test for z axis	31

8.3.3.	Low level Sweep for z axis	32

Test conclusions	33

Figures

Figure 1 DTNPL Configuration	11

Figure 2 Location for Sensors for Test	15

Figure 3 Test Configuration for X Axis	16

Figure 4 Test Configuration for Y Axis	16

Figure 5 Test Configuration for Z Axis	17

Figure 6 Vibration Test Flow for DTNPL	20

Tables

Table 1 Dimension Criteria	11

Table 2 Random Vibration Level (Lateral Axis, X-Y)	13

Table 3 Random Vibration Level (Longitudinal, Z)	13

Table 4 Test Condition Tolerances	14

Table 5 Pass/Fail Criteria	14

Table 6 Test Equipment	14

Table 7 Sensor	15

Table 8 Test R&R	19

Table 9 Vibration Test Equipment List	19

Table 10 X-axis Test Setup	21

Table 11 Low level Sweep for X-axis	22

Table 12 Random Vibration Test for X-axis	23

Table 13 Low level Sweep for X-axis	24

Table 14 Y-axis Test	25

Table 15 Low level Sweep for Y-axis	26

Table 16 Random Vibration Test for Y-axis	27

Table 17 Low level Sweep for Y-axis	28

Table 18 Z-axis Test	29

Table 19 Low level Sweep for Z-axis	30

Table 20 Random Vibration Test for Z-axis	31

Table 21 Low level Sweep for Z-axis	32

Introduction

Purpose and Scope

This document establishes the conditions, requirements, and test procedures for acceptance level vibration testing of the DTNPL payload. FM.

The purpose of this is to demonstrate that the DTN payload is working as expected under the required environmental conditions. Before and After Vibration Test a representative collection of functionality and performance test will be performed to check interfaces and hardware related functions.

Applicable & Reference Documents

Applicable Documents

|  | Document Number | Document Title |
| --- | --- | --- |
| AD01 | KPLO-D0-550-000 | System Requirement of DTN Payload |
| AD02 | KPLO-D0-550-001 | System Specification of DTN Payload |
| AD03 | KPLO-SP-320-002 | KPLO Environmental Design and Test Specification |
| AD04 | KPLO-SP-330-001 | KPLO EMC Requirements Specification |

    - Reference Documents

|  | Document Number | Document Title |
| --- | --- | --- |
| RD01 | MIL-STD-1540C | Test Requirements for Launch, Upper-Stage, and Space Vehicles |
| RD02 | PSS-01-801 | Test Requirements Specification for Space Equipment |
| RD03 | ECSS-E-10-03-A | ECSS Space Engineering-Testing |

  - Abbreviations

| AMS | Asynchronous Message Service |
| --- | --- |
| BP | Bundle Protocol |
| BSS | Bundle Streaming Service |
| CCSDS | Consultative Committee for Space Data Systems |
| CFDP | CCSDS File Delivery Protocol |
| DCC | DTN Control Center |
| CRC | Cyclical Redundancy Check |
| DMA | Direct Memory Access |
| DTN | Delay(Disruption) Tolerant Network |
| DTNPL | DTN Payload |
| EOD | End of Data |
| EOF | End of File |
| FW | Firm Ware |
| FSW | Flight Software |
| ION | Interplanetary Overlay Network |
| CAS500-1 | Korea Pathfinder Lunar Orbiter |
| LCM | Lander Communication Model |
| LTP | Licklider Transmission Protocol |
| OBC | On Board Computer |
| PDHU | Payload Data Handling Unit |
| RCM | Rover Communication Model |
| DTNPL | Single Board Computer |
| SOD | Start of Data |
| SOF | Start of File |
| SOH | Start of Head |
| TBC | To Be Confirmed |
| TBD | To Be Defined |
| TC | Tele-command |
| TCP | Transmission Control Protocol |
| TM | Telemetry |
| UART | Universal Asynchronous Receiver Transmitter |

Test Article

DTNPL is composed by major functional modules as follows:

  - SBC (Single Board Computer)

Figure 1 shows the DTNPL FM external view and its coordinate system. Size and mass of test article is given in Table 1.

The coordinate system is defined as follows.

- The plane defined by the unit coordinate system axes X and Y is parallel to the mounting surface.

- The unit Z axis is normal to the mounting plane.

- The X axis with the dimension along X (length) being greater than the dimension along Y(width).

L

W

H

Figure 1 DTNPL Configuration

Table 1 Dimension Criteria

| Description | Value |
| --- | --- |
| Length measurement | 170 mm |
| Width measurement | 120 mm |
| Height measurement | 39 mm |
| Weighing | 0.98 kg |

Verification Cross Reference

The requirements which have to be verified by a successful performance in the test and its relevant test steps are given below:

| Applicable Doc. | Requirement | Verified by Test Step | Compliance |
| --- | --- | --- | --- |
| KPLO-SP-320-002 | Vibration | All | C |

Notification for Acceptance Tests

This test procedure shall be approved by the KARI prior to performance of the test, either by signature on cover page of by performance of a dedicated Test Readiness Review (TRR).

Precaution and Warnings

For all testing and handling, ESD requirements concerning personnel, facilities and test equipment shall be regarded.

No special hazard exists during the test.

The following precautions apply:

- Only trained personnel are allowed to handle the test specimen.

- A test cable or breakout box shall never be connected of disconnected when the test specimen is powered.

- Correct set-up of the test equipment has to be checked carefully prior to the connection/switch-on of flight hardware, e.g. bonding, grounding, voltages, current limiters, etc.

- The test specimen shell be protected by a clean, antistatic bag all the time no activities are performed at the hardware.

. Test Specifications

Low level Sweep

| Frequency(Hz) | Acceleration(g’s) | ETC |
| --- | --- | --- |
| 5 - 2000 | 1 | - 2 octave per minute |

Random vibration

Table 2 and Table 3 are the acceptance random vibration specifications for the DTNPL in all three axes.

Table 2 Random Vibration Level (Lateral Axis, X-Y)

| Frequency(Hz) | Acceleration(g’s) | ETC |
| --- | --- | --- |
| 20 | 0.013 | - For testing: 1 minute per axis |
| 50 | 0.08 |  |
| 800 | 0.08 |  |
| 2000 | 0.013 |  |
| Overall | 10.00 grms |  |

Table 3 Random Vibration Level (Longitudinal, Z)

| Frequency(Hz) | Acceleration(g’s) | ETC |
| --- | --- | --- |
| 10 - 60 | +9 dB/oct | - For testing: 1 minute per axis |
| 60 - 400 | 0.22 |  |
| 400 - 2000 | -6 dB/oct |  |
| Overall | 12.3 grms |  |

DTNPL Function test item

Perform the additional STP (Short Functional Test) Before and After the Acceptance Vibration Test.

- Initialization

- Operation Test

Test condition tolerances

Table 4 Test Condition Tolerances

| Items | Tolerances |
| --- | --- |
| Random Vibration Accel. Spectral Density` RMS Level | ±3 dB +10% |

Pass/fail criteria

The pass/fail for the Vibration test is verified by the detailed visual check, function test. The pass/fail criteria are shown in the Table 5.

Table 5 Pass/Fail Criteria

| Item | Requirement |
| --- | --- |
| Visual | No anomaly |
| Function | No anomaly |

Test Equipment

The vibration excitation system and data acquisition system in DT&C will be used for the vibration test.

The specifications of the shaker system and data measurement system are as shown in Table 6.

Table 6 Test Equipment

| Item | Manufacturer | Description |
| --- | --- | --- |
| Vibration Accelerometer |  |  |
| Test Fixture | Lumir |  |
| Sensor | - |  |

Vibration test configuration

Sensor location

Location and information of the sensors are indicated in Figure 5-1. Normally two sensors will be attached beside mounting points and the maximum value is going to be used as criteria to know the responsive. The test sensor descriptions will be recorded on the Test Report.

Figure 2 Location for Sensors for Test

Table 7 Sensor

| No. | Position | Sensor S/N | Coordinate |  | Remark |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Sensor | Equip |  |
| 1 | C1 |  |  |  | Control |
| 2 | D1 |  |  |  | Measure |

Vibration test setup

Vibration Test setup for x Axis

Figure 3 Test Configuration for X Axis

Vibration Test setup for Y Axis

Figure 4 Test Configuration for Y Axis

Vibration Test setup for Z Axis

Figure 5 Test Configuration for Z Axis

Test Environments

The following test environment conditions shall be recorded prior to measurement.

equipment to be tested

Model Name:  DTNPL SBC FM

test location

DT&C

Test personnel

The personnel required for the performance of the tests is listed below:

Test Engineers

The test engineers are responsible for:

- Operation of measuring instrumentation, data acquisition, data handling, data recording

- Identification and compilation of data sheet

- Safety precaution for test articles

- Test personnel to be present in the test area

- The accuracy of the measured values & Data report

Test Manager

The test manager is responsible for:

- Test articles and technical aspects of the test performance

- Record the correct software version for test article and EGSE

- Examination of disturbances

- Test article integration and handling

- Test performance

- Evaluation of test results & Providing of test reports

Quality Assurance Engineer

The QA engineer is responsible for:

- Surveillance of test equipment according to regulations as well as the test procedure

- Statement that the test articles to be tested have passed all checks before testing

- Checking the identification markings on the test articles

- Supervision of the proceeding with respect to quality assurance performance

Table 8 Test R&R

| Responsibility | Company /Dep. | Name |
| --- | --- | --- |
| Test Engineer | Lumir / R&D | Kim, Sun-Ku |
| Test Manager | Lumir / R&D | Oh, Dae-Soo |
| Quality Assurance Engineer | Lumir / PA | Cho, Sung-Woon |
| Facility Engineer | DT&C |  |

Vibration test equipment

Table 9 Vibration Test Equipment List

| Equipment | Model No. | Manufacturer |
| --- | --- | --- |

Test Description

Vibration test

The test shall be performed in accordance with the following sequences and the test results are recorded per the indicated method.

Figure 6 Vibration Test Flow for DTNPL

Test Procedure

The test will be performed in accordance with Figure 3, 4 and 5.

X-axis test set up

| (a) | For the X axis evaluation, check and install the shaker. | (√) |
| --- | --- | --- |
| (b) | Assemble the test fixture and sensors on the shaker. | (√) |
| (c) | Perform the fixture evaluation per the test specification that is indicated in Table 3. | (√) |

Table 10 X-axis Test Setup

| Start Date / Time |  |
| --- | --- |
| End Date / Time |  |
| Temperature ℃ (16 to 32℃) |  |
| Relative Humidity % (20 to 70%) |  |

| (d) | Generate plot for the verification compliance with the spectrum level and tolerance for each channel, and verify the results. | (√) |
| --- | --- | --- |

Low level Sweep for x axis

| (a) | Check equipment mounting and sensor status before the sine test for X axis. | (√) |
| --- | --- | --- |
| (b) | Input the test condition data in the controller of the shaker and verify. | (√) |
| (c) | Perform the fixture evaluation per the test specification that is indicated in Table 3. | (√) |

Table 11 Low level Sweep for X-axis

| Start Date / Time |  |
| --- | --- |
| End Date / Time |  |
| Temperature ℃ (16 to 32℃) |  |
| Relative Humidity % (20 to 70%) |  |

| (d) | Generate plot for the verification compliance with the spectrum level and tolerance for each channel, and verify the results. | (√) |
| --- | --- | --- |

Random vibration test for x axis

| (a) | Check equipment mounting and sensor status before the random test for X axis. | (√) |
| --- | --- | --- |
| (b) | Input the test condition data in the controller of the shaker and verify. | (√) |
| (c) | Perform the fixture evaluation per the test specification that is indicated in Table 3. | (√) |

Table 12 Random Vibration Test for X-axis

| Start Date / Time |  |
| --- | --- |
| End Date / Time |  |
| Temperature ℃ (16 to 32℃) |  |
| Relative Humidity % (20 to 70%) |  |

| (d) | Generate plot for the verification compliance with the spectrum level and tolerance for each channel, and verify the results. | (√) |
| --- | --- | --- |

| (e) | After the random test is finished, perform the visual inspection of the equipment. | (√) |
| --- | --- | --- |

Low level Sweep for x axis

| (a) | Check equipment mounting and sensor status before the sine test for X axis. | (√) |
| --- | --- | --- |
| (b) | Input the test condition data in the controller of the shaker and verify. | (√) |
| (c) | Perform the fixture evaluation per the test specification that is indicated in Table 3. | (√) |

Table 13 Low level Sweep for X-axis

| Start Date / Time |  |
| --- | --- |
| End Date / Time |  |
| Temperature ℃ (16 to 32℃) |  |
| Relative Humidity % (20 to 70%) |  |

| (d) | Generate plot for the verification compliance with the spectrum level and tolerance for each channel, and verify the results. | (√) |
| --- | --- | --- |

Y-axis test

| (a) | For the Y axis evaluation, check and install the shaker. | (√) |
| --- | --- | --- |
| (b) | Assemble the test fixture and sensors on the shaker. | (√) |
| (c) | Perform the fixture evaluation per the test specification that is indicated in Table 4. | (√) |

Table 14 Y-axis Test

| Start Date / Time |  |
| --- | --- |
| End Date / Time |  |
| Temperature ℃ (16 to 32℃) |  |
| Relative Humidity % (20 to 70%) |  |

| (d) | Generate plot for the verification compliance with the spectrum level and tolerance for each channel, and verify the results. | (√) |
| --- | --- | --- |

Low level Sweep for Y axis

| (a) | Check equipment mounting and sensor status before the sine test for X axis. | (√) |
| --- | --- | --- |
| (b) | Input the test condition data in the controller of the shaker and verify. | (√) |
| (c) | Perform the fixture evaluation per the test specification that is indicated in Table 4. | (√) |

Table 15 Low level Sweep for Y-axis

| Start Date / Time |  |
| --- | --- |
| End Date / Time |  |
| Temperature ℃ (16 to 32℃) |  |
| Relative Humidity % (20 to 70%) |  |

| (d) | Generate plot for the verification compliance with the spectrum level and tolerance for each channel, and verify the results. | (√) |
| --- | --- | --- |

Random vibration test for Y axis

| (a) | Check equipment mounting and sensor status before the random test for Y axis. | (√) |
| --- | --- | --- |
| (b) | Input the test condition data in the controller of the shaker and verify. | (√) |
| (c) | Perform the fixture evaluation per the test specification that is indicated in Table 4. | (√) |

Table 16 Random Vibration Test for Y-axis

| Start Date / Time |  |
| --- | --- |
| End Date / Time |  |
| Temperature ℃ (16 to 32℃) |  |
| Relative Humidity % (20 to 70%) |  |

| (d) | Generate plot for the verification compliance with the spectrum level and tolerance for each channel, and verify the results. | (√) |
| --- | --- | --- |

| (e) | After the random test is finished, perform the visual inspection of the equipment. | (√) |
| --- | --- | --- |

Low level Sweep for Y axis

| (a) | Check equipment mounting and sensor status before the sine test for X axis. | (√) |
| --- | --- | --- |
| (b) | Input the test condition data in the controller of the shaker and verify. | (√) |
| (c) | Perform the fixture evaluation per the test specification that is indicated in Table 4. | (√) |

Table 17 Low level Sweep for Y-axis

| Start Date / Time |  |
| --- | --- |
| End Date / Time |  |
| Temperature ℃ (16 to 32℃) |  |
| Relative Humidity % (20 to 70%) |  |

| (d) | Generate plot for the verification compliance with the spectrum level and tolerance for each channel, and verify the results. | (√) |
| --- | --- | --- |

Z axis test

| (a) | For the Z axis evaluation, check and install the shaker. | (√) |
| --- | --- | --- |
| (b) | Assemble the test fixture and sensors on the shaker. | (√) |
| (c) | Perform the fixture evaluation per the test specification that is indicated in Table 5. | (√) |

Table 18 Z-axis Test

| Start Date / Time |  |
| --- | --- |
| End Date / Time |  |
| Temperature ℃ (16 to 32℃) |  |
| Relative Humidity % (20 to 70%) |  |

| (d) | Generate plot for the verification compliance with the spectrum level and tolerance for each channel, and verify the results. | (√) |
| --- | --- | --- |

Low level Sweep for z axis

| (a) | Check equipment mounting and sensor status before the sine test for Z axis. | (√) |
| --- | --- | --- |
| (b) | Input the test condition data in the controller of the shaker and verify. | (√) |
| (c) | Perform the fixture evaluation per the test specification that is indicated in Table 5. | (√) |

Table 19 Low level Sweep for Z-axis

| Start Date / Time |  |
| --- | --- |
| End Date / Time |  |
| Temperature ℃ (16 to 32℃) |  |
| Relative Humidity % (20 to 70%) |  |

| (d) | Generate plot for the verification compliance with the spectrum level and tolerance for each channel, and verify the results. | (√) |
| --- | --- | --- |

Random vibration test for z axis

| (a) | Check equipment mounting and sensor status before the random test for Z axis. | (√) |
| --- | --- | --- |
| (b) | Input the test condition data in the controller of the shaker and verify. | (√) |
| (c) | Perform the fixture evaluation per the test specification that is indicated in Table 5. | (√) |

Table 20 Random Vibration Test for Z-axis

| Start Date / Time |  |
| --- | --- |
| End Date / Time |  |
| Temperature ℃ (16 to 32℃) |  |
| Relative Humidity % (20 to 70%) |  |

| (d) | Generate plot for the verification compliance with the spectrum level and tolerance for each channel, and verify the results. | (√) |
| --- | --- | --- |

| (e) | After the random test is finished, perform the visual inspection of the equipment. | (√) |
| --- | --- | --- |

Low level Sweep for z axis

| (a) | Check equipment mounting and sensor status before the sine test for Z axis. | (√) |
| --- | --- | --- |
| (b) | Input the test condition data in the controller of the shaker and verify. | (√) |
| (c) | Perform the fixture evaluation per the test specification that is indicated in Table 5. | (√) |

Table 21 Low level Sweep for Z-axis

| Start Date / Time |  |
| --- | --- |
| End Date / Time |  |
| Temperature ℃ (16 to 32℃) |  |
| Relative Humidity % (20 to 70%) |  |

| (d) | Generate plot for the verification compliance with the spectrum level and tolerance for each channel, and verify the results. | (√) |
| --- | --- | --- |

- Test conclusions

The PA shall review the test data for acceptance of the Vibration test.

The following sign off is verification that the test article has satisfactorily passed Vibration test.

PA:                            Date: