Korea Lunar Exploration Program

DTNPL Qualification Model

Shock Test Procedure

| Date: 11 Mar. 2019 | Doc. No: KPLO-D0-550-470 Issue: D-01 Total page: 23 |
| --- | --- |

Superseding     Revised

| Reviewed By: Chang-Soo Lee Date DTN Product Assurance, Lumir Kyung-Rak Lee Date DTN System Engineer, ETRI |  | Prepared By: Jeong-Chan Lee Date DTN Engineer, Lumir Dae-Soo Oh Date DTN Project Manager, Lumir |
| --- | --- | --- |
| Reviewed By: Chol-Oh Jeong Date DTN Product Assurance, ETRI |  | Jin-Ho Jo Date DTN Project Leader, ETRI |

Original CDMO Release__________________

| Revision History |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version | Author(s) |  | Date | Description |
| D-01 | Jin-Ho Jo |  | 2017. 11. 07 | Draft |
| 1.0 | Jeong-Chan Lee |  | 2019. 03.28 | QM Test Spec Updated |

Documents Tree

Contents

1.	Introduction	8

1.1.	Purpose and Scope	8

1.2.	Applicable & Reference Documents	8

1.2.1.	Applicable Documents	8

1.2.1.	Reference Documents	8

1.3.	Abbreviations	8

2.	Test Article	10

2.1.	Verification Cross Reference	11

2.2.	Notification for Qualification and Acceptance Tests	11

2.3.	Precaution and Warnings	11

3.	. Test Specifications	12

3.1.	Shock	12

3.2.	DTNPL Function test item	13

3.3.	Test condition tolerances	13

3.4.	Pass/fail criteria	13

4.	Test Equipment	14

5.	Shock test configuration	15

5.1.	Sensor location	15

5.2.	Shock test setup	16

5.2.1.	Shock Test setup for x Axis	16

5.2.2.	Shock Test setup for Y Axis	16

5.2.3.	Shock Test setup for Z Axis	17

6.	Test Environments	18

6.1.	equipment to be tested	18

6.2.	test location	18

6.3.	Test personnel	18

7.	Test Description	19

7.1.	Shock test	19

8.	Test Procedure	20

8.1.	Shock Test For X-AXIS	20

Figures

Figure 1 DTNPL Configurations	10

Figure 2 Location for Sensors for Test	15

Figure 3 Test Configuration for X Axis	16

Figure 4 Test Configuration for Y Axis	16

Figure 5 Test Configuration for Z Axis	17

Figure 6 Shock Test Flow for DTNPL	19

Tables

Table 1 Dimension Criteria	10

Table 2 Shock Level	12

Table 3 Shock Profile(X-Y Axis)	12

Table 4 Shock Profile(Z Axis)	13

Table 5 Test Condition Tolerances	13

Table 6 Pass/Fail Criteria	13

Table 7 Shock Test Equipment List	14

Table 8 Location for Sensors for Test	15

Introduction

Purpose and Scope

This document establishes the conditions, requirements and test procedures for Shock testing of the DTNPL payload.

Applicable & Reference Documents

Applicable Documents

|  | Document Number | Document Title |
| --- | --- | --- |
| AD-01 | C1-SP-320-003 | Components Environmental Design and Test Specification |
| AD-02 | KPLO-SP-320-002 | KPLO Environmental Design and Test Specification |

    - Reference Documents

|  | Document Number | Document Title |
| --- | --- | --- |
| RD-01 | MIL-STD-1540C | Test Requirements for Space Launch, Upper-Stage, and Space Vehicle |
| RD-02 | ECSS-E-10-0 | Space Engineering, Testing |

  - Abbreviations

| AMS | Asynchronous Message Service |
| --- | --- |
| BP | Bundle Protocol |
| BSS | Bundle Streaming Service |
| CCSDS | Consultative Committee for Space Data Systems |
| CFDP | CCSDS File Delivery Protocol |
| DCC | DTN Control Center |
| CRC | Cyclical Redundancy Check |
| DMA | Direct Memory Access |
| DTN | Delay(Disruption) Tolerant Network |
| DTNPL | DTN Payload |
| EOD | End of Data |
| EOF | End of File |
| FW | Firm Ware |
| FSW | Flight Software |
| ION | Interplanetary Overlay Network |
| CAS500-1 | Korea Pathfinder Lunar Orbiter |
| LCM | Lander Communication Model |
| LTP | Licklider Transmission Protocol |
| OBC | On Board Computer |
| PDHU | Payload Data Handling Unit |
| RCM | Rover Communication Model |
| DTNPL | Single Board Computer |
| SOD | Start of Data |
| SOF | Start of File |
| SOH | Start of Head |
| TBC | To Be Confirmed |
| TBD | To Be Defined |
| TC | Tele-command |
| TCP | Transmission Control Protocol |
| TM | Telemetry |
| UART | Universal Asynchronous Receiver Transmitter |

Test Article

DTNPL is composed by major functional modules as follows :

  - SBC (Single Board Computer)

Figure 2-1 shows the DTNPL QM external view and its coordinate system. Size and mass of test article is given in Table 2-1.

The coordinate system is defined as follows.

- The plane defined by the unit coordinate system axes X and Y is parallel to the mounting surface.

- The unit Z axis is normal to the mounting plane.

- The X axis with the dimension along X (length) being greater than the dimension along Y(width).

L

W

H

Figure 1 DTNPL Configurations

Table 1 Dimension Criteria

| Description | Value |
| --- | --- |
| Length measurement | 170 mm |
| Width measurement | 120 mm |
| Height measurement | 39 mm |
| Weighing | 0.98 kg |

Verification Cross Reference

The requirements which have to be verified by a successful performance in the test and its relevant test steps are given below:

| Applicable Doc. | Requirement | Verified by Test Step | Compliance |
| --- | --- | --- | --- |
| KPLO-SP-320-002 | Shock | All | C |

Notification for Qualification and Acceptance Tests

This test procedure shall be approved by the KARI prior to performance of the test, either by signature on cover page of by performance of a dedicated Test Readiness Review(TRR).

Precaution and Warnings

For all testing and handling, ESD requirements concerning personnel, facilities and test equipment shall be regarded.

No special hazard exists during the test.

The following precautions apply:

- Only trained personnel are allowed to handle the test specimen.

- A test cable or breakout box shall never be connected of disconnected when the test specimen is powered.

- Correct set-up of the test equipment has to be checked carefully prior to the connection/switch-on of flight hardware, e.g. bonding, grounding, voltages, current limiters, etc.

- The test specimen shell be protected by a clean, antistatic bag all the time no activities are performed at the hardware.

. Test Specifications

Shock

Table 3-1 is the qualification sinusoidal Shock specifications for the DTNPL in all three axes.

Table 2 Shock Level

|  | In plane(X-Y) Axis |  | Out of plane (Z Axis) |  |
| --- | --- | --- | --- | --- |
| Description | Freq. (Hz) | SRS (g) | Freq. (Hz) | SRS (g) |
| +/- Y, +/- Z Upper Closure Wall (c) | 100 | 10 | 100 | 10 |
|  | 500 | 40 | 500 | 100 |
|  | 1300 | 100 | 1000 | 300 |
|  | 2000 | 700 | 3000 | 700 |
|  | 100000 | 700 | 1000 | 700 |

Table 3 Shock Profile(X-Y Axis)

Table 4 Shock Profile(Z Axis)

DTNPL Function test item

- Initialization

- Operation Test

Test condition tolerances

Table 5 Test Condition Tolerances

| Items | Tolerances |
| --- | --- |
| Response Spectrum Time History | +100%, -50% ±10% |

Pass/fail criteria

The pass/fail for the Shock test is verified by the detailed visual check, function test. The pass/fail criteria are shown in the Table 3-4.

Table 6 Pass/Fail Criteria

| Item | Requirement |
| --- | --- |
| Function | No anomaly |

Test Equipment

The Shock excitation system and data acquisition system in KARI will be used for the Shock test.

The specifications of the shaker system and data measurement system are as shown in Table 4-1.

Table 7 Shock Test Equipment List

| Name | Model No. | Manufacturer |
| --- | --- | --- |
| Shock Tester | TBD | KARI |
| Accelerometer | TBD | TBD |
| Test Fixture Z Axis | DTNE-MD-164003A | Lumir |
| Test Fixture X-Y Axis | DTNE-MD-164004A | Lumir |

Shock test configuration

Sensor location

Location and information of the sensors are indicated in Figure 5-1. Normally two sensors will be attached beside mounting points and the maximum value is going to be used as criteria to know the responsive. The test sensor descriptions will be recorded on the Test Report.

Figure 2 Location for Sensors for Test

Table 8 Location for Sensors for Test

| No. | Position | Sensor S/N | Coordinate |  | Remark |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Sensor | Equip |  |
| 1 | C1(TBD) |  |  |  | Control |
| 2 | D1(TBD) |  |  |  | Measure |
| 3 | D2(TBD) |  |  |  | Measure |
| 4 | D3(TBD) |  |  |  | Measure |

Shock test setup

Shock Test setup for x Axis

Figure 3 Test Configuration for X Axis

Shock Test setup for Y Axis

Figure 4 Test Configuration for Y Axis

Shock Test setup for Z Axis

Figure 5 Test Configuration for Z Axis

Test Environments

The following test environment conditions shall be recorded prior to measurement.

equipment to be tested

Model Name:

test location

Test personnel

The personnel required for the performance of the tests is listed below:

Test Engineers

The test engineers are responsible for:

- Operation of measuring instrumentation, data acquisition, data handling, data recording

- Identification and compilation of data sheet

- Safety precaution for test articles

- Test personnel to be present in the test area

- The accuracy of the measured values & Data report

Test Manager

The test manager is responsible for:

- Test articles and technical aspects of the test performance

- Record the correct software version for test article and EGSE

- Examination of disturbances

- Test article integration and handling

- Test performance

- Evaluation of test results & Providing of test reports

Quality Assurance Engineer

The QA engineer is responsible for:

- Surveillance of test equipment according to regulations as well as the test procedure

- Statement that the test articles to be tested have passed all checks before testing

- Checking the identification markings on the test articles

- Supervision of the proceeding with respect to quality assurance performance

The customer has the fight to participate or observe the test

| Responsibility | Company /Dep. | Name |
| --- | --- | --- |
| Test Engineer | Lumir / R&D |  |
| Test Manager | Lumir / R&D |  |
| Quality Assurance Engineer | Lumir / PA |  |
| Facility Engineer | KARI |  |

Test Description

Shock test

The test shall be performed in accordance with the following sequences and the test results are recorded per the indicated method.

Figure 6 Shock Test Flow for DTNPL

Test Procedure

The test will be performed in accordance with Figure 2 and Table 6.

Shock Test For X-AXIS

| (a) | For the X-axis shock test, check and install the test equipment. | (√) |
| --- | --- | --- |
| (b) | Assemble the test fixture and sensors. (Refer to Figure Figure 2 and Table 8.for the sensor mounting location.) | (√) |
| (c) | Assemble the dummy model on the test fixture. (Refer to Figure 3 for the mounting method and location.) | (√) |
| (d) | Perform the main shock test and calibrate the input data. (It shall be performed at least three times to assure the input data.) | (√) |
| (e) | Disassemble the dummy model and assemble the equipment on the test fixture and inspect the test configuration. (Refer to Figure 3 for the mounting method and location.) | (√) |
| (f) | Perform the main shock test in accordance with Table 2 (3 times) | (√) |

| Start Date / Time |  |
| --- | --- |
| End Date / Time |  |
| Temperature ℃ (23±5℃) |  |
| Relative Humidity % (60% maximum) |  |

| (g) | Generate shock response spectra for verification compliance with the spectrum level and tolerance for each channel and verify the results. (In order to have a successful shock test, more than 50% of the spectrum shall be higher than the nominal shock profile.) | (√) |
| --- | --- | --- |
| (h) | After the shock test is finished, perform the visual inspection of the equipment. (If there is anything that does not match with the pass/fail criteria specified, the test shall be aborted.) | (√) |
| (i) | Test Results | (√) |
| (j) | Remove the equipment from the test fixture | (√) |

Shock Test For Y-AXIS

| (a) | For the X-axis shock test, check and install the test equipment. | (√) |
| --- | --- | --- |
| (b) | Assemble the test fixture and sensors. (Refer to Figure Figure 2 and Table 8.for the sensor mounting location.) | (√) |
| (c) | Assemble the dummy model on the test fixture. (Refer to Figure 3 for the mounting method and location.) | (√) |
| (d) | Perform the main shock test and calibrate the input data. (It shall be performed at least three times to assure the input data.) | (√) |
| (e) | Disassemble the dummy model and assemble the equipment on the test fixture and inspect the test configuration. (Refer to Figure 4 for the mounting method and location.) | (√) |
| (f) | Perform the main shock test in accordance with Table 2 (3 times) | (√) |

| Start Date / Time |  |
| --- | --- |
| End Date / Time |  |
| Temperature ℃ (23±5℃) |  |
| Relative Humidity % (60% maximum) |  |

| (g) | Generate shock response spectra for verification compliance with the spectrum level and tolerance for each channel and verify the results. (In order to have a successful shock test, more than 50% of the spectrum shall be higher than the nominal shock profile.) | (√) |
| --- | --- | --- |
| (h) | After the shock test is finished, perform the visual inspection of the equipment. (If there is anything that does not match with the pass/fail criteria specified, the test shall be aborted.) | (√) |
| (i) | Test Results | (√) |
| (j) | Remove the equipment from the test fixture | (√) |

Shock Test For Z-AXIS

| (a) | For the X-axis shock test, check and install the test equipment. | (√) |
| --- | --- | --- |
| (b) | Assemble the test fixture and sensors. (Refer to Figure Figure 2 and Table 8.for the sensor mounting location.) | (√) |
| (c) | Assemble the dummy model on the test fixture. (Refer to Figure 3 for the mounting method and location.) | (√) |
| (d) | Perform the main shock test and calibrate the input data. (It shall be performed at least three times to assure the input data.) | (√) |
| (e) | Disassemble the dummy model and assemble the equipment on the test fixture and inspect the test configuration. (Refer to Figure 6 for the mounting method and location.) | (√) |
| (f) | Perform the main shock test in accordance with Table 2 (3 times) | (√) |

| Start Date / Time |  |
| --- | --- |
| End Date / Time |  |
| Temperature ℃ (23±5℃) |  |
| Relative Humidity % (60% maximum) |  |

| g) | Generate shock response spectra for verification compliance with the spectrum level and tolerance for each channel and verify the results. (In order to have a successful shock test, more than 50% of the spectrum shall be higher than the nominal shock profile.) | (√) |
| --- | --- | --- |
| (h) | After the shock test is finished, perform the visual inspection of the equipment. (If there is anything that does not match with the pass/fail criteria specified, the test shall be aborted.) | (√) |
| (i) | Test Results | (√) |
| (j) | Remove the equipment from the test fixture | (√) |

- 9. Test conclusions

The PA shall review the test data for acceptance of the Shock test.

The following sign off is verification that the test article has satisfactorily passed Shock test.

PA:                            Date: