| Korea Aerospace Research Institute Lunar Exploration Program Office 169-84 Gwahak-ro, Yuseong-gu, Daejeon 34133 Republic of Korea | ETRI DTN Program Office 218 Gajeongno, Yuseong-gu, Daejeon 34129 Republic of Korea |  |  |
| --- | --- | --- | --- |
|  | Lumir Inc. DTN Development Office A-1102,767 Sinsu-ro, Yongin, Gyunggi, KOREA |  |  |

| Korea Lunar Exploration Program DTN Reliability Prediction for SBC |  |
| --- | --- |
| Date: 11 Mar. 2020 | Doc. No: KPLO-D1-554-028 |
|  | Issue: F1.1 |
|  | Total page: 12 |

Superseding      F1.0

|  |  | Prepared By: |
| --- | --- | --- |
|  |  | Seong-Woon Cho DATE |
|  |  | DTN SBC Product Assurance |
| Reviewed By: |  |  |
| Dae-Soo Oh DATE |  |  |
| DTN SBC Project Manager |  |  |
| Approval Signature: |  |  |
| Cheol-Oh Jeong DATE |  | Jin-Ho Jo DATE |
| ETRI, DTN Product Assurance |  | ETRI, DTN System Engineer |
| Byoung-Sun Lee DATE |  |  |
| ETRI, DTN Program Manager |  |  |

Original CDMO Release ____________________

REVISION / CHANGE RECORD

| Revision | DOCUMENT DATE | REVISION / CHANGE DESCRIPTION | PAGES AFFECTED |
| --- | --- | --- | --- |
| D00 | 2016. 10. 07 | Draft | All |
| P00 | 2016. 11. 23 | Reliability Prediction Results Updated | 13 |
| P01 | 2017. 11. 27 | Doc Number & Form Change | All |
| F1.0 | 2018. 10. 15 | Reliability Prediction Results & Data Updated | 13 |
| F1.1 | 2020.03.11 | Update Reliability Prediction Result Add Reliability Block Diagram Add Appendix | 5 - 12 |

Table of Contents

1.	INTRODUCTION	４

1.1	SCOPE	４

1.2	DTN OVERVIEW	４

2.	DOCUMENTS	４

2.1	APPLICABLE DOCUMENTS	４

2.2	REFERENCE DOCUMENTS	４

3.	ACRONYMS AND ABBREVIATIONS	５

4.	Reliability Prediction	６

4.1	RELIABILITY MODELING	６

4.1.1	ASSUMPTIONS	６

4.1.2	ANALYSIS CONDITION	６

4.1.3	CALCULATIONS OF FAILURE RATES FOR PARTS	７

4.1.4	CALCULATIONS OF FAILURE RATES FOR SYSTEM	８

4.1.5	RELIABILITY BLOCK DIAGRAM	８

4.2	RELIABILITY ANALYSIS RESULT	９

5.	Appendix	１０

5.1	RELIABILITY PREDICTION BY MISSION TIME	１０

5.2	RELIABILITY PREDICTION BY EEE PARTS	１２

Figures

Figure 4-1 Configuration	６

Figure 4-2 Reliability block diagram	８

Figure 4-3 Reliability prediction result	９

Figure 5-1 Reliability prediction result by mission time 1 month	１０

Figure 5-2 Reliability prediction result by mission time 2 months	１０

Figure 5-3 Reliability prediction result by mission time 2 years	１０

Figure 5-4 Reliability prediction result by mission time 3 years	１１

Figure 5-5 Reliability prediction result by mission time 4 years	１１

Figure 5-6 Reliability prediction result by mission time 7 years	１１

Figure 5-7 Reliability prediction result by EEE parts	１２

INTRODUCTION

SCOPE

This document summarizes Reliability Prediction for the Single Board Computer (SBC) to be developed for the DTN analysis procedures and documentation shall be performed in accordance with MIL-HDBK-217.

DTN OVERVIEW

The mission of DTN is to validate DTN communication protocols by space-link testing through KPLO. So DTN operations are focused on testing the DTN protocols using KPLO space-links.

DOCUMENTS

APPLICABLE DOCUMENTS

|  | Document No. | Title |
| --- | --- | --- |
| AD-1 | KPLO-D0-210-003 | User Requirements Document (KARI) |
| AD-2 | KPLO-D0-524-008 | User Requirements Document (ETRI) |

REFERENCE DOCUMENTS

|  | Document No. | Title |
| --- | --- | --- |
| RD-1 | MIL-STD-461E | Requirements for the Control of Electromagnetic Interference Characteristics of Subsystems and Equipment |
| RD-2 | MIL-STD-462D | Measurements of Electromagnetic Interference Characteristics |
| RD-3 | ECSS-E-ST-10-06C | Space engineering Technical Requirements specification |
| RD-4 | ECSS-Q-60-11A | Derating and End of Life parameter drifts-EEE Components |
| RD-5 | MIL-HDBK-217 | Reliability Prediction for Electronic Equipment |

ACRONYMS AND ABBREVIATIONS

| AMS | Asynchronous Message Service |
| --- | --- |
| AOS | Advanced Orbiting System |
| BP | Bundle Protocol |
| BSS | Bundle Streaming Service |
| CCSDS | Consultative Committee for Space Data Systems |
| CFDP | CCSDS File Delivery Protocol |
| DCC | DTN Control Center |
| DTN | Delay(Disruption) Tolerant Network |
| DTNPL | DTN Payload |
| IP | Internet Protocol |
| KPLO | Korea Pathfinder Lunar Orbiter |
| LCM | Lander Communication Model |
| LTP | Liklider Transmission Protocol |
| M&C | Monitor & Control |
| RCM | Rover Communication Model |
| SLE | Space Link Extension |
| TBC | To Be Confirmed |
| TBD | To Be Defined |
| TCP | Transmission Control Protocol |
| UDP | User Datagram Protocol |

Reliability Prediction

The Configuration of SBC (Single Board Computer) is shown as below (Figure 4-1)

Figure 4-1 Configuration

RELIABILITY MODELING

A reliability prediction is one of the most common forms of reliability analysis, and an important element in the process of selecting electronic equipment.

The purpose of reliability prediction is to estimate the inherent reliability of electronic equipment and systems. Thus engineers can compare and evaluate reliability predictions of related or competitive designs.

As used here, reliability is a measure of the frequency of equipment failures as a function of time. The equipment(or system) failure rate is calculated using failure rates of parts and assemblies(or units). These predictions can help development engineers make decisions about component selection, stress levels and different designs.

ASSUMPTIONS

- The increased risk of failure during the early life can be disregarded.

- The wear-out period is never reached.

ANALYSIS CONDITION

Calculation Method : MIL-HDBK-217F Notice 2

Environment : Space Flight

Operating Temperature : 50 °C

Duty cycle : 100%

Mission Time : 8,760hours (1 year)

CALCULATIONS OF FAILURE RATES FOR PARTS

- Microcircuit, gate/logic arrays and microprocessors

-

- Microcircuit, memories

-

- Diodes, low frequency

-

- Diodes, high frequency

-

- Diodes, low frequency, bipolar

-

- Transistors, low frequency, bipolar

-

- Transistors, low frequency, Si FET

-

- Transistors, high frequency, Si FET

-

- Transistors, high frequency, GaAs FET

-

- Optoelectronics, detectors, isolators, emitters

-

- Resistors

-

- Capacitors

-

- Inductive Devices, Transformers, Coils

-

- Connectors, general

-

- Relay, mechanical

-

- Relay, solid state and time delay

-

- Switches

-

- Quartz crystals

-

- Fuses

-

CALCULATIONS OF FAILURE RATES FOR SYSTEM

-

RELIABILITY BLOCK DIAGRAM

SBC

Figure 4-2 Reliability block diagram

RELIABILITY ANALYSIS RESULT

The SBC reliability prediction result is show Table 4-1

| Value | Result |
| --- | --- |
| Duty Cycle | 100% |
| Failure Rate [FIT] | 2,706.725 |
| MTBF [Hours] | 369,450 |
| Reliability | 0.9766 |

Table 4-1 Reliability prediction result

Figure 4-3 Reliability prediction result (@Windchill Quality Solutions 11.0)

SBC reliability Prediction result is complied with allocated requirement.

Appendix

RELIABILITY PREDICTION BY MISSION TIME

|  | 1 month | 2 months | 1 year | 2 years | 3 years | 4 years | 7 years |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Hours | 720 | 1,440 | 8,760 | 17,520 | 26,280 | 35,040 | 61,320 |
| Reliability | 0.9980 | 0.9961 | 0.9766 | 0.9537 | 0.9313 | 0.9095 | 0.8470 |

Table 5-1 Reliability prediction result by mission time

Figure 5-1 Reliability prediction result by mission time 1 month

Figure 5-2 Reliability prediction result by mission time 2 months

Figure 5-3 Reliability prediction result by mission time 2 years

Figure 5-4 Reliability prediction result by mission time 3 years

Figure 5-5 Reliability prediction result by mission time 4 years

Figure 5-6 Reliability prediction result by mission time 7 years

RELIABILITY PREDICTION BY EEE PARTS

Figure 5-7 Reliability prediction result by EEE parts