Korea Lunar Exploration Program

DTNPL Software Requirement Specification

| Date: 28 Dec. 2017 | Doc. No: KPLO-D0-550-200 Issue: 1.0 Total page: 13 |
| --- | --- |

Superseding     Revised

| Prepared By: Hyun-Woong Choi Date DTN SBC S/W Engineer, Lumir Dae-Soo Oh Date DTN SBC Engineer, Lumir |  | Kyung-Rak Lee Date DTN Payload Engineer, ETRI |
| --- | --- | --- |
| Reviewed By: Jin-Ho Jo Date DTN System Engineer, ETRI |  | Sin-Ae Ji Date Proximity-1 System Engineer, ETRI |
| Sang-Man Moon Date RLE KPLO Electrical System, KARI |  | Hyun-Jin Shin Date RLE KPLO Mechanical System, KARI |
| Approval Signature: Byoung-Sun Lee Date DTN Project Leader, ETRI |  | Il-Young Kim Date RLE KPLO Product Assurance, KARI |
| Seong-Bin Lim Date Head, Lunar Exploration System Team, KARI |  | Seok-Weon Choi Date Director, Lunar Exploration Program Office, KARI |

Original CDMO Release__________________

| Revision History |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version | Author(s) |  | Date | Description |
| 0.1 | Hyun-Woong Choi |  | 2017. 12. 28 | Draft |

| Revision/Change Records |  |  |  |
| --- | --- | --- | --- |
| Version | Date | Revision/Change Description | Pages |
| 0.1 | 2017. 12. 28 | Initial | All |

Documents Tree

Contents

1.	Introduction	7

1.1.	Purpose and Scope	7

1.2.	Applicable Documents	7

1.2.1.	ETRI Documents	7

1.2.2.	Other Documents	7

1.3.	Abbreviations	7

2.	Overview of KPLO DTN	9

2.1.	DTN and Space Communications	9

2.2.	Mission of DTN Operation	9

2.3.	DTN Configurations	9

2.4.	Communication Nodes	10

2.4.1.	DTN nodes	10

2.4.2.	Non-DTN nodes	10

3.	Software Requirement Specification	11

3.1.	Functional Requirement	11

3.1.1.	DTN Protocols	11

3.1.2.	DTN User Utilities	11

3.1.3.	DTNPL Test Modes	11

3.1.4.	One Way Test Mode	11

3.1.5.	Round Trip Test Mode	11

3.1.6.	Health Report	11

3.1.7.	Fault, Detection and Recovery	11

3.2.	Performance Requirements	11

3.2.1.	Data Interface Speed	11

3.2.2.	Life Time	11

3.2.3.	Reliability	12

3.3.	Interface Requirements	12

3.3.1.	Data Interface	12

3.3.2.	Data Interface Format	12

3.3.3.	PPS Interface	12

4.	Verification Matrix	12

Figures

Figure 2.1 DTN test configuration in the first phase	10

Introduction

Purpose and Scope

This document defines the electrical and mechanical interfaces between KPLO bus and DTN payload.

Applicable Documents

ETRI Documents

| Document Number | Document Title |
| --- | --- |
| KPLO-D0-550-000 | System Requirement of DTN Payload |
| KPLO-D0-550-001 | System Specification of DTN Payload |

Other Documents

| Document Number | Document Title |
| --- | --- |
| KPLO-SP-310-001 | KPLO System Specification Document |
| KPLO-SP-340-001 | KPLO Payload/Spacecraft Bus ICD |

Abbreviations

| AMS | Asynchronous Message Service |
| --- | --- |
| BP | Bundle Protocol |
| BSS | Bundle Streaming Service |
| CCSDS | Consultative Committee for Space Data Systems |
| CFDP | CCSDS File Delivery Protocol |
| DCC | DTN Control Center |
| CRC | Cyclical Redundancy Check |
| DMA | Direct Memory Access |
| DTN | Delay(Disruption) Tolerant Network |
| DTNPL | DTN Payload |
| EOD | End of Data |
| EOF | End of File |
| FW | Firm Ware |
| FSW | Flight Software |
| ION | Interplanetary Overlay Network |
| KDSA | Korea Deep Space Antenna |
| KPLO | Korea Pathfinder Lunar Orbiter |
| LCM | Lander Communication Model |
| LTP | Licklider Transmission Protocol |
| MOC | Mission Operation Center |
| OBC | On Board Computer |
| OBT | On Board Time |
| PDHU | Payload Data Handling Unit |
| RCM | Rover Communication Model |
| RFU | Reserved for Future Use |
| SBC | Single Board Computer |
| SOD | Start of Data |
| SOF | Start of File |
| SOH | Start of Head |
| TBC | To Be Confirmed |
| TBD | To Be Defined |
| TC | Tele-command |
| TCP | Transmission Control Protocol |
| TM | Telemetry |
| UART | Universal Asynchronous Receiver Transmitter |
| USRT | Universal Synchronous Receiver Transmitter |

Overview of KPLO DTN

DTN and Space Communications

Delay/disruption tolerant networking (DTN) technology offers a new solution to highly stressed communications in space environments, especially those with long link delay and frequent link disruptions in deep space missions. Delay/disruption tolerant networking (DTN) technology offers a new solution that use a store-and-forward mechanism to combat long link delay and frequent link disruptions generally characterizing space communications. DTN communications use a bundle protocol (BP), to construct a store-and-forward overlay network. The BP, situated at the application layer of the Internet model, utilizes conventional Internet protocols to send and receive data bundles. The benefits of DTN communications are as follows.

- Interoperability and Reuse: A standardized DTN protocol suite enables interoperability of multi-agency communication assets and also allows NASA to use the same communication stack for future missions (LEO, NEO or Deep Space).

- Space Link Efficiency, Utilization and Robustness: DTN enables more reliable and efficient data transmissions resulting in more usable bandwidth. DTN also improves link reliability by having multiple network paths and assets for potential communication hops.

- Security: The DTN Bundle Security Protocol (BSP) allows for integrity checks, authentication and encryption, even on links where not previously used.

- Quality of Service: The DTN protocol suite allows for many priority levels to be set for different data types, ensuring that the most important data is received ahead of less important data.

- Improved Operations and Situational Awareness: The DTN store-and-forward mechanism along with automatic retransmission provides more insight into events during communication outages and significantly reduces the need for ground-based scheduling.

Mission of DTN Operation

The mission of DTN is to validate DTN communication protocols by space link testing through KPLO. So DTN operations are focused on testing the DTN protocols using KPLO space links.

DTN Configurations

In the first phase of lunar exploration, test lunar orbiter called Korea Pathfinder Lunar Orbiter (KPLO) will be launched without lunar lander. Figure 2.1 shows DTN configuration in first phase of lunar exploration. There are no Lander and Rover on the moon in first phase. So Lander Communication Model (LCM) and Rover Communication Model (RCM) will be located on earth for the DTN communication testing.

DTN Payload (DTNPL) is a DTN node in the space. DTNPL will be mounted on the KPLO for DTN communication. DTN Control Center (DCC) is a control center of DTN communication nodes. LCM and RCM are DTN nodes on ground. All DTN nodes on ground are interconnected by the terrestrial network such as Internet. Mission Operation Center (MOC) and Korea Deep Space Antenna (KDSA) are not DTN nodes. They just transfer DTN packets to next communication nodes.

Figure 2.1 DTN test configuration in the first phase

Communication Nodes

Many communication nodes are needed for the DTN operation. Some of communication nodes in the network are DTN nodes whereas otherwise nodes are non-DTN nodes. The function of communication nodes used in the DTN operation is as follows.

DTN nodes

- DCC is a ground DTN node which monitor and controls DTN nodes. DCC send commands to DTN nodes such as DTNPL, LCM and RCM. It also receives telemetries or acquired data from some DTN nodes such as RCM or LCM.

- DTNPL is a DTN node mounted on the KPLO. DTNPL can be operated as a DTN end point. And it also can be operated as relay nodes which relay DTN data between DCC and RCM or LCM.

- LCM is a ground DTN node. LCM relays DTN data between MOC and RCM.

- RCM is a ground DTN node and a DTN end point. The commands sent by the DCC is terminated at the RCM and the RCM returns telemetry or data to DCC through KPLO.

Non-DTN nodes

- MOC is a non-DTN node. MOC monitors and controls KPLO and payloads. MOC also receives and processes data generated from the payloads and transfers to each payload control center.

- KDSA is a ground station for deep space communication. KDSA will have a big size of antenna and RF equipment for space link interface. KDSA also has network equipment for terrestrial communication interface such as SLE link. KDSA is a non-DTN node.

Software Requirement Specification

Functional Requirement

DTN Protocols

The DTNPL shall support DTN protocols which include at least Bundle Protocol (BP) and Licklider Transmission Protocol (LTP).

DTN User Utilities

The DTNPL shall support DTN user utilities which include CCSDS File Delivery Protocol (CFDP), Asynchronous Message Service (AMS) and Bundle Streaming Service (BSS).

DTNPL Test Modes

The DTNPL shall support two kinds of tests, one-way test and round-trip test as defined in the document named ‘Operation concept of DTN’.

One Way Test Mode

The DTNPL shall be capable of performing test of AMS, CFDP and BSS for one-way test mode.

Round Trip Test Mode

The DTNPL shall be capable of performing test of AMS and CFDP for roundtrip test mode.

Health Report

The DTNPL shall reports it’s health status to KPLO OBC for the reliable operation.

Fault, Detection and Recovery

Autonomous fault detection shall be provided to monitor the health of DTNPL and to perform corrective procedures in the event of any anomaly.

Performance Requirements

Data Interface Speed

The DTNPL shall have enough speed of data interface for the communication with KPLO.

Life Time

The operational life time of the DTNPL shall be no less than 1 year from the entering lunar orbit.

Reliability

DTNPL reliability shall be more than 0.7 over the operational life time.

Interface Requirements

Data Interface

The DTNPL shall have serial data bus interface for the communication with KPLO.

Data Interface Format

DTN transaction data shall be compatible with CCSDS DTN standards and RFC specifications.

PPS Interface

The DTNPL shall have PPS interface for receiving PSS signal from the KPLO.

Verification Matrix

The verification matrix is shown below.

| SRS ID | Title | Methods |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T | A | I | D | S |
| SRS-DPL-10010 | DTN Protocols |  |  |  | O |  |
| SRS-DPL-10020 | DTN User Utilities |  |  |  | O |  |
| SRS-DPL-10030 | DTNPL Test Modes |  |  |  | O |  |
| SRS-DPL-10040 | One Way Test Mode |  |  |  | O |  |
| SRS-DPL-10050 | Round Trip Test Mode |  |  |  | O |  |
| SRS-DPL-10060 | Health Reports |  |  |  | O |  |
| SRS-DPL-10070 | Fault, Detection and Recover |  |  | O |  |  |
| SRS-DPL-20020 | Data Interface Speed |  |  |  | O |  |
| SRS-DPL-20040 | Life Time |  | O |  |  |  |
| SRS-DPL-20050 | Reliability |  | O |  |  |  |
| SRS-DPL-30030 | Data Interface |  |  | O |  |  |
| SRS-DPL-30040 | Data Interface Format |  |  | O |  |  |
| SRS-DPL-30050 | PPS Interface |  |  | O |  |  |