Korea Lunar Exploration Program

DTNPL Flight Model Thermal Vacuum Test Procedure

| Date: 12 March. 2019 | Doc. No: KPLO-D0-550-421 Issue: 1.0 Total page: 34 |
| --- | --- |

Superseding     Revised

| Reviewed By: Seong-Woon Cho Date Lumir, DTN SBC Product Assurance Kyung-Rak Lee Date ETRI, DTN Payload System Engineer |  | Prepared By: Sun-Ku Kim Date Lumir, DTN SBC Hardware Engineer Dae-Soo Oh Date Lumir, DTN SBC Project Manager |
| --- | --- | --- |
| Approval Signature: Chol-Oh Jeong Date ETRI, DTN Payload Product Assurance |  | Jin-Ho Jo Date ETRI, DTN Payload Project Leader, ETRI |

Original CDMO Release__________________

| Revision History |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version | Author(s) |  | Date | Description |
| 1.0 | Sun-ku Kim |  | 2019. 03. 12 | Initial |

Documents Tree

Contents

1.	Introduction	8

1.1.	Purpose and Scope	8

1.2.	Applicable Documents	8

1.2.1.	ETRI Documents	8

1.2.2.	Reference Documents	8

1.3.	Abbreviations	8

2.	Test Article	10

2.1.	DTNPL Configurations	10

2.1.1.	CON_PWR	10

2.1.2.	CON_COMM	10

2.1.3.	CON_DEBUG	11

2.2.	Verification Cross Reference	12

2.3.	Notification for Qualification and Acceptance Test	12

3.	Test Condition	13

3.1.	Test Personal	13

3.2.	Precaution and Warnings	14

4.	Test Setup	15

4.1.	Test Facility	15

4.2.	Thermal Vacuum Chamber Setup	15

4.3.	Test Preparation	15

4.3.1.	Control and Measuring Equipment	15

5.	Test Description	17

5.1.	Thermal Vacuum Test Levels	17

5.2.	Temperature Profile	17

5.3.	Test Parameters	18

6.	Test	19

6.1.	Test preparation	20

6.2.	Ambient Test Before Thermal Vacuum Test	21

6.3.	1st Cycle - Hot Plateau	22

6.4.	1st Cycle - Cold Plateau	23

6.5.	2nd Cycle - Hot Plateau	24

6.6.	2nd Cycle - Cold Plateau	25

6.7.	3rd Cycle - Hot Plateau	26

6.8.	3rd Cycle – Cold Plateau	27

6.9.	4th Cycle – Hot Plateau	28

6.10.	4th Cycle – Cold Plateau	29

6.11.	5th Cycle – Hot Plateau	30

6.12.	5th Cycle – Cold Plateau	31

6.13.	Back to Ambient Temperature	32

6.14.	Functional Test After Thermal Vacuum Test	33

6.15.	Remove DTNPL From Chamber	34

7.	Test Conclusion	34

Figures

Figure 2-1 DTNPL Configurations	10

Figure 2-2 Power Connector	10

Figure 2-3 Commend Connector	11

Figure 2-4 Debug Connector	12

Figure 4-1 Thermal Vacuum Chamber Layout	15

Figure 4-2 Location of Thermal Sensors	16

Figure 5-1 Temperature Design and Test Limits	17

Figure 5-2 Timeline for Thermal Cycle Profile	18

Tables

Table 4-1 Control Thermal Sensors	16

Table 4-2 Measurement Thermal Sensors	16

Table 5-1 DTNPL Test Temperature Limits	17

Introduction

Purpose and Scope

This document defines the Thermal Vacuum test procedure DTN payload of KPLO.

The purpose of this is to demonstrate that the DTN payload is working as expected under the required environmental conditions. During the Thermal Test a representative collection of functionality and performance test will be performed to check interfaces and hardware related functions.

Applicable Documents

ETRI Documents

|  | Document Number | Document Title |
| --- | --- | --- |
| AD01 | KPLO-D0-550-000 | System Requirement of DTN Payload |
| AD02 | KPLO-D0-550-001 | System Specification of DTN Payload |
| AD03 | KPLO-SP-320-002 | KPLO Environmental Design and Test Specification |

Reference Documents

|  | Document Number | Document Title |
| --- | --- | --- |
| RD01 | MIL-STD-1540C | Test Requirements for Launch, Upper-Stage, and Space Vehicles |
| RD02 | PSS-01-801 | Test Requirements Specification for Space Equipment |
| RD03 | ECSS-E-10-03-A | ECSS Space Engineering-Testing |

Abbreviations

| AMS | Asynchronous Message Service |
| --- | --- |
| BP | Bundle Protocol |
| BSS | Bundle Streaming Service |
| CCSDS | Consultative Committee for Space Data Systems |
| CFDP | CCSDS File Delivery Protocol |
| DCC | DTN Control Center |
| CRC | Cyclical Redundancy Check |
| DMA | Direct Memory Access |
| DTN | Delay(Disruption) Tolerant Network |
| DTNPL | DTN Payload |
| EOD | End of Data |
| EOF | End of File |
| FW | Firm Ware |
| FSW | Flight Software |
| ION | Interplanetary Overlay Network |
| KDSA | Korea Deep Space Antenna |
| KPLO | Korea Pathfinder Lunar Orbiter |
| LCM | Lander Communication Model |
| LTP | Licklider Transmission Protocol |
| MOC | Mission Operation Center |
| OBC | On Board Computer |
| OBT | On Board Time |
| PDHU | Payload Data Handling Unit |
| RCM | Rover Communication Model |
| RFU | Reserved for Future Use |
| SBC | Single Board Computer |
| SOD | Start of Data |
| SOF | Start of File |
| SOH | Start of Head |
| TBC | To Be Confirmed |
| TBD | To Be Defined |
| TC | Tele-command |
| TCP | Transmission Control Protocol |
| TM | Telemetry |
| UART | Universal Asynchronous Receiver Transmitter |
| USRT | Universal Synchronous Receiver Transmitter |

Test Article

The Objective of this Thermal Vacuum test is to demonstrate compliance of the DTNPL Requirements Specification [AD 03]

This test procedure shall be used for test of the DTNPL FM.

The test procedure is written such that results and other information shall be filled into it.

DTNPL Configurations

The unit under test is shown in the following figures :

Figure 2-1 DTNPL Configurations

Dimension: 120 x 170 x 39 ㎜

Weight: 1.23 kg

CON_PWR

- Power Input (2 lines for +28V, 2 lines for RTN)

Figure 2-2 Power Connector

CON_COMM

- 1 UART for OBC

- 2 USRT for PDHU(P/R)

- 1 Bi-level Status for OBC

- 1 Bi-level Command for OBC

Figure 2-3 Commend Connector

CON_DEBUG

- 1 JTAG for Firmware Uploading

- 1 RS422 for Debugging UART

- 1 SpaceWire for Image Uploading, 40 Mbps (TBR)

- 1 GPIO for Debugging

- 1 SPI for DT10 (S/W Test Platform)

Figure 2-4 Debug Connector

Verification Cross Reference

The requirements which have to be verified by a successful performance in the test and its relevant test steps are given below:

| Applicable Doc. | Requirement | Verified by Test Step | Compliance |
| --- | --- | --- | --- |
| KPLO-SP-320-002 | Environmental Design and Test Specification | All | C |

Notification for Qualification and Acceptance Test

This test procedure shall be approved by the ETRI prior to performance of the test, either by signature on cover page of by performance of a dedicated Test Readiness Review(TRR).

Test Condition

Test Personal

The personnel required for the performance of the test is listed below:

Test Engineers

The test engineers are responsible for :

- Operation of measuring instrumentation, data acquisition, data handling, data recording

- Identification and compilation of data sheet

- Safety precaution for test articles

- Test personnel to be present in the test area

- The accuracy of the measured values & Data report

Test Manager

The test manager is responsible for :

- Test articles and technical aspects of the test performance

- Record the correct software version for test article and EGSE

- Examination of disturbances

- Test article Integration and handling

- Test performance

- Evaluation of test results & Providing of test reports

Quality Assurance Engineer

The QA engineer is responsible for :

- Surveillance of test equipment according to regulations as well as the test procedure

- Statement that the test articles to be tested have passed all checks before testing

- Checking the identification markings on the test articles

- Supervision of the proceeding with respect to quality assurance performance

The customer has the fight to participate or observe the test

| Responsibility | Company /Dep. | Name |
| --- | --- | --- |
| Test Engineer | Lumir / R&D | Sun-ku Kim |
| Test Manager | Lumir / R&D | Dae-Soo Oh |
| Quality Assurance Engineer | Lumir / PA | Seong-Woon Cho |
| Facility Engineer / Operator | KASI / Engineer |  |

Precaution and Warnings

For all testing and handling, Environmental requirements concerning personnel, facilities and test equipment shall be regarded.

No special hazard exists during the test.

The following precautions apply:

- Only trained personnel are allowed to handle the test specimen.

- A test cable or breakout box shall never be connected of disconnected when the test specimen is powered.

- Correct set-up of the test equipment has to be checked carefully prior to the connection/switch-on of flight hardware, e.g. bonding, grounding, voltages, current limiters, etc.

- The test specimen shell be protected by a clean, antistatic bag all the time no activities are performed at the hardware.

Test Setup

Test Facility

The DTNPL test will be performed at the Thermal Vacuum facility. Thermal Vacuum chamber provides the anechoic chamber.

Thermal Vacuum Chamber Setup

For Thermal Cycling Test the following setup will be used:

Figure 4-1 Thermal Vacuum Chamber Layout

Following Software will be used for Thermal Vacuum Test

- Software : KPLO_DTN_EGSE_SW.exe

- Version : v1.0

Test Preparation

The test harness shall be connected from DTNPL EGSE to the DTNPL through the Thermal Vacuum Chamber feed-thru. During the test, the DTNPL shall be operated in representative manner

(i.e. data are transferred via all interfaces and stored).

Control and Measuring Equipment

The thermal plate and the DTNPL unit will be equipped with temperature sensors(Thermocouples) on the positions defined in the descriptions below. The actual sensor positions will be noted after fixing on the equipment as given in the sensor location plan at the following tables:

Table 4-1 Control Thermal Sensors

| No. | Position | Remark |
| --- | --- | --- |
| C1 | Base Plate | Defined by Test Engineer |
| C2 | Base Plate (optional) | Defined by Test Engineer |

Table 4-2 Measurement Thermal Sensors

| No | Position | Remark |
| --- | --- | --- |
| T1 | DTNPL Top Cover |  |
| T2 | DTNPL Case Side |  |

Figure 4-2 Location of Thermal Sensors

Test Description

Thermal Vacuum Test Levels

The nominal sequence of the thermal cycles is depicted in and temperature range for tests are summarized in

Table 5-1 DTNPL Test Temperature Limits

|  | Temperature Limits (degree C) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Equipment | Proto-flight (Operating) Range |  | Cold Turn-On | Acceptance (Operating) Range |  | Survival (Non-Operating) Range |  |
|  | Min | Max |  | Min | Max | Min | Max |
| DTNE_DTNPL | -30 | 60 | -30 | -20 | 50 | -35 | 70 |

Figure 5-1 Temperature Design and Test Limits

Temperature Profile

The DTNPL shall be subjected to temperature cycles as described in the following figure. DTNPL operation over the range of temperatures shall be demonstrated at specific points during the thermal cycling test according to the following figure.

Figure 5-2 Timeline for Thermal Cycle Profile

Test Parameters

The following test parameters shall apply:

- TN min/max : -35℃ / 70℃

- TA min/max : -25℃ / 55℃

- TS: -30℃

- Tambient : ambient

- ts : minimum 1hr

- tA : minimum 2hr

- dT/dt ≤ 2℃

- Pressure ≤ 1 x 10-5

- Number of cycles : 5 cycle

Test

The DTNPL shall have a full functional test performed at ambient temperature prior to beginning the thermal cycling test. Thermal soak duration shall be at least 6 hours at the hot temperatures and 6 hours at

the cold temperatures during the first and last cycle (Figure 5-2). Intermediate cycles shall give at least 1 hour soak at the hot and cold temperatures. During the thermal soak, the unit shall be turned off until the temperature stabilizes and turn on. Remaining on until the next soak period off on sequence. Measurement of thermal soak duration shall begin at the time of unit turn on (Figure 5-2).

Functional tests shall be conducted after the DTNPL temperature have stabilized at hot and cold temperatures during the first and last thermal cycle, and after return to ambient. During the remainder of the test, electrical and electronic units, including all redundant circuits and paths, shall be cycled through various operation modes.

DTNPL shall meet their performance requirements within specification over the maximum expected temperature range extended at both temperatures by 10°C. The final thermal cycle should employ a sufficiently slow temperature transition to permit a complete functional check to be repeated at essentially all temperatures.

Moisture condensation in side of DTNPL shall be prevented.

Test preparation

Start Date : ________________________   /  Time : ________________________

| (1) | Fill out test environment conditions | Pass / Fail | Sign |
| --- | --- | --- | --- |
| (2) | Fill out test personnel | Pass / Fail | Sign |
| (3) | Check test articles and fill out, check, and sign Test Article list above | Pass / Fail | Sign |
| (4) | Check test equipment and fill out, check, Test Equipment list above setup complete | Pass / Fail | Sign |
| (5) | Power off for each test article and test equipment switched off Install DTNPL in Thermal Vacuum Cycling chamber | Pass / Fail | Sign |
| (6) | Mount DTNPL onto the base plate by fixing of screws. | Pass / Fail | Sign |
| (7) | Attach temperature sensors according to chapter 4.4.1 | Pass / Fail | Sign |
| (8) | Check that connector savers installed on all used interfaces | Pass / Fail | Sign |
| (9) | Connect harness to DTNPL and connect harness to chamber feed- through | Pass / Fail | Sign |
| (10) | Take pictures of the mounted DTNPL when ready for thermal cycling test | Pass / Fail | Sign |
| (11) | Start recording temperature measurement points and chamber temperature continuously | Pass / Fail | Sign |
| (12) | Correlate time, Temperature and test results exactly | Pass / Fail | Sign |

End Date:                        /  Time:

Test Engineer:                     (Signature)

Ambient Test Before Thermal Vacuum Test

Start Date:                        /  Time:

Temperature:                   ℃ (19 ~ 25℃)

Humidity:                    % (35 ~ 55℃)

With feed assembly in the thermal chamber, the functional test should be performed at the ambient condition in accordance with the following sequence and record the test results per the indicated method.

|  | Test Description | Nominal Value | Actual Value |
| --- | --- | --- | --- |
| (1) | Measure input voltage | Under 0.5A |  |
| (2) | Measure input voltage | 28V |  |
| (3) | Perform test ‘STP’ | PASS |  |

End Date:                        /  Time:

Test Engineer:                     (Signature)

1st Cycle - Hot Plateau

Start Date:                        /  Time:

- Increase the temperature in the T/V chamber (Ambient to +70℃)

|  | Temperature |  |  |  |
| --- | --- | --- | --- | --- |
|  | C1 | C2 | T1 | T2 |
| Start |  |  |  |  |
| End |  |  |  |  |

- After maintaining at least 1 hour at 70℃, then start the temperature drop

End Date:                        /  Time:

Test Engineer:                     (Signature)

1st Cycle - Cold Plateau

Start Date:                        /  Time:

- Start thermal transition Temperature(70℃ to -35℃)

|  | Temperature |  |  |  |
| --- | --- | --- | --- | --- |
|  | C1 | C2 | T1 | T2 |
| Start |  |  |  |  |
| End |  |  |  |  |

- Maintain at least 1 hour at -35℃, and then increase the temperature by -30℃.

-30℃ arrival time :

|  | Temperature |  |  |  |
| --- | --- | --- | --- | --- |
|  | C1 | C2 | T1 | T2 |
| Start |  |  |  |  |
| End |  |  |  |  |

- Verification ON/OFF switching

On/Off Normal state ?  :

- Maintain at least 1hour with power off state at -30℃

- DTN Power On

DTN P/S Parameter Value : __________V___________A

End Date:                        /  Time:

Test Engineer:                     (Signature)

2nd Cycle - Hot Plateau

Start Date:                        /  Time:

- Start thermal transition (-30℃ to 55℃)

|  | Temperature |  |  |  |
| --- | --- | --- | --- | --- |
|  | C1 | C2 | T1 | T2 |
| Start |  |  |  |  |
| End |  |  |  |  |

- Perform DTNPL test and fill out the table below

|  | Test Description | Nominal Value | Actual Value |
| --- | --- | --- | --- |
| (1) | Measure input voltage | Under 0.5A |  |
| (2) | Measure input voltage | 28V |  |
| (3) | Perform test ‘STP’ | PASS |  |

- Verification OFF/ON switching

Off/On Normal state ?  :

- Hold temperature for at least 1 hour

End Date:                        /  Time:

Test Engineer:                     (Signature)

2nd Cycle - Cold Plateau

Start Date:                        /  Time:

- Start thermal transition (55℃ to -25℃)

|  | Temperature |  |  |  |
| --- | --- | --- | --- | --- |
|  | C1 | C2 | T1 | T2 |
| Start |  |  |  |  |
| End |  |  |  |  |

- Perform DTNPL test and fill out the table below.

|  | Test Description | Nominal Value | Actual Value |
| --- | --- | --- | --- |
| (1) | Measure input voltage | Under 0.5A |  |
| (2) | Measure input voltage | 28V |  |
| (3) | Perform test ‘STP’ | PASS |  |

- Hold temperature for at least 1 hours.

End Date:                        /  Time:

Test Engineer:                     (Signature)

3rd Cycle - Hot Plateau

Start Date:                        /  Time:

- Start thermal transition (-25℃ to 55℃)

|  | Temperature |  |  |  |
| --- | --- | --- | --- | --- |
|  | C1 | C2 | T1 | T2 |
| Start |  |  |  |  |
| End |  |  |  |  |

- Perform DTNPL test and fill out the table below

|  | Test Description | Nominal Value | Actual Value |
| --- | --- | --- | --- |
| (1) | Measure input voltage | Under 0.5A |  |
| (2) | Measure input voltage | 28V |  |
| (3) | Perform test ‘STP’ | PASS |  |

- Hold temperature for at least 1 hour

End Date:                        /  Time:

Test Engineer:                     (Signature)

3rd Cycle – Cold Plateau

Start Date:                        /  Time:

- Start thermal transition (55℃ to -25℃)

|  | Temperature |  |  |  |
| --- | --- | --- | --- | --- |
|  | C1 | C2 | T1 | T2 |
| Start |  |  |  |  |
| End |  |  |  |  |

- Perform DTNPL test and fill out the table below.

|  | Test Description | Nominal Value | Actual Value |
| --- | --- | --- | --- |
| (1) | Measure input voltage | Under 0.5A |  |
| (2) | Measure input voltage | 28V |  |
| (3) | Perform test ‘STP’ | PASS |  |

- Hold temperature for at least 1 hours.

End Date:                        /  Time:

Test Engineer:                     (Signature)

4th Cycle – Hot Plateau

Start Date:                        /  Time:

- Start thermal transition (-25℃ to 55℃)

|  | Temperature |  |  |  |
| --- | --- | --- | --- | --- |
|  | C1 | C2 | T1 | T2 |
| Start |  |  |  |  |
| End |  |  |  |  |

- Perform DTNPL test and fill out the table below

|  | Test Description | Nominal Value | Actual Value |
| --- | --- | --- | --- |
| (1) | Measure input voltage | Under 0.5A |  |
| (2) | Measure input voltage | 28V |  |
| (3) | Perform test ‘STP’ | PASS |  |

- Hold temperature for at least 1 hour

End Date:                        /  Time:

Test Engineer:                     (Signature)

4th Cycle – Cold Plateau

Start Date:                        /  Time:

- Start thermal transition (55℃ to -25℃)

|  | Temperature |  |  |  |
| --- | --- | --- | --- | --- |
|  | C1 | C2 | T1 | T2 |
| Start |  |  |  |  |
| End |  |  |  |  |

- Perform DTNPL test and fill out the table below.

|  | Test Description | Nominal Value | Actual Value |
| --- | --- | --- | --- |
| (1) | Measure input voltage | Under 0.5A |  |
| (2) | Measure input voltage | 28V |  |
| (3) | Perform test ‘STP’ | PASS |  |

- Hold temperature for at least 1 hours.

End Date:                        /  Time:

Test Engineer:                     (Signature)

5th Cycle – Hot Plateau

Start Date:                        /  Time:

- Start thermal transition (-25℃ to 55℃)

|  | Temperature |  |  |  |
| --- | --- | --- | --- | --- |
|  | C1 | C2 | T1 | T2 |
| Start |  |  |  |  |
| End |  |  |  |  |

- Perform DTNPL test and fill out the table below

|  | Test Description | Nominal Value | Actual Value |
| --- | --- | --- | --- |
| (1) | Measure input voltage | Under 0.5A |  |
| (2) | Measure input voltage | 28V |  |
| (3) | Perform test ‘STP’ | PASS |  |

- Hold temperature for at least 2 hour

End Date:                        /  Time:

Test Engineer:                     (Signature)

5th Cycle – Cold Plateau

Start Date:                        /  Time:

- Start thermal transition (55℃ to -25℃)

|  | Temperature |  |  |  |
| --- | --- | --- | --- | --- |
|  | C1 | C2 | T1 | T2 |
| Start |  |  |  |  |
| End |  |  |  |  |

- Perform DTNPL test and fill out the table below.

|  | Test Description | Nominal Value | Actual Value |
| --- | --- | --- | --- |
| (1) | Measure input voltage | Under 0.5A |  |
| (2) | Measure input voltage | 28V |  |
| (3) | Perform test ‘STP’ | PASS |  |

- Hold temperature for at least 2 hours.

End Date:                        /  Time:

Test Engineer:                     (Signature)

Back to Ambient Temperature

Start Date:                        /  Time:

- Start thermal transition +25℃.

|  | Temperature |  |  |  |
| --- | --- | --- | --- | --- |
|  | C1 | C2 | T1 | T2 |
| Start |  |  |  |  |
| End |  |  |  |  |

End Date:                        /  Time:

Test Engineer:                     (Signature)

Functional Test After Thermal Vacuum Test

Start Date:                        /  Time:

- Perform DTNPL test and fill out the table below

|  | Test Description | Nominal Value | Actual Value |
| --- | --- | --- | --- |
| (1) | Measure input voltage | Under 0.5A |  |
| (2) | Measure input voltage | 28V |  |
| (3) | Perform test ‘STP’ | PASS |  |

- Switch DTNPL off.

Turn off time:

- Thermal Vacuum chamber vacuum release start

- When the Thermal Vacuum level returns to the default, Stop recording chamber data

End Date:                        /  Time:

Test Engineer:                     (Signature)

Remove DTNPL From Chamber

Start Date:                        /  Time:

- Disconnect Test Harness from DTNPL EGSE and chamber feed-through.

- Disconnect TC Harness from DTNPL EGSE and chamber feed-through.

- Remove temperature sensors.

- Dismount DTNPL Base plate fixations.

- Remove DTNPL from Chamber.

End Date:                        /  Time:

Test Engineer:                     (Signature)

Test Conclusion

The PA shall review the test data for acceptance of the thermal Vacuum test.

The following sign off is verification that the test article has satisfactorily passed thermal Cycling test.

PA:                            Date: